3GPP TSG-RAN WG3 Meeting #124	R3-243861

Fukuoka, Japan, May 20th – 24th 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	(pCR for TR 38.799): Mobility Support for WAB-Nodes

Document for:	Agreement

# pCR for TR 38.799

-------------------------------------------Start of changes-------------------------------------------

## 4.3.X	Mobility handling

The following scenarios for WAB-node mobility are supported:

- UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- UE´s AMF/UPF changes as the WAB-gNB moves.

- BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- BH UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

### 4.3.X.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves through the BH-RAN.

#### 4.3.X.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

#### 4.3.X.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501/TS 23.502. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

#### 4.3.X.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

### 4.3.X.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Editor’s NOTE: FFS whether NG connections can be suspended.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

#### 4.3.X.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and Mobility Registration Update as defined in TS 23.502.

#### 4.3.X.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s).

Editor’s NOTE: FFS whether a new logical WAB-gNB needs to be established for the NG connections with the new AMF(s).

Editor’s NOTE: FFS whether the following steps is needed to support the AMF change for UE.

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

-------------------------------------------End of changes-------------------------------------------