3GPP TSG-RAN WG3 Meeting #124	R3-243933

20th – 24th May 2024, Fukuoka, Japan

Title:	LS on Indicating Emergency Service Support over NG

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN3

To:	SA2

Cc:

Contact person:	Sen Xu

xusen@chinatelecom.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 discussed specification texts specified in clause 5.16.4.1 of TS 23.501 as quoted below.

<first quote> During Registration procedures over 3GPP access in a PLMN, the 5GC includes the Emergency Services Support indicator, valid for the current Registration Area and indicating per RAT that Emergency Services are supported if any of the following conditions is true within the current Registration Area: - the Network is able to support Emergency Services natively over 5GS; - E-UTRA connected to 5GC supports IMS Emergency Services (e.g. voice), and the NG-RAN is able to trigger handover or redirection from NR to E-UTRA connected to 5GC at QoS Flow establishment for IMS Emergency Services (e.g. voice); - NG-RAN is able to trigger handover to EPS at QoS Flow establishment for IMS Emergency Services (e.g. voice); - NG-RAN is able to trigger redirection to EPS at QoS Flow establishment for IMS Emergency Services (e.g. voice); or - NG-RAN is able to trigger 5G SRVCC handover to UTRAN for IMS Emergency Services (i.e. voice). <second quote > If a certain RAT is restricted for Emergency Services, AMF signals that the corresponding RAT is restricted for Emergency Services Support to the Master RAN Node. This helps assist the Master RAN node determine whether to set up Dual Connectivity for Emergency Services.

RAN3 found that the above second quoted function specified in TS 23.501 is not supported in TS 38.413 since R15. RAN3 has decided that a RAN configuration-based approach applies when some RAT is restricted for emergency service support. Therefore, RAN3 does not intend to introduce respective support in TS 38.413.

2	Actions

Actions to SA2

RAN3 respectfully asks SA2 to take the above into account and consider to align TS 23.501 accordingly

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125		19 – 23 August, 2024 		 Maastricht, Netherlands

RAN3#125-bis		14 – 18 October, 2024		 China, China