3GPP TSG-RAN WG3 #125bis	R3-245753

Hefei, China, 15th – 19th October, 2024

Agenda Item:	12.2

Source:	NTTDOCOMO (moderator)

Title:	Summary of Offline Discussion on WAB

Document for:	Approval

Introduction

This document provides a summary of the offline discussion on WAB.

CB: # WAB

- Decide in which Stage 2 specs text reflecting the agreements shall be included

- Based on the agreements taken, draft stage 2 CRs

- Discuss solutions on multi hop WAB and down select, if possible

- Draft LS to SA2 on ULI. Reply LS in R3-245752 (Qualcomm)

(Moderator - DoCoMo)

Summary of offline disc R3-245753

Discussion

RAN3 to consider the following RAN based solutions to avoid multi hop WAB:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No involvement of 5GC is expected

Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Solution4: BH-gNB broadcasts a new indicator “WAB allowed” in SIB. WAB-gNB does not broadcast “WAB allowed”.

Solution5: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

To be continued…

| solutions | Pros | Cons |
| --- | --- | --- |
| Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters. | No stage3 impact. | - Since operators have limited Frequencies/PCI resources, it is difficult to realize it in reality. |
| Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No involvement of 5GC is expected | No stage3 impact. | - In legacy, list of S-NSSAIs in RRCSetupComplete is used for AMF selection. If the list of S-NSSAIs is not supported by AMF. RAN node release the WAB-MT by RRCRelease. - WAB-MT may re-access the WAB-gNB after the waitTimer is expired. If the WAB-MT is stationary, then it may re-access the WAB-gNB endless. - Hence, access control is done at AMF rather than RAN node. |
| Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. | This solution explicitly bar the WA-MT, so it is a clean solution. | - There is stage3 impact. - This solution only work for initial access. - For handover, it is not workable. |
| Solution4: BH-gNB broadcasts a new indicator “WAB allowed” in SIB. WAB-gNB does not broadcast “WAB allowed”. |  | - There is stage3 impact. - It requires legacy gNB to support new indicator in SIB. |
| Solution5: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node. |  | - There is stage3 impact. - This solution only for work for handover. |

Q1: Please fill your companies views (Pros/Cons) for each solution (only add comments that are not reflected in the table above)

| Company name | Comments |
| --- | --- |

Q2: which solutions are preferred? Please explain the reasons.

| Company name | Comments |
| --- | --- |

Conclusion, Recommendations

References

| R3-245402 | Functional Aspects of WAB-Nodes (Ericsson) | discussion |
| --- | --- | --- |
| R3-245391 | (TPs for TS 38.300/38.413) Architecture and Access control for WAB (Huawei) | other |
| R3-245247 | (draft Reply LS to SA2) Discussion on SA2 questions on multi-hop WAB and UE ULI (Qualcomm Inc.) | other |
| R3-245175 | (TP for TS 38.401) Discussion on high level aspects for WAB (Nokia, Nokia Shanghai Bell) | other |
| R3-245176 | (TP for TS 38.423) Discussion on WAB mobility (Nokia, Nokia Shanghai Bell) | other |
| R3-245248 | Discussion on assumptions and architecture for WAB (Qualcomm Inc.) | other |
| R3-245252 | Discussion on stage-2 aspects for WAB (CATT) | discussion |
| R3-245253 | Other issues for WAB (CATT) | discussion |
| R3-245286 | (TP to BL CR of 38.423 on WAB) Discussion on the reliability and mobility for WAB (NEC) | other |
| R3-245381 | Discussion on Wireless Access Backhaul (NTTDOCOMO, INC.) | discussion |
| R3-245383 | Discussion on enhancements for WAB (CANON Research Centre France) | discussion |
| R3-245392 | (TP for TS 38.300) Discussion on WAB related procedures (Huawei) | other |
| R3-245155 | Discussion on WAB mobility (Samsung) | discussion |
| R3-245156 | Discussion on other aspects for the support of WAB (Samsung) | discussion |
| R3-245403 | Reply to SA2 Regarding WAB-MT Access Control and Additional ULI for WAB-Nodes (Ericsson) | discussion |
| R3-245446 | Architecture and configuration for WAB-node (Lenovo) | discussion |
| R3-245447 | Integration and migration for WAB node (Lenovo) | discussion |
| R3-245637 | Access control and location information in WAB (LG Electronics) | discussion |
| R3-245640 | Discussion on RAN2 impact of WAB (China Telecom) | discussion |
| R3-245641 | On Xn and NG interface management (China Telecom) | discussion |
| R3-245655 | (TP to 38.300) Discussion on supporting WAB and the reply LS to SA2 (ZTE Corporation) | other |
| R3-245656 | (TP to 38.305 38.455) Support of location service involving WAB (ZTE Corporation) | other |