3GPP TSG-RAN WG3 Meeting #125bis	R3-245780

Hefei, China, 14 - 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1357 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Retrieve UE Context Confirm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Issue #1: The Retrieve UE Context Confirm procedure is used in two scenarios: - new NG-RAN node to inform the old NG-RAN node whether the S-NG-RAN node associated with the old NG-RAN node for the UE that was indicated during UE context retrieval is kept or not by the new NG-RAN node during RRC resumption. - in case of RACH based SDT without UE context relocation, to request the termination of SDT session from the new NG-RAN node to the old NG-RAN node. The following procedural text is found: - Upon reception of the RETRIEVE UE CONTEXT CONFIRM message, the old NG-RAN node shall release the resources related to the UE-associated signalling connection between the old NG-RAN node and the new NG-RAN node, as specified in TS 37.340 [8]. In case of (a), the old NG-RAN node will only release the resources related to the UE-associated signalling connection between the old NG-RAN node and the new NG-RAN node upon receiving the UE CONTEXT RELEASE message from the new NG-RAN node, therefore, the above procedural text is incorrect. In case of (b), if the old NG-RAN node decides to terminate the ongoing SDT, it will send RETRIEVE UE CONTEXT FAILURE message to the new NG-RAN node, and if the old NG-RAN node decides to not terminate the ongoing SDT session, the SDT session will keep going, therefore, the above procedural text is incorrect as well. Issue #2: In 9.1.1.16, the message type is defined as reject, this aligns with the other procedures related to Retrieve UE Context or Partial UE Context Transfer, but in asn.1 criticality ignore is used. Issue #3: In 9.2.3.68, it is said that - This IE indicates whether the UE Context is kept at the S-NG-RAN node in case of an M-NG-RAN node handover without S-NG-RAN node change or inter-M-NG-RAN node RRC resume without S-NG-RAN node change. It is unclear what is “M-NG-RAN node handover without S-NG-RAN node change”, considering that this IE will not be used in case of intra-M-NG-RAN node handover, it is better to update it to “inter-M-NG-RAN node handover without S-NG-RAN node change”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Issue #1: remove the mentioned incorrect procedural text. For Issue #2: change the criticality of retrieveUEContextConfirm procedure from ignore to reject in asn.1. (NBC) For Issue #3: update the “M-NG-RAN node handover without S-NG-RAN node change” to “inter-M-NG-RAN node handover without S-NG-RAN node change”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only impacts the handling of Retrieve UE Context Confirm procedure and the related IEs. This CR has functional and protocol impact, and it is NBC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong procedural text exists and wrong criticality is used by the Retrieve UE Context Confirm procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.12.2, 9.2.3.68, 9.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V0, R3-245076, submisson. V1, update coverpage. |  |  |  |  |  |  |  |  |

-----------Start of the First Change-----------

8.2.12	Retrieve UE Context Confirm

8.2.12.1	General

The Retrieve UE Context Confirm procedure is used by the new NG-RAN node to inform the old NG-RAN node whether the S-NG-RAN node associated with the old NG-RAN node for the UE that was indicated during UE context retrieval is kept or not by the new NG-RAN node during RRC resumption.

In case of RACH based SDT without UE context relocation, the Retrieve UE Context Confirm procedure is also used to request the termination of SDT session from the new NG-RAN node to the old NG-RAN node.

The procedure uses UE-associated signalling.

8.2.12.2	Successful Operation

Figure 8.2.12.2-1: Retrieve UE Context Confirm, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT CONFIRM message to the old NG-RAN node.

If the UE Context Kept Indicator IE is included and set to "True", the old NG-RAN node shall consider that the S-NG-RAN node was kept by the new NG-RAN node and use this information as specified in TS 37.340 [8].

If the old NG-RAN node receives the SDT Termination Request IE in the RETRIEVE UE CONTEXT CONFIRM message, the old NG-RAN node shall, if supported, consider that the termination of the ongoing SDT session is requested from the new NG-RAN node for this UE and act as specified in TS 38.300 [9].

-----------Start of the Next Change-----------

9.1.1.16	RETRIEVE UE CONTEXT CONFIRM

This message is sent by the new NG-RAN node to the old NG-RAN node to inform the old NG-RAN node whether the S-NG-RAN node associated with the old NG-RAN node for the UE that was indicated during UE context retrieval is kept or not by the new NG-RAN node during RRC resumption.

In case of RACH based SDT without UE context relocation, the Retrieve UE Context Confirm procedure is also used to request termination of SDT session from the new NG-RAN node to the old NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| SDT Termination Request | O |  | ENUMERATED (radio link problem, normal, …) | Indicate the reason of request for termination of an ongoing SDT session. | YES | ignore |

-----------Start of the Next Change-----------

9.2.3.68	UE Context Kept Indicator

This IE indicates whether the UE Context is kept at the S-NG-RAN node in case of an inter-M-NG-RAN node handover without S-NG-RAN node change or inter-M-NG-RAN node RRC resume without S-NG-RAN node change.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Context Kept Indicator | M |  | ENUMERATED (true, …) |  |

-----------Start of the Next Change-----------

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

//skip unchanged part

iABResourceCoordination XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		IABResourceCoordinationRequest

SUCCESSFUL OUTCOME		IABResourceCoordinationResponse

PROCEDURE CODE			id-iABResourceCoordination

CRITICALITY				reject

}

retrieveUEContextConfirm	XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		RetrieveUEContextConfirm

PROCEDURE CODE			id-retrieveUEContextConfirm

CRITICALITY 			reject

}

cPCCancel XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		CPCCancel

PROCEDURE CODE			id-cPCCancel

CRITICALITY 			ignore

}

-----------End of the Changes-----------