3GPP TSG-RAN WG3 Meeting #125	R3-244702

Maastricht, NL, 19 - 23 Aug, 2024

Agenda Item:	16.2

Source:	Huawei - Moderator

Title:	Summary of Offline Discussion – CB: # AIoT1_ArchitectureProtocol

Document for:	Approval

# Introduction

This is the summary document for the following come back:

CB: # AIoT1_ArchitectureProtocol

- Start with R3-244102, figure out the system architectures for Topology1 and Topology2

- Check the potential solutions for Topology1 and Topology2 and capture protocol stacks if agreeable, any commonality with Topology1 and Topology2 can be further checked

(moderator - HW)

Summary of offline disc R3-244702

- For the Chairman’s Notes

For topology 1:

- For architecture, the figure in section 3.1 in R3-244102 is captured in TR 38.769.

- For protocol stack, the figure of option 3 in section 3.2 in R3-244102 is captured in TR 38.769, is captured in the TR with the following clarification texts:

  - The figure for option 3 serves as a starting point for further discussions

  - XXAP is terminated at AIoT RAN node.

  - The protocol stack in option 3 does not detail how Upper layer information is transported over XXAP, details are pending on SA2 agreements.

  - FFS on interaction between upper layer information exchange and XXAP in order to trigger the AIoT RAN node functions.

  - Signaling transport for XXAP is FFS.

For topology 2:

- For architecture, the figure of option 1 in section 4.1. in R3-244102 is captured in the TR with the following updates and clarification texts:

  - UE reader => AIoT enabled UE

  - NG/XX => XX,

  - XX could be based on NG or a new interface carried over NG or a new interface,

  - XX signaling could be transported via XX-C or XX-U

  - AIoT CN could include AMF and AIoT related network functions, but the figure mainly serves the purpose to further discuss AIoT related functions.

  - Option 1 doesn’t illustrate the protocol between AIoT enabled UE and CN, if there is any defined by SA2

- Additional agreements for topology 2:

  - AIoT enabled gNB performs radio resource management for AIoT related radio resources, details pending on RAN1 and RAN2 mechanisms

  - To support Topology 2, the following solution options are to be studied for conveying AIoT upper layer contents:

    - RRC based solution [With this solution, AIoT CN applies AIoT upper layer information explicitly per AIoT transaction over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC.]

    - NAS based solution [With this solution, there is no explicit termination of AIoT upper layer information at AIoT enabled gNB. AIoT upper layer information is transmitted over AIoT enabled UE's NAS.]

    - UP based solution [With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data.]

Editor’s note: how to enable radio resource control (i.e. trigger AIoT RAN node functions) for above solution options is FFS

- For protocol stack, to be continued

Agree the TP in R3-244716

# Discussion on Topology 1

## 3.1	System architecture for Topology 1

Proposal 1-1: Capture Logical System Architecture for Topology 1 as below:

Note: the details of AIoT CN are subject to SA2.

## 3.2	Protocol Stack for Topology 1

Proposal 1-2: For Topology 1 XXAP/SCTP/IP solution, select one protocol stack among the following options: (please show your preference)

Note: the details of AIoT CN are subject to SA2.

| Option 1 Upper layer ==> App and NAS, or App only | Option 2 |
| --- | --- |
| Option 3 | Option 4 |

Proposal 1-3: For Topology 1 XXAP/SCTP/IP solution, XX interface is NG interface, AIoT RAN is gNB.

Start with option 3, for this topology, XXAP is terminated in AIoT RAN node.

Upper layer information might be transported over XXAP, details are pending on SA2 agreements.

FFS on interaction between upper layer information exchange and XXAP in order to trigger the AIoT RAN node functions.

Signaling transport for XXAP is FFS.

# Discussion on Topology 2

## 4.1	System architecture for Topology 2

Proposal 2-1: select one System architecture for Topology 2 among the following options: (please show your preference)

Note: the details of AIoT CN are subject to SA2.

|  | System architecture for Topology 2 | Comment |
| --- | --- | --- |
| Option 1 (recommended) |  | NG/XX is used in the Figure, to covers all solutions: - RRC based solution, it is XX - NAS/UP based solutions, it is NG, and AIoT CN covers “AMF + AIoTMF” |
| Option 2 |  | Not cover RRC based solution. |
| Option 3 |  | Not cover NAS/UP based solutions. |
| Option 4 |  | In case Topo1 XX is NG, it covers all solutions. Otherwise, it only covers NAS based and UP based solution. YY could be UE Reader’s NAS, UE Reader’s PDU session. |

Take option 1

UE reader =》AIoT enabled UE

NG/XX => XX, XX could be based on NG or a new interface carried over NG or a new interface, XX signaling could be transported via XX-C or XX-U

AIoT CN could include AMF and AIoT related network functions

Option 1 doesn’t illustrate the protocol between AIoT enabled UE and CN

Proposal 2-2: CN performs the authorization of whether the UE can act as a UE Reader, and informs the gNB about the authorization status.

SA2 related

Proposal 2-3: AIoT enabled gNB performs radio resource management for AIoT related radio resources, details pending on RAN1 and RAN2 mechanisms

Proposal 2-4: To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer contents:

- RRC based solution [explicit XXAP signaling, and then forward the related information explicitly to the UE Reader via NR Uu RRC.]

- NAS based solution [No explicit termination of AIoT upper layer information at AIoT enable gNB, AIoT upper layer information is transmitted over AIoT enable UE's NAS]

- UP based solution [No explicit termination of AIoT upper layer information at AIoT enable gNB, AIoT upper layer information is transmitted as AIoT enable UE's User plane data]

Editor’s note: how to enable radio resource control for above is FFS

## 4.2	Protocol Stack for Topology 2

Proposal 2-2: For Topology 2 RRC based solution, select the following protocol stack

Note: In this solution, XX interface is the same interface in Topology 1.

Note: the details of AIoT CN are subject to SA2.

| Option 1 | Option 2 Device NAS: similar as upper layer |
| --- | --- |
| Option 3 | Option 4 |

- Remove the device NAS

Proposal 2-3: For Topology 2 NAS based solution, select the following protocol stack

Note: the details of AIoT CN are subject to SA2.

| Option 1 | Option 2 |
| --- | --- |
| Option 3 | Option 4 |

Proposal 2-4: For Topology 2 UP based solution, select the following protocol stack

| Option 1 | Option 2 |
| --- | --- |
| Option 3 | Option 4 |

No discussion on 4.2 due to lack of time.