3GPP TSG-RAN WG3 Meeting #125                                                       R3-244705

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	16.4

Source:	CMCC

Title:	Summary of Offline Discussion – CB: # AIoT3_Location

Document for:	Approval

# Introduction

CB: # AIoT3_Location

- Discuss the open issue above and capture agreements

(moderator - CMCC)

Summary of offline disc R3-244705

# For the Chairman’s Notes

Propose to capture the following:

# Discussion

## Use Cases agreed in RAN3-124 meeting

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

## FFS in online session

For use case1, the AIoT RAN node ID can be regarded as location information of AIoT device in Topology1. The encoding of AIoT RAN node ID needs to be further checked. Other granularities are FFS, e.g., TRP ID.

## Topology 1

Proposal 1: At least the AIoT RAN node ID is included in the location information of AIoT device in topo 1.

Q1: Companies are invited to provide your views on whether to agree proposal 1.

| Company | Yes/No | Comment |
| --- | --- | --- |

Proposal 2: Introduce finer granularity (e.g. TRP ID, Reader ID) besides the AIoT RAN node ID in Topo1.

Q2: Companies are invited to provide your views on whether to agree proposal 2.

| Company | Yes/No | Comment |
| --- | --- | --- |

## Topology 2

Proposal 3: UE ID of the UE reader is regarded as location information of AIoT device in topo 2. Which UE ID to be used is FFS as it is subject to the Topo 2 solutions.

Q3: Companies are invited to provide your views on whether to agree proposal 3.

| Company | Yes/No | Comment |
| --- | --- | --- |

## Other aspects

Proposal 4: The utilization of more than one “reader” for location purposes is not considered in this release.

Q4: Companies are invited to provide your views on whether to agree proposal 4.

| Company | Yes/No | Comment |
| --- | --- | --- |

Companies are invited to provide other potential issues using below tabular.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations

If needed

# References

| 1 | R3-244596 | Discussion on locating Ambient IoT device (CMCC) |
| --- | --- | --- |
| 2 | R3-244061 | (TP for TR38.769) Locating an AIoT device (ZTE Corporation) |
| 3 | R3-244103 | (TP for TR 38.769) Ambient IoT device locating (Huawei) |
| 4 | R3-244159 | Further discussion on ambient IoT locating (NEC) |
| 5 | R3-244186 | (TP to TR 38.879) Locating of A-IoT device (CATT) |
| 6 | R3-244193 | (TP for TR 38.769) Locating AIoT device (Xiaomi) |
| 7 | R3-244390 | [TP for TR 38.769] On locating AIoT devices (Ericsson) |
| 8 | R3-244429 | Methods for locating Ambient IoT devices (Qualcomm Incorporated) |