3GPP TSG-RAN WG3 Meeting #125                                                       R3-244705

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	16.4

Source:	CMCC

Title:	Summary of Offline Discussion – CB: # AIoT3_Location

Document for:	Approval

# Introduction

CB: # AIoT3_Location

- Discuss the open issue above and capture agreements

(moderator - CMCC)

Summary of offline disc R3-244705

# For the Chairman’s Notes

Propose to capture the following:

AIoT RAN node can be regarded as location information of AIoT device in topology 1.

UE ID of the AIoT enabled UE can be regarded as location information of AIoT device in topology 2.

FFS to introduce finer granularity to locating an Ambient device besides the AIoT RAN in topology 1.

FFS support more than one “reader” for locating AIoT device.

# Discussion

## Use Cases agreed in RAN3-124 meeting

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

## FFS in online session

For use case1, the AIoT RAN node ID can be regarded as location information of AIoT device in Topology1. The encoding of AIoT RAN node ID needs to be further checked. Other granularities are FFS, e.g., TRP ID.

## Topology 1

Proposal 1: At least the AIoT RAN node ID is included in the location information of AIoT device in topo 1.

Q1: Companies are invited to provide your views on whether to agree proposal 1.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | See comments | We can agree that at least AIoT RAN node can be regarded as Reader ID, but we cannot agree to include it in the location information in the signalling, as the AIoT CN knows which RAN node it’s communicating, it knows the RAN node ID when receiving signalling from the RAN node, there’s no need to include redundant information in the siganlling. |
| CATT | See comments | Similar view with Xiaomi |
| Nokia | OK but | Depends on the solution for reporting. |
| NEC | See comments | We are not agree that the AIoT RAN node could be regarded as reader ID because single AIoT RAN node may control multiple readers. Others are similar with Xiaomi. |
| Qualcomm | OK | This is similar to how gNB ID is part of the ULI reported for NR. So it makes sense to have a similar “AIoT RAN ID” as a coarse location information. To Xiaomi/CATT: The AIoT CN might know the AIoT RAN node only if we agree that there is an interface management procedure and the AIoT CN maintains a XX association with the AIoT RAN node. If there is no such XX association, indicating the AIoT RAN node ID explicitly (like in ULI) is needed/useful. Whether multiple BS readers can be part of a single AIoT RAN node in T1 is yet to be confirmed. We can discuss finer granularity if this is confirmed. |
| CMCC | See comments | Similar view with Xiaomi |
| Huawei | See comments | Similar view with Xiaomi |
| ZTE | OK | It is needed to associate a selected reader |

Summary: Majority of companies propose that AIoT RAN node can be regarded as Reader ID, but people cannot agree to include it in the location information in the signaling.

Thus, it is proposed that AIoT RAN node can be regarded as location information of AIoT device in topology 1.

Proposal 2: Introduce finer granularity (e.g. TRP ID, Reader ID) besides the AIoT RAN node ID in Topo1.

Q2: Companies are invited to provide your views on whether to agree proposal 2.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes, TRP ID | We think a RAN node may have multiple AIoT antennas in different locations. The geographically co-located antennas that can transmitting or receiving AIoT radio signal can a reader. Referring following definitions of TRP, we think the TRP can re-used as reader ID in topology 1. There’s no need to introduce new concept or new ID. Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality. |
| CATT | Yes, but | Using TRP ID or define new Reader ID is FFS. |
| Nokia | OK but | Using TRP ID or define new Reader ID is FFS. |
| NEC | Yes, but | Same as CATT |
| Qualcomm | Not sure | We are not yet sure whether TRP terminology is applicable to AIoT RAN node. So propose to postpone this discussion. |
| CMCC | Yes, but | Using TRP ID or define new Reader ID is FFS. |
| Huawei | Yes |  |
| ZTE | Not sure | Pending |

Summary: Majority of companies propose that introducing finer granularity besides the AIoT RAN node ID in Topo1, but still some companies suggests to postpone the discussion.

Thus, it is proposed that FFS to introduce finer granularity to locating an Ambient device besides the AIoT RAN in topology 1.

## Topology 2

Proposal 3: UE ID of the UE reader is regarded as location information of AIoT device in topo 2. Which UE ID to be used is FFS as it is subject to the Topo 2 solutions.

Q3: Companies are invited to provide your views on whether to agree proposal 3.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | Yes |  |
| CATT | Yes | By UE specific signalling, AIoT CN could know where the device is. May not need to add additional UE ID in the XXAP,NGAP. |
| Nokia | OK but | Rephrase: UE ID of the UE reader is regarded as location information of AIoT device in topo 2. How UE is identified is FFS as it is subject to the Topo 2 solutions. |
| NEC | Yes |  |
| Qualcomm | Same view as NOK | To CATT: same comment as Q1, you are assuming there is UE associated signaling over XXAP and there is a XXAP ID. |
| CMCC | Yes |  |
| Huawei | Ok |  |
| ZTE | Yes |  |

Summary: Majority of companies propose that UE ID of the AIoT enabled UE can be regarded as location information of AIoT device in topo 2.

Thus, it is proposed that UE ID of the AIoT enabled UE can be regarded as location information of AIoT device in topology 2.

## Other aspects

Proposal 4: The utilization of more than one “reader” for location purposes is not considered in this release.

Q4: Companies are invited to provide your views on whether to agree proposal 4.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Xiaomi | See comments | We can focus on the reader level location information first, if time allows, other finer granularity can also be studied to meet the locating requirements. |
| CATT | Yes | Using more readers to support locating, sounds link positioning methods with multiple TRP to do corresponding measurement… RAN3 could not decide, RAN1 study and solution design is required. |
| Nokia | OK but | Depends if standards impact or not as per the SID. |
| NEC | Yes |  |
| Qualcomm | Wait |  |
| CMCC | Yes |  |
| Huawei | Yes |  |
| ZTE | Yes | Agree with CATT |

Summary: Some companies think that the utilization of more than one “reader” for location purposes is not considered in this release. Some companies think RAN3 could not decide, and the study and solution design from RAN1 is required.

Thus, it is proposed that FFS support more than one “reader” for locating AIoT device.

Companies are invited to provide other potential issues using below tabular.

| Company | Comment |
| --- | --- |
| CATT | Should we have the agreement or common understanding that one AIoT RAN may holds one or multiple readers(like TRPs) in topology 1. |
| NEC | Support CATT’s opinion. |

# Conclusion, Recommendations

If needed

# References

| 1 | R3-244596 | Discussion on locating Ambient IoT device (CMCC) |
| --- | --- | --- |
| 2 | R3-244061 | (TP for TR38.769) Locating an AIoT device (ZTE Corporation) |
| 3 | R3-244103 | (TP for TR 38.769) Ambient IoT device locating (Huawei) |
| 4 | R3-244159 | Further discussion on ambient IoT locating (NEC) |
| 5 | R3-244186 | (TP to TR 38.879) Locating of A-IoT device (CATT) |
| 6 | R3-244193 | (TP for TR 38.769) Locating AIoT device (Xiaomi) |
| 7 | R3-244390 | [TP for TR 38.769] On locating AIoT devices (Ericsson) |
| 8 | R3-244429 | Methods for locating Ambient IoT devices (Qualcomm Incorporated) |