3GPP TSG-RAN WG3 Meeting #127bis	R3-251649

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	11.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.473) Further discussion on signalling for AI/ML-based CCO

Document for:	Discussion and TP

# 1	Introduction

The following agreements were taken at RAN3#126:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn.

FFS on whether to send time information related to coverage modification cause over Xn.

Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

Further following agreements were taken at RAN3#127:

Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU.

The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

Value ‘0’ for the Future Cell Coverage State IE is needed and has the same meaning as cell inactive in legacy CCO.

We here provide further discussion on possible solutions for F1 and Xn signalling following these agreements, WAs and open points.

# 2	Discussion

## 2.1	CCO scenario with multiple DUs involved

In the last RAN3 meeting following agreements were made for the AI/ML based CCO signalling solution.

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn.

In order to provide consistent signalling solutions for F1 and Xn, with consideration of above agreements we propose to look into the following overall scenario, where gNB-CU1 (CU1) is serving three gNB-DUs, namely gNB-DU1 (DU1), DU2 and DU3, while CU2 serves DU’ as illustrated in Figure 1:

Figure 1: Illustration of scenario where CU1 is serving three DUs, namely DU1, DU2 and DU3. Similarly, CU2 is serving DU’.

Figure 2: F1 and Xn signalling for CCO issue detected by CU1 in a cell served by DU1, with neighbour cells served by other DUs in same gNB and neighbour gNB.

Figure 2 shows F1 and Xn signalling triggered by CCO issue within a cell served by DU1, with neighbour cells served by other DUs in same gNB (under CU1) and neighbour gNB (CU2). Along with predicted CCO issue, CU1 predicts the start time denoted as t0. The CU1 sends the predicted CCO issue and start time t0 to DU1 via Predicted CCO Assistance Information IE in the gNB-CU Configuration Update message. DU1 determines its own future coverage state c1 along with Time for Future Coverage Modification t1. DU1 provides the c1 and t1 information to CU1 via Future Coverage Modification Notification IE in gNB-DU Configuration Update message. At this point, CU1 sends the Predicted CCO Assistance Information IE in the gNB-CU Configuration Update to the remaining DUs involved in the predicted CCO issue i.e. DU2 and DU3. Additionally, it should be noted that DU2 and DU3 needs to know when DU1 is applying its selected Future coverage configuration i.e. Time for Future Coverage Modification t1 selected by DU1. If t1 is not provided to DU2 and DU3 then the time when DU2 and DU3 applies their matching coverage configuration will be different than t1, and therefore, leading to incompatible coverage configurations at different DUs. Therefore, it is proposed to add Time for Future Coverage Modification to the Predicted CCO Assistance Information IE in the gNB-CU Configuration Update message.

Note: As shown in Figure 2, the value of Time for Future Coverage Modification sent from CU1 to DU2 and DU3, denoted as t1’, may be different from t1 due to time required for message exchanges and processing at CU1 as it is expected to be expressed as relative time from the reception of gNB-DU Configuration Update message by CU1 from DU1. Similarly, the value Time for Future Coverage Modification sent from CU1 to CU2, denoted as t1’’, may be different from t1.

By sharing predicted CCO issue and time for predicted CCO issue t0 first to the DU affected by the CCO issue, there will it is possible to ensure one unique Time for Future Coverage Modification. This enables time coordination for coverage configuration change, hence avoiding any issue of application of incompatible coverage configurations at different DUs and at the neighbour nodes due to different times of application of future coverage configuration.

As shown in Figure 2, CU1 waits to receive the Future Coverage Modification Notification from the DUs involved in the predicted CCO issue before it initiates the XnAP NG-RAN Node Configuration Update which conveys the Future Coverage Modification List to the concerned neighboring nodes. We assume that the CU1 triggers a single XnAP procedure per predicted CCO issue, e.g. avoiding exposing the internal architecture of gNB1 to the neighboring nodes via the Xn interface.

Observation 1: Time-synchronized application of CCO state change is possible based on RAN3’s framework if additional time information is included in the F1AP Predicted CCO Assistance Information IE.

Proposal 1: Add Time for Future Coverage Modification to the F1AP Predicted CCO Assistance Information IE in the gNB-CU Configuration Update message.

## 2.2	Xn aspects

In the last meesting there was following open point:

FFS on whether to send time information related to coverage modification cause over Xn.

Currently, the Future Coverage Modification List IE in the NG-RAN NODE CONFIGURATION UPDATE message includes the "Time for Future Coverage State" IE, which specifies the relative time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1. The intention of this timing information is, at the time instant specified in Time for Future Coverage State IE, the receiving NG-RAN node2 shall match its own coverage. However, providing additional timing information related to the coverage modification cause i.e. time for predicted CCO issue, does not benefit but rather creates problems discussed below.

The only way for NG-RAN node2 to use the time for the predicted CCO issue is to apply the matching coverage configuration after the "Time for Future Coverage State" but before “time for predicted CCO issue” i.e between the time instant when NG-RAN node1 applies Future coverage state and time instant when predicted CCO issue occurs. By providing time for predicted CCO issue to NG-RAN node2, one allows NG-RAN node2 to delay the application of matching coverage configuration to after the neighbouring NG-RAN node1 has already applied the new coverage state. This delay results in NG-RAN nodes having different and likely incompatible coverage configurations for a certain duration (from the "Time for Future Coverage State" to the time when NG-RAN node2 applies the matching coverage configuration). Such a mismatch in the CCO configuration between neighboring nodes could negatively impact the UE and overall network performance.

Furthermore, by delaying the application of matching coverage configuration at NG-RAN node2, it could happen that a new CCO issue (immediate CCO issue or a CCO issue prediction) could emerge between the same NG RAN nodes (NG-RAN node1 and NG-RAN node2). The reason of emergence of this new CCO issue could be the mismatch in the CCO configuration between NG-RAN node1 and NG-RAN node2 node.  Similarly, there is also a possibility that a new CCO issue (immediate CCO issue or a CCO issue prediction) emerged can involve one or both of NG-RAN node1 and NG-RAN node2 and different NG RAN node(s). Again, the cause of emergence of this new CCO issue could be the mismatch in the CCO configuration between NG-RAN node1 and NG-RAN node2 node. Thus, there is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue. The issue created is similar to the one discussed in section 2.1 for the case where different DUs are served under the same CU.

Observation 2: There is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue.

Another issue that arises by delaying the application of matching coverage configuration at NG-RAN node2, is that NG RAN node1 will not know when NG-RAN node2 has applied the matching coverage state. Due to this the data collection at NG-RAN node1 that may be required to validate the outcome of the CCO issue prediction (selection of Future CCO state) will be hindered as NG-RAN node1 cannot determine the time of change of coverage configuration at NG-RAN node2.

Observation 3: In order to evaluate the outcome of the CCO issue prediction (selection of Future CCO state), the NG RAN node1 should know when NG-RAN node2 has applied the matching coverage state.

To overcome the issues mentioned above, it is proposed that additional timing information related to the coverage modification cause i.e. time for predicted CCO issue should not be provided over Xn.

Proposal 2: It is not beneficial to send the time for predicted CCO issue over Xn.

## 2.3 Timing Horizon

The following range and definition for requested prediction time were agreed in Rel-18 (XnAP):

| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) |
| --- | --- | --- | --- | --- |

This means that the maximum prediction time currently available for predicted radio resources is 60 seconds. There was no time in the study item to discuss the time scale of CCO issues, e.g. whether a prediction horizon within the Rel-18 agreement of 1 minute is sufficient or whether consideration of a longer time scale could have benefits and would be feasible. Also, discussions in normative phase has not lead to any conclusion, and there are corresponding FFSs in the F1AP and XnAP BL CRs:

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| Time for Predicted CCO Issue | O | INTEGER (1..FFS, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

[…]

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| […] |  |  |  |  |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..FFS, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

[…]

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1 à NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..FFS, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | - |  |

[…]

As part of this discussion, we observe that the list of input information for the AI/ML model, as captured in the TR, is very short, and in particular doesn’t include predicted UE trajectory. The latter allows for predictions well beyond 1 minute. We believe that the predicted UE trajectory information is valuable information both for prediction of coverage issue and cell edge capacity issue. Information about incoming UE traffic from neighbour cells, e.g. predicted incoming data volume, would also be valuable information in our view to predict cell edge capacity issue.

Furthermore, because CCO issues are often determined based on variations in UE concentrations and the Rel-19 work targets to enable proactive mitigation of such issues, it seems to us that a maximum prediction horizon of 5 minutes (300 seconds) should be considered for the Time for Predicted CCO Issue.

The Time for Future Coverage State could in theory be shorter than the Time for Predicted CCO Issue and/or the Time for Future Coverage Modification, while still maintaining a relatively similar time horizon. However, for simplicity it seems best to align the definitions of these IEs, and it is proposed that the time range for 'Time for Predicted CCO Issue,' 'Time for Future Coverage Modification,' and 'Time for Future Coverage State' should be considered from 1 to 300 seconds.

Proposal 3: RAN3 to discuss and agree on the time range of ‘Time for Predicted CCO Issue’, ‘Time for Future Coverage Modification’ and ‘Time for Future Coverage State’ to be from 1 to 300 sec.

## 2.4	Legacy and AI/ML based CCO co-existence

One of discussion in the RAN3#125bis meeting was on the co-existence of legacy and AI/ML based CCO. Here in this subsection, we analyse scenarios that exhibit such co-existence.

A) Sending legacy (immediate) CCO issue and predicted CCO issue for the same cell in the same message

Sending both a legacy (immediate) CCO issue and a predicted CCO issue for the same cell in the same message means gNB-CU has determined a legacy CCO issue that occurs immediately and also gNB-CU has predicted a CCO issue that will happen some time in future. However, it is important to note that the gNB-CU will be unaware of the new CCO state adopted by the gNB-DU as a result of the immediate CCO issue resolution, and therefore it is impossible for gNB-CU to predict a future CCO issue accurately i.e. the predicted future CCO issue is likely to be a wrong prediction. Therefore, legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be sent in the same message.

Proposal 4: Legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be sent in the same message.

B) Sending legacy (immediate) CCO issue and predicted CCO issue for the same cell in the two different messages

Depending on the order in which the CCO issue (legacy or AI/ML based) is sent there can be following two subcases.

1. gNB-CU sends first a legacy (immediate CCO issue) and then a predicted CCO issue

It means that gNB-CU first determines a legacy CCO issue which it sends and then gNB-CU predicts a CCO issue that will happen some time in future. Now, it should be understood that the gNB-CU while predicting the CCO issue will take into account new CCO state adopted by the gNB-DU as a consequence of the immediate CCO issue that was determined earlier. Therefore, it can be said that the prediction is based on latest information/situation and thus valid.

Proposal 5: It is possible to send a legacy (immediate) CCO issue first in one message and then the predicted CCO issue for the same cell later in a different message.

2. gNB-CU sends first predicted CCO issue and then legacy (immediate CCO issue)

Opposite to case 1, now gNB-CU first predicts and sends a CCO issue that will happen some time in future (say t). Then gNB-CU determines and sends a legacy CCO issue before expiry of t. Now, in order to solve the legacy (immediate) CCO issue, new cell and/or beam configuration (additionally, matching of coverage configuration in own/neighbour cells) will be applied at some of the gNB-DU served by gNB-CU. It means that the situation at which the CCO prediction was made will no longer be valid, and thus, the prediction itself will be invalid. In other words, legacy CCO issue resolution creates a new situation, rendering the initial CCO prediction obsolete.

Proposal 6: In case a legacy gNB-CU sends first predicted CCO issue with time for CCO prediction t, and then legacy (immediate) CCO issue for the same cell before expiry of prediction time t; the first predicted CCO issue should be discarded (deemed as invalid).

## 2.5	Information to validate the prediction

The following open point was identified at last meeting to be continued:

Information to validate the prediction is needed.

Before discussing the exact feedback information to validate the prediction, it is necessary to understand if the information required for validation should be pursued for predicted CCO issue and/or for the future coverage state adopted as a consequence of the predicted CCO issue.

The goal of using AI/ML based CCO is to proactively prevent the CCO issue from occurring, and therefore if the correct actions are taken to solve this predicted issue, then the CCO issue will not materialize and therefore can’t be confirmed. It can additionally be observed that any CCO state change happening in the time interval between the prediction is made and until the time for predicted CCO issue will in principle change the assumptions behind the prediction which also reduces the possibilities to verify the prediction.

Observation 5: It is not possible to validate the prediction of the CCO issue because it may not materialize due to corrective action and/or because the underlying assumptions of the prediction will change if a CCO state change is applied.

Let us therefore discuss the process of validating the future coverage state. For the case of CCO issue and CCO state changes concerning cells controlled by a single node, information locally available in the gNB-CU may be used for such validation. However, standardisation is needed for the case of CCO issue and CCO state changes concerning cells controlled by more than one node. In the legacy (reactive) case, a CCO configuration profile selected by an NG-RAN node 1 is signaled to a neighboring NG-RAN node 2, so that this NG-RAN node 2 matches/ adapts its own CCO configuration accordingly. However, in the case where a CCO issue is predicted in-advance by an NG-RAN node 1, there is currently a gap on how this NG-RAN node 1 can effectively coordinate with a neighboring NG-RAN node 2 in order for them to jointly apply their CCO configuration states in a way such that: i) the CCO issue predicted by NG-RAN node 1 is resolved in time (i.e., ahead of its predicted occurrence) and ii) the resolution of this CCO issue does not create another CCO issue to either of the neighboring NG-RAN nodes.

The following feedback information was captured in TR 38.743 to be considered to be collected from gNBs:

-	Measured radio resource status

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON Reports (e.g., RLF, CEF, RA)

The resolution of the CCO issue can be verified by UE performance feedback as indicated by the TR. The NG-RAN node 1 will also monitor CCO issue detection or prediction provided by NG-RAN node 2 and other neighbouring nodes, and in this way detect any potential issue created there.

Proposal 7: UE performance feedback and CCO issue detection in neighbour nodes to be used as information to validate the future coverage state.

# 3	Conclusion

We have made the following proposals:

Observation 1: Time-synchronized application of CCO state change is possible based on RAN3’s framework if additional time information is included in the F1AP Predicted CCO Assistance Information IE.

Proposal 1: Add Time for Future Coverage Modification to the F1AP Predicted CCO Assistance Information IE in the gNB-CU Configuration Update message.

Observation 2: There is a risk that uncoordinated application of the matching coverage configurations becomes the origin of a new CCO issue.

Observation 3: In order to evaluate the outcome of the CCO issue prediction (selection of Future CCO state), the NG RAN node1 should know when NG-RAN node2 has applied the matching coverage state.

Proposal 2: It is not beneficial to send the time for predicted CCO issue over Xn.

Proposal 3: RAN3 to discuss and agree on the time range of ‘Time for Predicted CCO Issue’, ‘Time for Future Coverage Modification’ and ‘Time for Future Coverage State’ to be from 1 to 300 sec.

Proposal 4: Legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be sent in the same message.

Proposal 5: It is possible to send a legacy (immediate) CCO issue first in one message and then the predicted CCO issue for the same cell later in a different message.

Proposal 6: In case a legacy gNB-CU sends first predicted CCO issue with time for CCO prediction t, and then legacy (immediate) CCO issue for the same cell before expiry of prediction time t; the first predicted CCO issue should be discarded (deemed as invalid).

Proposal 7: UE performance feedback and CCO issue detection in neighbour nodes to be used as information to validate the future coverage state.

A corresponding F1AP TP is provided in annex of this paper.

# Annex	- TP for BLCR to TS 38.473

<<< start of changes >>>

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and contains the Time for Future Coverage Modification IE, the gNB-DU may use it to adjust the coverage of its cells. 
Editor´s note: to be further discussed whether any more details need to be added to the above paragraph.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.B |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.212	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue or predicted to be affected by the predicted CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxAffectedCells> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxAffectedCells | Maximum numbers of cells affected by a CCO issue. Value is 32. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..300, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |
| Time for Future Coverage Modification | O |  | INTEGER (1..300, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU that serves the NR CGI IE contained in the Predicted Affected Cells and Beams IE, relative to the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NG CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values Indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicates the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..300, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

ASN.1 to be added

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>