3GPP TSG-RAN WG3 Meeting #127-bis	R3-251752

Wuhan, China, 7-11 April, 2025

Title: 	(TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei

Agenda item:	17.3

Document Type:	Other

- Introduction

In the previous RAN3 meetings, the following agreements were made:

Agreement at RAN3#125bis meeting The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast. Further check on first activation of UL WUS configuration broadcast in Cell A. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. Agreement at RAN3#126 meeting The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures. Agreement at RAN3#127 meeting It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

This contribution continues the discussion and provides our views.

- Discussion

As observed from the WID in [1], for Case 2, the UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell, and receives on-demand SIB1 from NES Cell. The overall flowchart is given in Fig.1 focusing on what we have achieved and the remaining issues that are needed to be discussed.

- Figure 1. The overall On-demand SIB1 procedure

2.1 The class 1 procedure

- The exact class1 name is still to be determined. It seems consensus to name the class 1 procedure as UL WUS Configuration Provision procedure (including request, response, and failure message) based on the contributions from previous meetings.

Naming the class 1 procedure as UL WUS Configuration Provision procedure (including request, response, and failure message).

In the UL WUS Configuration Provision Request message, a list of NES cells (e.g., Cells To UL WUS Request List) at the sending node should be included for UL WUS configuration provision. In the UL WUS Configuration Provision Response message, a list of accepted NES cells (i.e., Cells To UL WUS Result List) for which the receiving NG-RAN node would broadcast UL WUS configuration.

In the UL WUS Configuration Provision Request message, include a list of NES cells (i.e., Cells To UL WUS Request List) of the sending node for UL WUS configuration provision.

In the UL WUS Configuration Provision Response message, include a list of accepted NES cells (i.e., Cells To UL WUS Result List) for which the receiving NG-RAN node would broadcast UL WUS configuration.

- As given above, it was agreed that for the UL WUS configuration provision procedure, Cell A gNB would broadcast the UL WUS configuration at the first opportunity. The main issue how to design the new codepoint/choice “start with UL WUS Configuration” to trigger Cell A to broadcast UL WUS configuration in SIBx.

- Based on the previous discussions, it seems that there are following options.

- Option 1: Choice structure CHOICE On-demand SIB1; M. >UL WUS Config. >>UL WUS Config; M; OCTET STRING. >On-demand SIB1 Cell status. >> SIB1 Broadcast Status Indicator; M; ENUMERATED(broadcast, not broadcast, …). - Option 2: Choice Structure CHOICE On-demand SIB1; M. >UL WUS Config. >>UL WUS Config; M; OCTET STRING. >On-demand SIB1 Cell status. >> SIB1 Broadcast Status Indicator; M; ENUMERATED(not broadcast, …). - Option 3: Sequence Structure >>UL WUS Config; O; OCTET STRING. >>SIB1 Broadcast Status Indicator; O; ENUMERATED(broadcast, not broadcast, …).

- For the comparison between choice structure and sequence structure, the difference is whether the UL WUS Config and SIB1 Broadcast Status Indicator could be indicated simultaneously. Since it was agreed that Cell A gNB would broadcast the UL WUS configuration at the first opportunity, there is no need to indicate them simultaneously. Hence, the choice structure can be taken to define the basic structure.

- For the choice structure, option 1 is preferred to have also the “broadcast” indicator for the SIB1 broadcast status indication. Specifically, when the Cell A receives the UL WUS config, it can broadcast UL WUS at the first opportunity. Also, when the NES Cell indicates the SIB1 Broadcast Status Indication set to “not broadcast”, the Cell A can resume broadcasting the uplink WUS configuration.  On the contrary, upon receiving the SIB1 Broadcast Status Indication set to “broadcast”, e.g., when the NES cell receives the UL WUS request from the UE, the Cell A can stop broadcasting the uplink WUS configuration. Hence the SIB1 Broadcast Status Indicator (e.g., broadcast, not broadcast) would reflect the current SIB1 broadcast status of NES Cell, based on which the Cell A could determine whether to start or stop broadcasting the NES Cell's WUS configuration.

To take option 1 (choice structure with the “broadcast, not broadcast” codepoints) as the encoding of the “start with UL WUS configuration”

2.3 The class 2 procedure

At previous RAN3 meetings, it was agreed that the Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. A class 2 procedure is agreed to be introduced by using non-UE-associated signaling.

Then, after the NES Cell gNB has received the information from Cell A, in the split structure, the “stop” WUS configuration broadcast indication should also be sent to the NES Cell gNB-DU. With the information, DU could decide to deactivate the on-demand SIB1 state (i.e., beginning SIB1 broadcast).

- Figure 2. the Class 2 procedure for stop UL WUS configuration broadcast

Over the F1 for the NES node, when the NES Cell CU received the “stop” UL WUS configuration broadcast indication from the Cell A, it should deliver the Cells to stop UL WUS Provision List to the NES Cell DU in the gNB-DU configuration update message.

- There is a remaining issue whether there is a need for the Cell A gNB itself to send the “resume” or “restart” indication to NES Cell to assist the WUS configuration broadcast again.

Whether CellA can stop and resume the broadcasting of UL WUS configuration?

- We consider this is not much needed. First, Cell A possibly will not know if the NES Cell is currently in OD-SIB1 mode, hence it becomes useless for NES Cell. Second, the Cell A may not obtain the latest WUS config in time when the WUS config was updated.

There is no need for Cell A gNB itself to indicate the “start” or “restart” indication to NES Cell to assist the UL WUS configuration broadcast again.

Naming the class 2 procedure as UL WUS Configuration Provision Stop Indication.

2.4 UL WUS configuration provision over F1

- And in the previous RAN3#126 meeting, the BLCRs in [4, 5] were agreed to carry the UL WUS configuration from the NES Cell DU to the NES Cell CU, then to the Cell A CU over Xn interface, but marked as FFS.

- In the previous RAN2#127 bis meeting, it was agreed that the UL WUS configuration is included in a new SIB.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- It can be observed that the not only the Cell A can broadcast the UL WUS config for NES cell, the NES Cell itself can also broadcast its neighbour cell’s WUS configuration, including its own.

- At the Cell A, the UL WUS configuration of its neighbour NES cells can be obtained from the NES Cells and then encoded by the Cell A CU, and included in the gNB-CU system information to the Cell A DU. In case of any update/stop of the UL WUS configuration broadcast, the Cell A CU could update/delete the new SIB and send the gNB-CU system information to the Cell A DU. The same is true for the NES cell when it broadcast its neighbour cell’s WUS configuration.

At Cell A node, the UL WUS configuration can be encoded in a new SIB by the Cell A CU, and include in the gNB-CU System Information to the Cell A DU. The same is true for the NES cell when it broadcast its neighbor cell’s WUS configuration.

To remove the FFS on the UL WUS configuration in the F1AP BLCR.

2.5 Which node (NES Cell CU vs. NES Cell DU) decides whether the SIB1 should be broadcast or not

- Then, the next issue is which node determines to start/stop on-demand SIB1 broadcast. Basically, there are two options.

- Option 1: The NES Cell CU takes full decision and sends request to the Cell A CU. Upon positive response, it sends the request to the NES Cell DU.

- Option 2: The NES Cell DU itself decides when to stop or begin the SIB1 broadcast, and sends the request to the NES Cell CU, which signals to the Cell A CU. Upon positive response, the NES Cell CU sends the response to the NES Cell DU.

As analysed above, when the NES Cell DU receives the UL WUS signal (i.e., RACH) from the UE, according to RAN2 agreements (copied below), it is the NES Cell DU to send the RAR (e.g., accept, or reject) to the UE. Hence it is the NES Cell DU to decide whether to begin the SIB1 broadcast. And it makes sense the NES Cell DU decides when to stop the SIB1 broadcast. From energy saving perspective, the DU could know the exact energy cost of system information broadcast, number of UEs connected, paging quantities, RACH request etc, the DU itself could dynamically enter to the energy saving state.

RAR monitoring for MSG1 based OD-SIB1 REQ 13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

- Observation: The NES Cell DU receives the UL WUS signal from the UE, and decides the SIB1 broadcast.

The NES Cell DU itself decides when to stop or begin the SIB1 broadcast. Specifically,

- the NES Cell DU can send an SIB1 Broadcast Status Indicator to NES Cell CU in the gNB-DU configuration update message,

- based on the response (accept, or reject after the Xn signaling exchange), it decides whether to stop or begin the SIB1 broadcast.

On the other hand, considering that the CU has general system level information, e.g., the neighbour Cell A deployment, system overload etc, it would be beneficial for the CU to provide some assistance information. Hence the CU could provide a cell list to indicate which cells are allowed to enable on-demand SIB1 operation. After that, the DU could determine itself whether to enter the on-demand SIB1 operation to save the network energy based on the load etc in time.

The NES Cell CU can provide the NES Cell DU with a list of allowed cells for on-demand SIB1 in the gNB-CU Configuration Update message.

2.6 Other aspects

In the SOD R3-245737, the coverage cases of the NES Cell and Cell A were listed as follows.

- NES cell is partially covered by different Cells, not all cells are Cell A; - NES Cell is covered by multiple Cell As; - gNBs are not aware their neighboring gNB NES Cell?

For the coverage information, it is up to the OAM to determine the Cell A and NES cell coverage status, e.g., whether the Cell A is overlapped or partially overlapped with NES Cell etc. Even for the CCO issue, in current RAN spec, it does not specify the coverage scenarios among neighbouring cells. It is up to the OAM/implementation when the receiving node decides to adjust its cell/beam coverage when it receives the cell coverage state information. Following the same logic, the OAM can ensure that there is no coverage hole when the NES Cell enters on-demand SIB1 mode etc.

It is up to OAM implementation to ensure the OD-SIB1 works, e.g., the role of Cell A and NES cell, or to avoid coverage hole when the NES Cell enters on-demand SIB1 mode etc.

The Annex provides the TPs for TS 38.473 and TS 38.423.

3. Conclusion

In this contribution, we discussed the on-demand SIB1 operation related issues and made the following proposals:

The class 1 procedure

- Naming the class 1 procedure as UL WUS Configuration Provision procedure (including request, response, and failure message)

In the UL WUS Configuration Provision Request message, include a list of NES cells (i.e., Cells To UL WUS Request List) of the sending node for UL WUS configuration provision.

In the UL WUS Configuration Provision Response message, include a list of accepted NES cells (i.e., Cells To UL WUS Result List) for which the receiving NG-RAN node would broadcast UL WUS configuration.

To take option 1 (choice structure with the “broadcast, not broadcast” codepoints) as the encoding of the “start with UL WUS configuration”

Over the F1 for the NES node, when the NES Cell CU received the “stop” UL WUS configuration broadcast indication from the Cell A, it should deliver the Cells to stop UL WUS Provision List to the NES Cell DU in the gNB-DU configuration update message.

The class 2 procedure

There is no need for Cell A gNB itself to indicate the “start” or “restart” indication to NES Cell to assist the UL WUS configuration broadcast again.

Naming the class 2 procedure as UL WUS Configuration Provision Stop Indication.

UL WUS configuration provision over F1

At Cell A node, the UL WUS configuration can be encoded in a new SIB by the Cell A CU, and include in the gNB-CU System Information to the Cell A DU. The same is true for the NES cell when it broadcast its neighbor cell’s WUS configuration.

To remove the FFS on the UL WUS configuration in the F1AP BLCR.

Which node (NES Cell CU vs. NES Cell DU) decides whether the SIB1 should be broadcast or not

- Observation: The NES Cell DU receives the UL WUS signal from the UE, and decides the SIB1 broadcast.

The NES Cell DU itself decides when to stop or begin the SIB1 broadcast. Specifically,

- the NES Cell DU can send an SIB1 Broadcast Status Indicator to NES Cell CU in the gNB-DU configuration update message,

- based on the response (accept, or reject after the Xn signaling exchange), it decides whether to stop or begin the SIB1 broadcast.

The NES Cell CU can provide the NES Cell DU with a list of allowed cells for on-demand SIB1 in the gNB-CU Configuration Update message.

Other aspects

It is up to OAM implementation to ensure the OD-SIB1 works, e.g., the role of Cell A and NES cell, or to avoid coverage hole when the NES Cell enters on-demand SIB1 mode etc.

4. Reference

- RP-242354 Revised WID: Enhancements of network energy savings for NR

- RAN3_126_agenda_20241122_EOM1.doc

- R3-245737 Summary of offline discussions: Rel-19 Network Energy Saving, Ericsson

- R3-250055 (BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson etc

- R3-250056 (BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei etc

5. TP for TS 38.473 – on top of R3-251566

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

Editor’s Note: The procedure, message and the IE names are FFS.

8.2.3	F1 Setup

8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.2.4	gNB-DU Configuration Update

8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the NES Cell SIB1 Broadcast Status Indicator IE is included in the Served Cell To Modify IE in the gNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to determine the broadcast status of the UL WUS configuration, and return the result in the gNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message including the NES Cell SIB1 Broadcast Status Indication Response List IE.

If the NES Cell SIB1 Broadcast Status Indication Response List IE is included in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall, if supported, take into account when deciding to stop periodical SIB1 broadcast.

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to be On-Demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to enter the on-demand SIB1 operation within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells to Stop UL WUS Provision List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that the indicated cells will begin SIB1 broadcast.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| >>NES Cell SIB1 Broadcast Status Indicator | O |  | ENUMERATED (broadcast, not broadcast, ...) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | Ignore |
| NES Cell SIB1 Status Indication Response List |  | 0..1 |  | Only used between the NES Cell gNB-DU and NES gNB-CU | YES | reject |
| >NES Cell SIB1 Broadcast Status Indication Response List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >NES Cell SIB1 Broadcast Status Indication Response | O |  | ENUMERATED (accept, reject, ...) |  | - |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to be On-Demand SIB1 List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be On-Demand SIB1 List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cell To Stop UL WUS Provision List |  | 0..1 |  | Only used between the NES Cell gNB-CU and NES gNB-DU | YES | ignore |
| >Cell To Stop UL WUS Provision List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| UL WUS Configuration Information | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

6. TP for TS 38.423 on top of R3-250923

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

8.x.x	UL WUS Configuration Provision

8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 and request NG-RAN node2 to transmit UL WUS configuration information .

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.

The procedure uses non UE-associated signaling.

8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2.

If the UL WUS Config IE is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall broadcast UL WUS Configuration for the indicated cells indicated by the Cell ID IE, and reply with the UL WUS CONFIGURATION PROVISION RESPONSE message for which cells the request is fulfilled. The NG-RAN node2  stores the UL WUS configuration information for the cell indicated by the Cell ID IE.

If the SIB1 Broadcast Status Indicator IE set to “not broadcast” is included the included in the UL WUS CONFIGURATION PROVISION REQUEST message, and the NG-RAN node2 stores the UL WUS configuration information for the cells indicated by the Cell ID IE, the NG-RAN node2 shall broadcast UL WUS Configuration for the indicated cell, and reply with the UL WUS CONFIGURATION PROVISION RESPONSE message for which cells the request is fulfilled.

If the SIB1 Broadcast Status Indicator IE set to “broadcast” is included the included in the UL WUS CONFIGURATION PROVISION REQUEST message for the cells indicated by the Cell ID IE, the NG-RAN node2  shall stop broadcast UL WUS Configuration for the indicated cell, and reply with the UL WUS CONFIGURATION PROVISION RESPONSE message for which cells the request is fulfilled.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the UL WUS CONFIGURATION PROVISION REQUEST message and the UL WUS CONFIGURATION PROVISION RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot accept the indicated UL WUS config or the SIB1 Broadcast Status Indicator in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

8.x.x.4	Abnormal Conditions

Void.

8.x.y	UL WUS Configuration Provision Stop Indication

8.x.y.1	General

The purpose of the UL WUS Configuration Provision Stop Indication procedure is to stop the provision of the UL WUS configuration accepted by the NG-RAN node following a successful UL WUS Configuration Provision procedure.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION STOP INDICATION message to NG-RAN node2.

8.x.y.3	Unsuccessful Operation

Not applicable.

8.x.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.1.3.x1	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request the provision of the UL WUS configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cell To UL WUS Request List |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To UL WUS Request Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>CHOICE On-demand SIB1 | M |  |  |  |  |  |
| >>>UL WUS Config |  |  |  |  |  |  |
| >>>>UL WUS Config |  |  | OCTET STRING | Refer to [FFS] | – |  |
| >>>On-demand SIB1 Cell status |  |  |  |  |  |  |
| >>>>SIB1 Broadcast Status Indicator |  |  | ENUMERATED(broadcast, not broadcast, …) |  | - |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

9.1.3.x2	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) has (have) been successfully provisioned with the UL WUS configuration.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cell To UL WUS Result List |  | 0..1 |  | List of cells that successfully to be initiated. | YES | ignore |
| >Cell To UL WUS Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

9.1.3.x3	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate UL WUS configuration provision failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.1.3.y1	UL WUS CONFIGURATION PROVISION STOP INDICATION

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to indicate stop the provision of the UL WUS configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cell To Stop UL WUS Prevision List |  | 1 |  | Cell ID list to which the indication applies. | YES | ignore |
| >Cell To Stop UL WUS Prevision Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>