3GPP TSG-RAN WG3 Meeting #127-bis	R3-251759

Wuhan, China, Apr 7th - Apr 11th, 2025

Agenda item:	13.2

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Inter-CU LTM handover

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #127 [1]), significant progress was made, and many agreements were made towards specifying Inter-CU LTM.

RAN3 #127 agreements:

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

The format of the new introduced IE is same with the gNB-DU ID.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

Continue to work on the solutions comparison and details…

In this contribution, we discuss further signalling enhancements for inter-CU LTM handover, focusing on the following procedures and scenarios:

- Early TA RACH configuration,

- TA Information Transfer,

- LTM reference configuration,

- Cell Switch Notification,

- UE association information signalling upon LTM cell switch,

- SN initiated Inter-CU SCG LTM:

  - TA Information Transfer,

  - LTM reference configuration.

# 2 Early TA RACH configuration

Upon receiving the request for Early TA RACH configuration from the source gNB-CU via the candidate gNB-CU, a candidate gNB-DU allocates a set of CFRA resources for each ID of Early TA RACH configuration provided by the source gNB-CU in the request message. Each ID of Early TA RACH configuration corresponds to an underlying DU of the source gNB-CU in a split gNB architecture, and the allocated CFRA resources are to be used by the UEs served by the DU when initiating RACH for Early TA acquisition.

Proposal 1. Upon receiving the request for Early TA RACH configuration from the source gNB-CU in Handover Request message, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of IDs of Early TA RACH configuration provided by source gNB-CU,

- The source gNB ID.

Proposal 2. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of (ID of Early TA RACH configuration, CFRA resource set (i.e., RAP IDs)),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 3. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of (ID of Early TA RACH configuration, CFRA resource set (i.e., RAP IDs)),

- CFRA configuration for Early TA acquisition.

Figure 1: Example signalling flow of Early TA configuration procedure

Proposal 4. Upon receiving the Handover Request Ack from the candidate gNB-CU, the source gNB-CU may forward the received information to each of its underlying DUs in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

The source gNB-CU may use the LTM Configuration Update procedure, to obtain the RACH configuration and for allocation of CFRA resources for the candidate gNB-CUs. However, it was agreed in the last meeting RAN3 #127 [1] that the LTM Configuration Update procedure may be triggered at any later point in time depending on network implementation.

For obtaining the Early TA RACH configuration, in Handover Request Ack, each candidate gNB-CU provides (candidate gNB ID, List of IDs of Early TA RACH configuration).

Proposal 5. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

# 3 TA Information Transfer

Following Rel-18 Intra-CU LTM ideas, a candidate DU determines the TA value based on the received RACH preamble from the UE. However, the candidate DU cannot determine the UE ID from the received RACH preamble since the RAP ID is from the pool of assigned RAP IDs to the source DU, for the candidate cell. Therefore, the related WA from the last RAN2 #127 [1] meeting can be confirmed.

Proposal 6. Confirm the WA into agreement: Introduce a new non-UE associated Class 2 procedure on Xn, namely TA Information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 7. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

Proposal 8. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 9. The source gNB-CU forwards the TA information to the source gNB-DU using F1 CU-DU TA Information Transfer.

The source DU then identifies the UE based on the TA information associated with RAP ID and provides to the UE the TA value in the LTM Cell Switch MAC CE command during LTM execution.

Figure 2: TA information transfer from a candidate DU to source DU via signalling over  Xn

During subsequent LTM operation, a similar set of proposals as the above hold.

Proposal 10. During subsequent LTM, upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

Proposal 11. During subsequent LTM, the candidate gNB-CU forwards the TA information to the serving gNB-CU using TA Information Transfer over Xn.

Proposal 12. During subsequent LTM, the serving gNB-CU then forwards the TA information to the serving gNB-DU using the CU-DU TA Information Transfer message.

Proposal 13. As in Rel-18 Intra-CU LTM, the source or the serving gNB-DU maintains a check on the validity of the TA values, and it is up to network implementation how to do so.

# 4 LTM reference configuration

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

In the last RAN3 #127 meeting [1] the following issue was kept FFS.

Our thinking is that the source gNB generating the reference configuration for LTM is a good enough solution.

Companies proposing that a candidate gNB may also provide the LTM reference configuration were considering the case where a group of the candidate gNBs belong to a different network vendor than the source gNB. In that case, they argued that a candidate gNB belonging to the group may be able to generate a reference configuration that works for the configured candidate cells that belong to the group.

However, in that case the signaling may become more complicated, and the benefits are not clear. The source gNB can ask a candidate gNB from the group to provide the reference configuration. Then another two rounds of signaling is required so that the source gNB can send a request including the reference configuration to the other candidate gNBs, and then each candidate gNB responds with either a delta configuration with respect to the reference or with a complete configuration. Additionally source gNB itself will be from another infra vendor and candidate gNB provided configuration will not work for source gNB and source gNB has to provide full configuration for its cells. In 3GPP specification, source gNB does not know whether candidate gNBs are from same vendor or different vendor unless some OAM based approach is used, which is undesirable.

Observation 1. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 14. The agreed solution where the source gNB generates a reference configuration for LTM is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

# 5 Cell Switch Notification

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

The following agreement was reached in a previous meeting on the Cell Switch Notification procedure.

As in Rel-18 Intra-CU LTM, it is useful to transmit information regarding the TA values of LTM candidate cells from the source gNB-DU to the target gNB-DU upon LTM cell switch, since the UE does not need to invoke the Early TA acquisition procedure when it is in the target cell, for those LTM candidate cells for which the TA information is already available. This impacts the Cell Switch Notification procedure since this information needs to be carried over it.

Proposal 15. The Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value).

# 6 UE association information signalling upon LTM cell switch

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Upon LTM cell switch, it was agreed in the previous RAN3 #126 meeting [2], that the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNBs.

Thus, after each inter-CU LTM cell switch, the signaling to establish UE association between the new source gNB and another candidate gNB consists of:

- The new source gNB sends LTM Configuration Update message to another candidate gNB including a Source NG-RAN node UE XnAP ID. This identifies the UE associated signaling connection.

- The candidate gNB responds with LTM Configuration Update Ack message to the new source gNB including the Source NG-RAN node UE XnAP ID and the Target NG-RAN node UE XnAP ID.

The above information is not sufficient for the candidate gNB to identify the UE. The LTM Configuration Update message should contain a UE ID so that the candidate gNB knows for which UE is the UE association being established.

Observation 2. After an inter-CU LTM cell switch, the LTM Configuration Update message to establish UE association between the new source gNB and a candidate gNB should include a UE ID so that the candidate gNB can identify the UE.

An example of a UE ID that can be used by RAN3 for this purpose is the pair (source CU UE AP ID, candidate CU UE AP ID) (i.e., the pair (Source NG-RAN node UE XnAP ID, Target NG-RAN UE XnAP ID)) that was used by the source CU and the candidate CU when the LTM preparation procedure was initiated, i.e., in the Handover Request and Handover Request Ack messages. If the source gNB-CU used different source CU UE AP IDs for preparation of different candidate cells with the candidate gNB-CU, then one of the source CU UE AP IDs can be used for this purpose.

Proposal 16. After an inter-CU LTM cell switch, the new source CU sends LTM Configuration Update to each of the candidate CUs to establish the UE association, including the pair (source CU UE AP ID, candidate CU UE ID) used by the source CU and the candidate CU when LTM preparation was initiated, i.e., in the Handover Request and Handover Request Ack messages, and this information is used by the candidate CU to identify the UE.

# 7 SN initiated Inter-CU SCG LTM – TA Information Transfer

Similar ideas as for Inter-CU LTM can be applied to the case of SN initiated Inter-CU SCG LTM. A candidate SN-DU in a candidate SN determines the TA value based on the received RACH preamble from the UE. The candidate SN-DU transfers the TA information to the source SN-DU over the Xn signaling connection from the candidate SN to the MN using the non-UE associated Class 2 procedure, TA Information Transfer, defined for Inter-CU LTM (see Figure 3).

Figure 3: TA information transfer from candidate SN-DU to source SN-DU via signalling over Xn

Proposal 17. The TA information from a candidate SN to the MN and from the MN to the source SN is transferred using the non-UE associated Class 2 procedure message, TA Information Transfer.

Proposal 18.  The candidate SN-DU transmits to the candidate SN-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- MN ID,

- Source SN ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

The candidate SN forwards the TA information to the MN, which then forwards the TA information to the source SN using the TA Information Transfer message.

Proposal 19. The source SN-CU forwards the TA information to the source SN-DU using the F1 CU-DU TA Information Transfer.

The source SN-DU then identifies the UE based on the TA information associated with RAP ID and provides to the UE the TA value in the LTM Cell Switch MAC CE command during LTM execution.

# 8 SN initiated Inter-CU SCG LTM – LTM reference configuration

Regarding LTM reference configurations, one possible solution is to follow the principles of the Rel-18 Subsequent CPAC feature:

- There is one MCG reference configuration and one SCG reference configuration.

- The MN generates the MCG part of the reference configuration (if any).

- The source SN or a candidate SN generates the SCG part of the reference configuration.

- The MN can request an SCG reference configuration from either the source SN or a candidate SN.

Proposal 20. Similar as in Rel-18 Subsequent CPAC, the MN can request an SCG reference configuration from either the source SN or a candidate SN.

# Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Early TA RACH configuration

Proposal 1. Upon receiving the request for Early TA RACH configuration from the source gNB-CU in Handover Request message, the candidate gNB-CU forwards the request to its underlying candidate gNB-DU in F1 UE Context Setup Request, including:

- LTM candidate cell ID,

- List of IDs of Early TA RACH configuration provided by source gNB-CU,

- The source gNB ID.

Proposal 2. If the suggested LTM candidate cell can be prepared, in the F1 UE Context Setup Response message, the candidate DU provides to the candidate CU:

- LTM candidate cell ID,

- List of pairs of (ID of Early TA RACH configuration, CFRA resource set (i.e., RAP IDs)),

- CFRA configuration for Early TA acquisition,

- The source gNB ID.

Proposal 3. The candidate gNB-CU forwards the provided information from its underlying candidate DU to the source gNB-CU in Handover Request Ack:

- LTM candidate cell ID,

- List of pairs of (ID of Early TA RACH configuration, CFRA resource set (i.e., RAP IDs)),

- CFRA configuration for Early TA acquisition.

Proposal 4. Upon receiving the Handover Request Ack from the candidate gNB-CU, the source gNB-CU may forward the received information to each of its underlying DUs in F1 UE Context Modification Request:

- LTM candidate cell ID,

- CFRA resource set, i.e., RAP IDs,

- CFRA configuration for Early TA acquisition.

Proposal 5. For obtaining the Early TA RACH configuration, each candidate gNB-CU provides the pair (candidate gNB ID, List of IDs of Early TA RACH configuration) to the source gNB-CU in Handover Request Ack.

TA Information Transfer

Proposal 6. Confirm the WA into agreement: Introduce a new non-UE associated Class 2 procedure on Xn, namely TA Information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 7. The candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Source gNB ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

Proposal 8. The candidate gNB-CU forwards the TA information to the source gNB-CU using TA Information Transfer message over Xn.

Proposal 9. The source gNB-CU forwards the TA information to the source gNB-DU using F1 CU-DU TA Information Transfer.

Proposal 10. Upon successfully receiving the RACH preamble from the UE on a candidate cell, a candidate gNB-DU transmits to the candidate gNB-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- Serving gNB ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

Proposal 11. The candidate gNB-CU forwards the TA information to the serving gNB-CU using TA Information Transfer over Xn.

Proposal 12. The serving gNB-CU then forwards the TA information to the serving gNB-DU using the CU-DU TA Information Transfer message.

Proposal 13. As in Rel-18 Intra-CU LTM, the source or the serving gNB-DU maintains a check on the validity of the TA values, and it is up to network implementation how to do so.

LTM reference configuration

Observation 1. The case where the source gNB requests a candidate gNB to provide a reference configuration involves more signaling and complexity.

Proposal 14. The agreed solution where the source gNB generates a reference configuration for LTM is enough, and the source gNB does not request a candidate gNB to provide a reference configuration.

Cell Switch Notification

Proposal 15. The Cell Switch Notification message includes the TA information list, which consists of a list of pairs of (LTM candidate cell ID, TA value).

UE association information signaling upon LTM cell switch

Observation 2. After an inter-CU LTM cell switch, the LTM Configuration Update message to establish UE association between the new source gNB and a candidate gNB should include a UE ID so that the candidate gNB can identify the UE.

Proposal 16. After an inter-CU LTM cell switch, the new source CU sends LTM Configuration Update to each of the candidate CUs to establish UE association, including the pair (source CU UE AP ID, candidate CU UE ID) used by the source CU and the candidate CU when LTM preparation was initiated, i.e., in the Handover Request and Handover Request Ack messages, and this information is used by the candidate CU to identify the UE.

SN initiated Inter-CU SCG LTM – TA Information Transfer

Proposal 17. The TA information from a candidate SN to the MN and from the MN to the source SN is transferred using the non-UE associated Class 2 procedure message, TA Information Transfer.

Proposal 18.  The candidate SN-DU transmits to the candidate SN-CU the following TA information using F1 DU-CU TA Information Transfer message:

- TA value,

- RACH resource information (Detected RAP ID, RO information (RA-RNTI)),

- MN ID,

- Source SN ID,

- ID of Early RACH configuration,

- LTM candidate cell ID.

The candidate SN forwards the TA information to the MN, which then forwards the TA information to the source SN using the TA Information Transfer message.

Proposal 19. The source SN-CU forwards the TA information to the source SN-DU using the F1 CU-DU TA Information Transfer.

SN initiated Inter-CU SCG LTM – LTM reference configuration

Proposal 20. Similar as in Rel-18 Subsequent CPAC, the MN can request an SCG reference configuration from either the source SN or a candidate SN.

# References

[1] Chair notes, RAN3 #127 meeting.

[2] Chair notes, RAN3 #126 meeting.