3GPP TSG-RAN WG3 Meeting #127bis	R3-251857

Wuhan, China, 7-11 April 2025

Agenda Item:	21.3

Source:	Huawei

Title:	(TP for XR BL CR) Discussion on the miscellaneous issues for XR

Document for:	Approval

1. Introduction

Based on the progress in SA2, there are several new objectives for XR in RAN3, such as enhancements for PDU set based QoS handling and dynamic characteristics change. The detailed objectives are listed in the updated WID in RP-250107 [1], as excerpted below.

- Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). - QoS Handling when Traffic Characteristics Change Dynamically [RAN3] - TTNB and burst size to be provided over GTP-U - Support of exposure of available data rate [RAN3] Note: Coordination with SA2, as needed

In this contribution, we will analyse the specification impacts on the above listed miscellaneous issues and provide the corresponding TPs.

2. Discussion

2.1 DL PDU Set marking without PDU Set QoS

SA2 has investigated the solution to provide the PDU Set information to RAN in case there is no PDU Set QoS parameters. This is because the PDU Set and Data Burst information can be useful to RAN scheduling and power saving even there is no PDU set level QoS requirement. According to the following CR agreed in SA2, when there is no PDU set QoS parameters, the SMF may send a DL PDU Set Information Marking Support Indication to the NG-RAN to indicate that DL PDU Set Information marking is supported at the UPF. Upon receiving the indication, RAN can feedback the PDU Set based handling support Indication to indicate that PDU Set information is needed at RAN. Then the SMF would indicate the UPF to perform PDU Set marking.

------Excerption from S2-2502672 (agreed CR to TS 23.501)------ When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based QoS handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF. If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set Handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported. ------------unrelated parts are skipped------------ If the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported and a DL Protocol Description together with 1) a PSIHI and/or 2) PSDB and PSER is included in the PCC rule, the SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the DL Protocol Description used by the service data flow. The DL Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1. If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

To enable the above mechanism, NGAP should be enhanced to support the provision of the DL PDU Set Information Marking Support Indication sending from SMF to NG-RAN. The Indication can be included in the QoS flow level QoS parameters IE so that it can be provided though the PDU session setup and modification procedures. Besides, to support the scenario of handover and dual-connectivity, XnAP should also be enhanced to transfer the Indication. In Rel-18, RAN3 has agreed that, if a gNB supports PDU Set handling, all parts support it, including the gNB-CU-CP, the gNB-CU-UP and the gNB-DU. It should be the same for Rel-19, so that there is no need to further transfer the DL PDU Set Information Marking Support Indication to the gNB-CU-UP or the gNB-DU.

Proposal 1: RAN3 to enhance NGAP and XnAP to support the provision of DL PDU Set Information Marking Support Indication for a QoS flow.

Regarding the indication sent from NG-RAN to the SMF, the R18 indication used for PDU set based QoS handling can be reused since it has the same effect to the CN. According to the CR in SA2, they also used the same term for the indication in R18 PDU Set based QoS handling and R19 PDU Set marking. From RAN3 perspective, the IE should be reused, and the only specification impact is to update the procedure descriptions to support the R19 function.

Proposal 2: RAN3 to reuse the existing PDU Set Based Handling Indicator IE for PDU Set marking without PDU set QoS in Rel-19.

2.2 Alternative PDU Set QoS

SA2 also agreed to enhance alternative QoS profiles to take the PDU Set QoS parameters into account. As implemented in SA2 CR S2-2502678 [2], the SMF can provide one or more Alternative QoS Profile(s) which contains PSDB and PSER with UL and/or DL PDU Set QoS parameters to NG-RAN. To support this, NGAP should be enhanced by adding UL and DL PDU Set QoS parameters into the Alternative QoS Parameters Set List IE, which can be included in the QoS Flow Level QoS Parameters IE and provided to NG-RAN via the PDU session setup and modification procedures.

------Excerption from CR S2-2502678 (agreed CR to TS 23.501)------- 5.7.1.2a Alternative QoS Profile The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL as described in clause 5.7.7; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER. NOTE 1: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER (or PSDB or PSER) monotonically increase as the Alternative QoS Profiles become less preferred.

Moreover, F1AP and E1AP should also be enhanced to allow the gNB-DU and the gNB-CU-UP know the alternative PDU Set QoS information in case of split architecture. To support scenario of handover and dual-connectivity, XnAP should be considered as well. Similar IE designs can be applied to all the specifications.

Proposal 3: Add the PSDB and PSER in UL and DL as alternative PDU Set QoS parameters into the Alternative QoS Parameters Set List IE in NGAP, F1AP, E1AP and XnAP.

2.3 Dynamic Traffic Characteristics Change

SA2 concluded that NG-RAN can be provided with the dynamic traffic characteristic information, including Data Burst Size and Time to Next Burst via the user plane. This has already been captured in the SA2 specification TS 23.501 [3], as excerpted below.

----------Excerption from TS 23.501--------- 5.37.10 Supporting dynamically changing traffic characteristics via the User Plane 5.37.10.1 DL Data Burst Size marking The Data Burst Size may be provided to the NG-RAN in the downlink GTP-U header by the UPF in order to assist radio resource management. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Data Burst Size Marking Indication within a PCC Rule to request the UPF to identify and mark the Data Burst Size of the data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF. Based on the Data Burst Size Marking Indication in a PCC Rule, the SMF instructs the UPF to identify and mark the Data Burst Size of the data burst in downlink for the corresponding PDR. The SMF may provide the UPF the Protocol Description received in the PCC Rule. According to the Data Burst Size Marking Indication and DL Protocol Description from the SMF, the UPF identifies in the first DL packets of the data burst the Data Burst Size carried in an N6 RTP Header Extension/ transport protocol header as defined in TS 26.522 [179]. The UPF sends the identified Data Burst size to NG-RAN in the downlink GTP-U header of the first PDUs of the data burst. 5.37.10.2 Time to Next Burst marking The time to next data burst may be provided to the NG-RAN by the UPF to assist NG-RAN's behaviour in downlink. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the time to next data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF. Based on the Time to Next Burst indication in a PCC Rule, the SMF instructs the UPF to identify and mark the time to next data burst in downlink. The SMF may provide the UPF the Protocol Description used by the service data flow received in the PCC Rule. According to the Time to Next Burst indication and DL Protocol Description from the SMF, the UPF identifies the time to next data burst carried in an N6 RTP Header Extension/transport protocol header as defined in TS 26.522 [179] and sends the identified time to next data burst to NG-RAN in a downlink GTP-U header. Time to next data burst marking is only used when the deployment is such that the time to next data burst is sufficiently accurate i.e. the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of the time to next data burst.

It can be seen that the UPF may identify the Data Burst Size and the Time to Next Burst and send the information to NG-RAN in the GTP-U header. This will have impacts on NG-U and F1-U, and possibly Xn-U. To implement the impacts, there could be two alternatives.

Alternative 1: Add Data Burst Size and Time to Next Burst into the DL PDU SET INFORMATION frame defined in TS 38.415.

Alternative 2: Add Data Burst Size and Time to Next Burst into the DL PDU SESSION INFORMATION frame defined in TS 38.415 and the DL USER DATA frame defined in TS 38.425 separately.

It seems Alternative 1 is simpler and has less specification change. However, its disadvantage is obvious as it couples the provision of R19 dynamic traffic characteristics and the provision of R18 PDU Set information. This will restrict the usage of the dynamic traffic characteristics since the DL PDU SET INFORMATION is provided only when the CN receives PDU Set Based Handling Indicator from NG-RAN, which relies on the existence of PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication in the message from SMF to RAN. However, there is no description saying the provision of Data Burst Size and Time to Next Burst requires PDU Set based handling support in SA2 specification. Actually, for QoS flows without PDU set information, the information about Data Burst Size and Time to Next Burst could still be useful for the gNB, e.g., to perform scheduling preparation or better power saving.

Observation 1: The provision of the Data Burst Size and Time to Next Burst should be independent to the provision of DL PDU Set information from the perspective of usage.

It is also noted that SA4 has discussed how to carry the Data Burst Size and Time to Next Burst in the RTP header. According to the latest endorsed CR in SA4 in S4-250394 [4], new RTP header extensions, which are independent on the R18 header extensions for PDU Set information, is added for the marking of dynamically changing traffic characteristics. The formats for the new header extensions and the definitions for Burst Size and Time to Next Burst are excerpted below.

----------Excerption from S4-250394 (endorsed CR for TS 26.522)--------- ### 4.X.2 One-byte RTP header extension format The one-byte RTP HE for the marking of dynamically changing traffic characteristics is defined as follows: 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ 0xBE ¦ 0xDE ¦ length ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ ID ¦ len ¦ R ¦ BSSize +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ TTNB ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ### 4.X.3 Two-byte RTP Header Extension Format The two-byte RTP HE for the marking of dynamically changing traffic characteristics is defined as follows: 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ 0x100 ¦appbits¦ length ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ ID ¦ len ¦ R ¦ BSSize +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ TTNB ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ### 4.X.4 Semantics The semantics of the fields of the RTP HE for marking dynamically changing traffic characteristics are defined as follows: - Reserved [R] 8 bits): This field is reserved for future usage, it shall be set to 0 by the RTP sender and shall be ignored by the RTP receiver. - Burst Size [BSSize] (24 bits): The Burst Size indicates the total size of the burst to be transmitted (in bytes (including the overhead of the RTP Header).). If the burst size is not known it shall be set to 0 NOTE 1: If a packager generates all packets of the burst at once, no additional delay is introduced when setting the burst size, as the packets can be marked with the complete burst size. If this is not the case (e.g. multiple frames combined in one burst) a delay as large as the burst duration could be introduced by marking the entire burst. Therefore, this approach may not be suitable for all types of packagers/encoders, especially those that gradually produce packets additional latency may be introduced if the size is not known in advance. - Time To Next Burst [TTNB] (16 bits): Indicates the approximate time in tenth of milliseconds to the next burst If the time to next burst is not known, it shall be set the reserved value 65535. NOTE: The definition of time to next burst in this context is for further study

Observation 2: SA4 uses new header extensions for the marking of Data Burst Size and Time to Next Burst, the new header extensions are decoupled from the header extension for PDU Set Information.

Based on the above observations, we think it is better that RAN3 can support the way to deliver Data Burst Size and Time to Next Burst independent from the R18 PDU Set information, so that the principle is aligned with SA4. In such way, the dynamic traffic characteristics information can be used more widely, and the benefit of introducing Data Burst Size and Time to Next Burst will be maximized. Thus, we would like to recommend that Alternative 2 to be adopted by RAN3. The specific formats and definitions can refer to SA4’s work. We provide the TPs in annex-1.

Observation 3: The benefit of introducing Data Burst Size and Time to Next Burst will be maximized, if the two parameters can be provided from to RAN regardless of whether the PDU Set Information container present or not.

Proposal 4: Add Data Burst Size and Time to Next Burst into the DL PDU SESSION INFORMATION frame defined in TS 38.415 and the DL USER DATA frame defined in TS 38.425 separately.

2.4 Exposure of Available Data Rate

The solution on network available data rate exposure has been agreed by SA2. The procedure of the solution can be found in the agreed CR for TS 23.501 in S2-2502675 [5].

------------Excerption for S2-2502675 (agreed CR for TS 23.501)------------- 5.45.x Available bitrate monitoring The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation. NOTE 1: How the NG-RAN measures the available bitrate is NG-RAN implementation specific. NOTE 2: The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles. For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets. NOTE 3: The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis. The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed. During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

There are some RAN3 works to align with SA2 on this available data rate monitoring.

- First, the SMF requests NG-RAN to monitor the available data rate and provides reporting threshold(s) to NG-RAN via NGAP.

- Then, when the available data rate crosses the indicated reporting threshold(s), the NG-RAN would report the highest fulfilled threshold to the UPF via NG-U.

- Moreover, during handover, the available data rate monitoring request and the corresponding reporting threshold(s) should be forwarded to the target NG-RAN via XnAP (for Xn-based handover) or NGAP (for NG-based handover).

To be more specific, the RAN3 impacts can be concluded as two aspects: control plane impact and user plan impact. The control plane impacts mainly focus on CP-based configuration to the RAN, which should be implemented in both NGAP and XnAP. Considering the case of split architecture, E1AP and F1AP should also be effected because the gNB-DU is responsible for monitoring and evaluating the available data rate, while the gNB-CU-UP is responsible for reporting the available data rate to the UPF directly.

Proposal 5: Specify the signalling on the configuration of available data rate exposure in NGAP, F1AP, E1AP and XnAP.

For the signalling design, an IE similar to the existing QoS Monitoring Request IE can be added into the GBR QoS Flow information IE to indicate whether available data rate exposure is requested for a GBR QoS flow of UL and/or DL. Besides, when it is requested, the corresponding reporting threshold(s) should be provided as well. A list containing one or more bit rate values can be defined for the configuration of the threshold(s).

Proposal 6: Add IEs for available data rate exposure request and reporting threshold(s) into the GBR QoS Flow information IE.

The user plane impact is the UP-based data rate report to the UPF, which influences NG-U and F1-U in case of CU-DU split architecture. Xn-U may also be considered to support dual-connectivity. The available data rate, i.e. the highest fulfilled reporting threshold, can be carried within the GTP-U headers of the UL packets. Similar to R18 congestion information report, two separate IEs can be defined for UL and DL respectively.

Proposal 7: Include the available data rate in the GTP-U header for UL and DL in NG-U, F1-U and Xn-U.

TPs for UP are provided in annex-2.

3. Conclusion

In this contribution, we analysed miscellaneous issues for XR based on the updated WID, considering the SA2 works impacts on RAN3. The following observations and proposals were made.

DL PDU Set marking without PDU Set QoS:

Proposal 1: RAN3 to enhance NGAP and XnAP to support the provision of DL PDU Set Information Marking Support Indication for a QoS flow.

Proposal 2: RAN3 to reuse the existing PDU Set Based Handling Indicator IE for PDU Set marking without PDU set QoS in Rel-19.

Alternative PDU Set QoS:

Proposal 3: Add the PSDB and PSER in UL and DL as alternative  PDU Set QoS parameters into the Alternative QoS Parameters Set List IE in NGAP, F1AP, E1AP and XnAP.

Dynamic Traffic Characteristics Change:

Observation 1: The provision of the Data Burst Size and Time to Next Burst should be independent to the provision of DL PDU Set information from the perspective of usage.

Observation 2: SA4 uses new header extensions for the marking of Data Burst Size and Time to Next Burst, the new header extensions are decoupled from the header extension for PDU Set Information.

Observation 3: The benefit of introducing Data Burst Size and Time to Next Burst will be maximized, if the two parameters can be provided from to RAN regardless of whether the PDU Set Information container present or not.

Proposal 4: Add Data Burst Size and Time to Next Burst into the DL PDU SESSION INFORMATION frame defined in TS 38.415 and the DL USER DATA frame defined in TS 38.425 separately.

Exposure of Available Data Rate:

Proposal 5: Specify the signalling on the configuration of available data rate exposure in NGAP, F1AP, E1AP and XnAP.

Proposal 6: Add IEs for available data rate exposure request and reporting threshold(s) into the GBR QoS Flow information IE.

Proposal 7: Include the available data rate in the GTP-U header for UL and DL in NG-U, F1-U and Xn-U.

4. References

[1]. RP-250107, 3GPP TSG RAN, Revised WID on XR (eXtended Reality) for NR Phase 3.

[2]. S2-2502678, 3GPP TSG SA WG2, Support Alternative QoS profile with PDU Set QoS and Notification Control .

[3]. 3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]. S4-250394, 3GPP TSG SA WG4, RTP Header Extension for Dynamically Changing Traffic Characteristics.

[5]. S2-2502675, 3GPP TSG SA WG2, KI#9, Support of Available Data Rate.

Annex-1 TPs for Dynamic Traffic Characteristics Change

Annex-1.1 TP for TS 38.415

CHANGES START

5.5.2	Frame format for the PDU Session user plane protocol

5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  | 0 |  |
| PDU Type (=0) |  |  |  | QMP | SNP |  | MSNP |  | Spare | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| PPI |  |  | Spare |  |  | BSSize Ind. |  | TTNB Ind. |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 4 |
| BSSize |  |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

NEXT CHANGE

5.5.3.a	BSSize Ind.

Description: This parameter indicates the presence of BSSize.

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

5.5.3.b	TTNB Ind.

Description: This parameter indicates the presence of TTNB.

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

5.5.3.c	BSSize

Description: The BSSize field indicates the total size of the burst to be transmitted (in bytes). If the burst size is not known it shall be set to 0.

Value range: (0–224-1).

Field Length:  3 octets.

5.5.3.d	TTNB

Description: The TTNB filed indicates the approximate time in tenth of milliseconds to the next burst. If the time to next burst is not known, it shall be set to the reserved value 65535.

Value range: (0–216-1).

Field Length: 2 octets.

CHANGES END

Annex-1.2 TP for TS 38.425

CHANGES START

5.5.2	Frame format for the NR user plane protocol

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  | Spare | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare | BSSize Ind. |  | TTNB Ind. | Request OutofSeq Report | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| BSSize |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

NEXT CHANGE

5.5.3.a	BSSize Ind.

Description: This parameter indicates the presence of BSSize.

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

5.5.3.b	TTNB Ind.

Description: This parameter indicates the presence of TTNB.

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

5.5.3.c	BSSize

Description: The BSSize field indicates the total size of the burst to be transmitted (in bytes). If the burst size is not known it shall be set to 0.

Value range: (0–224-1).

Field Length:  3 octets.

5.5.3.d	TTNB

Description: The TTNB filed indicates the approximate time in tenth of milliseconds to the next burst. If the time to next burst is not known, it shall be set to the reserved value 65535.

Value range: (0–216-1).

Field Length: 2 octets.

CHANGES END

Annex-2 TPs for Available Data Rate Exposure

Annex-2.1 TP for TS 38.415

CHANGES START

5.4.2	Transfer of UL PDU Session Information

5.4.2.1	Successful operation

The purpose of the Transfer of UL PDU Session Information procedure is to send control information elements related to the PDU Session from NG-RAN to UPF.

An UL PDU Session user plane instance making use of the Transfer of UL PDU Session Information procedure is associated to a single PDU Session. The Transfer of UL PDU Session Information procedure may be invoked whenever packets for that particular PDU Session need to be transferred across the related interface instance.

The UL PDU SESSION INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet.

If QoS monitoring has been requested for the included QFI field, the UL PDU SESSION INFORMATION frame may include a QoS Monitoring Packet (QMP) field, a DL Sending Time Stamp Repeated field, a DL Receiving Time Stamp field, a UL Sending Time Stamp field, and/or Delay Result for UL or DL. If QoS monitoring with N3/N9 delay reporting has been requested for the included QFI field, the I-UPF may include in the UL PDU SESSION INFORMATION frame a N3/N9 Delay Ind. field, a N3/N9 Delay Result field and delay result for UL and DL if received from the RAN. The UPF shall, if supported, use this information to calculate UL, DL, or RTT delay as specified in TS 23.501 [5].

The UL PDU SESSION INFORMATION frame may also include a UL QFI Sequence Number field associated with the transferred packet. The UPF shall, if the QoS flow has been configured eligible for redundant transport bearer in TS 38.413 [6], use the received UL QFI Sequence Number field to determine and eliminate duplicated packets for a given QoS flow as specified in TS 23.501 [5].

If the ECN Marking or Congestion Information Reporting Status is active for a QoS flow indicated by the QFI field, the UL PDU SESSION INFORMATION frame should include an UL Congestion Information field and/or a DL Congestion Information field. The UPF shall, if supported, use this information to perform ECN marking at UPF or information exposure as specified in TS 23.501 [5].

If the available data rate report is requested and supported for a QoS flow indicated by the QFI field, the UL PDU SESSION INFORMATION frame may include an UL Available Data Rate field and/or a DL Available Data Rate field. The UPF shall, if supported, use this information to perform available data rate exposure as specified in TS 23.501 [5].

NEXT CHANGE

5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate |  |  |  |  |  |  |  | 0 or 1 |
| DL Available Data Rate |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate is present (1) or not (0).

NEXT CHANGE

5.5.3.x	UL Available Data Rate

Description: This parameter indicates highest fulfilled UL reporting threshold of the available data rate at NG-RAN for the QoS flow, as specified in TS 23.501 [5]. The unit is Mbps.

Value range: {0..255}.

Field length: 1 octets.

5.5.3.y	DL Available Data Rate

Description: This parameter indicates highest fulfilled DL reporting threshold of the available data rate at NG-RAN for the QoS flow, as specified in TS 23.501 [5]. The unit is Mbps.

Value range: {0..255}.

Field length: 1 octets.

CHANGES END

Annex-2.2 TP for TS 38.425

CHANGES START

5.4.3	Transfer of Assistance Information

5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or to further send it to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

The ASSISTANCE INFORMATION DATA frame may include UL Available Data Rate and/or DL Available Data Rate evaluated by the corresponding node. The node hosting the NR PDCP entity shall, if supported, further send it to the UPF for available data rate exposure as specified in TS 23.501 [8].

NEXT CHANGE

5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. | Assistance Info. Ind. | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  | UL Available Data Rate Ind. | DL Available Data Rate Ind. | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate |  |  |  |  |  |  |  | 0 or 1 |
| DL Available Data Rate |  |  |  |  |  |  |  | 0 or 1 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

NEXT CHANGE

5.5.3.a	UL Available Data Rate Ind

Description: This parameter indicates the presence of UL Available Data Rate.

Value range: {0= UL Available Data Rate not present, 1= UL Available Data Rate present}.

Field length: 1 bit.

5.5.3.b	DL Available Data Rate Ind

Description: This parameter indicates the presence of DL Available Data Rate.

Value range: {0= DL Available Data Rate not present, 1= DL Available Data Rate present}.

Field length: 1 bit.

5.5.3.x	UL Available Data Rate

Description: This parameter indicates highest fulfilled UL reporting threshold of the available data rate at NG-RAN for the QoS flow, as specified in TS 23.501 [8]. The unit is Mbps.

Value range: {0..255}.

Field length: 1 octets.

5.5.3.y	DL Available Data Rate

Description: This parameter indicates highest fulfilled DL reporting threshold of the available data rate at NG-RAN for the QoS flow, as specified in TS 23.501 [8]. The unit is Mbps.

Value range: {0..255}.

Field length: 1 octets.

CHANGES END