3GPP TSG-RAN WG3 Meeting #127-bis	R3-251977

# Wuhan, China, 7th – 11th April, 2025

Title: 	On collection of delay measurements for UE performance

Source: 	Ericsson, JIO Platforms

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we further present arguments in context of the discussion in the preceding RAN3 meetings. During RAN3#127, the following agreement was captured:

- Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

- Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

This paper focuses on the above, primarily discussing the collection and signalling of delay measurements as part of UE performance reporting in split architecture.

# 2. Discussion

### 2.1 Average UE Delay in UE performance

During Release 18, RAN3 formalized the Data Collection Reporting Initiation and the Data Collection Reporting procedures for non-split architectures. One of the intended use-cases for the procedures was UE performance collection, where information/feedback could be collected from a neighbour node to which UEs were handed over to and signalled to the source node. The defined UE performance metrics are Average UE Throughput UL, Average UE Throughput DL, Average Packet Delay, and Average Packet Loss DL.

In the context of delay measurements in split architecture, the metrics are defined in TS 28.558. They correspond to the following metrics collected at different entities. In the downlink direction, the delay components consist of the following, as defined in TS 28.558:

- D1: Average delay in DL over air-interface (available at gNB-DU)

- D2: Average delay in DL in gNB-DU (available at gNB-DU)

- D3: Average DL delay over F1-U (available at gNB-CU-UP)

- D4: Average DL delay in CU-UP (available at gNB-CU-UP)

In the uplink direction, the delay components consist of the following, as defined in TS 28.558:

- D1: UL PDCP packet average delay (available at the UE and gNB-CU-CP)

- D2.1: Average over-the-air interface UL packet delay (available at gNB-DU)

- D2.2: Average UL RLC packet delay (available at gNB-DU)

- D2.3: Average UL delay over F1-U (available at gNB-CU-UP)

- D2.4: Average UL PDCP re-ordering delay (available at gNB-CU-UP)

Summing up the earlier agreement in R3#127 with the above discussion, we arrive at the following observation:

Observation 1: The following metrics should be made available at the gNB-CU-UP:

- For the Downlink metrics: D1 and D2. D3 and D4 already available at gNB-CU-UP

- For the Uplink metrics: D2.1 and D2.2. D2.3 and D2.4 already available at gNB-CU-UP

Before discussing the specifics of how to exchange the above metrics, we bring forward a couple of fundamental questions:

- The metrics defined in TS 28.558 are defined for the purpose of QoS monitoring.

  - Question 1: How can such metrics be collected when QoS monitoring is not enabled by the AMF?

  - Question 2: How to enable UE performance collection independent of the QoS monitoring feature?

#### 2.1.1 UE performance collection and QoS monitoring

QoS in NR enables service differentiation and QoS monitoring provides packet delay level observability into the delivered service performance across the 5GS. However, QoS monitoring is an optional feature that is enabled by the Application Function towards the core network, and the core subsequently indicates to the RAN to monitor the QoS of associated QoS flows. The request for QoS monitoring is then propagated across network entities and the corresponding data is collected and reported in a QoS Report.

In the context of UE performance, the request to collect metrics comes from a neighbour gNB, where the configuration to perform the UE performance collection is done as part of a Xn: Data Collection Reporting Initiation procedure and the request to monitor the UE is done as part of the Xn: Handover procedure. The difference between the UE performance case when compared to the QoS monitoring case is that the request to monitor a certain UE’s performance can be independent of a QoS monitoring request for the same UE/QoS flow. Even in cases where both QoS monitoring and UE performance co-exist, it may happen that the reporting periodicity may be different for each procedure. In such a case, the RAN node receiving either the QoS monitoring measurements or the UE performance measurements should be able to differentiate them, such that the RAN measurements for UE performance can be configured and signalled to gNB-CU-CP and the QoS monitoring measurements can be configured and signalled to the UPF.

Fundamentally, the chosen solution should satisfy the following:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance ecase), or (legacy) per-UE UE performance metrics without additional complexity

Proposal 1: RAN3 to discuss solutions for UE performance according to the following principles:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance case), or (legacy) per-UE UE performance metrics without additional complexity

#### 2.2.2 Mechanism to enable RAN-based metric collection for UE performance

In order to enable a RAN-based framework for the collection of delay measurements for UE performance, a similar mechanism as used for QoS monitoring is required, with the difference that the mechanism is controlled by RAN-based entities.

Below we propose a mechanism for RAN-based collection of UE performance metrics for split architecture. The trigger to collect UE performance is always associated to an incoming handover at the gNB-CU. Provided that the UE is admitted, and the handover executes without failures, the target gNB-CU sets up resources for the UE at the gNB-DU and the gNB-CU-UP.

Towards the gNB-DU, the initial setup of the DRBs corresponding to the UE is performed over the F1: UE CONTEXT SETUP procedure. If the DRB being setup corresponds to a UE for which UE performance is being requested by the source gNB, then the target gNB-CU is aware of this (via the corresponding configuration of UE performance reporting as part of the Xn: DATA COLLECTION REPORTING INITIATION and via the presence of the corresponding Data Collection ID in the Handover Request message). In order to make the gNB-DU aware of the need to gather UE performance metrics, the gNB-CU-CP may, as part of the F1: UE CONTEXT SETUP, indicate to the gNB-DU the need to monitor delay metrics for UE performance reasons in addition to the delay metrics reporting periodicity (note that we only mention delay metrics here as this paper discusses only delay measurements in UE performance). Below, we present the tabular of the UE CONTEXT SETUP message with the suggested enhancements.

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Delay Monitoring for UE Performance Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated DRB. | YES | ignore |
| >>>>>Delay Monitoring for UE Performance Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Indicates the periodicity of reporting Delay metrics for UE performance | YES | ignore |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

The presence of the highlighted additional IEs triggers the gNB-DU to report delay metrics to the gNB-CU-UP with the specified reporting periodicity over the F1-U Transfer of Assistance Information procedure (as it is the case with QoS monitoring).

Proposal 2: Extend the F1-C UE CONTEXT SETUP REQUEST message with two IEs (Delay monitoring for UE performance and Reporting frequency for UE performance)

Similar to the discussion above, the gNB-CU-CP triggers the BEARER CONTEXT SETUP REQUEST message towards the gNB-CU-UP to setup the DRBs associated to the PDU Session. In order to make the gNB-CU-UP aware of the need to gather UE performance metrics and the need to aggregate and report the RAN delay metrics to the gNB-CU-CP, the gNB-CU-CP may, as part of the BEARER CONTEXT SETUP, indicate to the gNB-CU-UP of the need to monitor delay metrics for UE performance reasons (possibly in addition to the delay metrics reporting periodicity). The gNB-CU-CP may also indicate the procedure over which the collected metrics shall be reported by specifying a corresponding ID, e.g., measurement IDs corresponding to a Resource Status Reporting procedure. Below, we present the tabular of the E1: BEARER CONTEXT SETUP message with the suggested enhancements.

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - |  |
| >PDU Session ID | M |  | 9.3.1.21 |  | - |  |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to "DRB". | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >>>DAPS Request Information | O |  | 9.3.1.91 |  | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | Indicates that the target gNB-CU-CP requests QoS flow remapping during an intra-system lossless handover as specified in TS 38.300 [4]. | YES | reject |
| >>>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates that the DRB is for SDT. | YES | reject |
| >>>Special Triggering Purpose | O |  | ENUMERATED (indirect-data-forwarding, …) |  | YES | ignore |
| >>>Delay Monitoring for UE Performance Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated DRB. | YES | ignore |
| >>>Delay Monitoring for UE Performance Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Indicates the periodicity of reporting Delay metrics for UE performance | YES | ignore |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

The presence of the additional IEs triggers the gNB-CU-UP to collect gNB-CU-UP-based delay metrics and also be prepared to receive the delay measurements from the gNB-DU. The overall RAN delay measurement is then aggregated and signalled to the gNB-CU-CP at the specified reporting periodicity over the E1 interface.

Proposal 3: Extend the E1 BEARER CONTEXT SETUP REQUEST message with two IEs (Delay monitoring for UE performance and Reporting frequency for UE performance)

The final component of the solution is signalling the delay measurements aggregated at the gNB-CU-UP to the gNB-CU-CP. As part of the release 19 discussions on the split architecture, two options have been discussed, i.e., extending the Resource Status Reporting procedure or introducing Data Collection Reporting Initiation and Data Collection Reporting procedure on the E1 interface.

Our view of the above alternatives is that introducing the Data Collection Reporting procedures on E1 AP is too disruptive, considering that the new procedure will only be used to exchange UE performance related data and not used to exchange e.g. inferred data. Therefore, we propose the following.

Proposal 4: Extend the E1 Resource Status Reporting Initiation message with a new Report Characteristics bit corresponding to UE delay measurements and corresponding configuration for UE performance collection.

Proposal 5: Extend the E1 Resource Status Reporting Update message with IEs corresponding to the delay measurements aggregated at the gNB-CU-UP. This IE shall be a UE associated IE.

Changes to E1 AP include changes to both Bearer Context Setup procedures and Resource Status procedures.  Changes to the Bearer Context related procedures have been shown above. Below we detail the changes to the Resource Status Reporting Initiation message.

#### 9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,…) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,…) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(36)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Periodic. Third Bit = Average Packet Delay. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | Ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.xxx. |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful UE context setup within which the UE performance measurements are collected. Unit: millisecond |

Below are the enhancements that are needed to the Resource Status Reporting Update message. Followed by the newly introduced IE group’s definition.

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  | YES | ignore |
| HW Capacity Indicator | O |  | 9.3.1.73 |  | YES | Ignore |
| UE Associated Info Result list |  | 0..1 |  |  | YES | Ignore |
| >UE Associated info Result Item |  | 1 .. < maxnoofUEReports > |  |  | - |  |
| >> E1AP UE Assistant Identifier | M |  | RANUE ID |  | - |  |
| >> DRB Assistant Identifier | M |  | DRB ID 9.3.1.16 | Unique DRB identifier | - |  |
| >> Delay measurements for UE performance | O |  | x.x.x.xxx |  | - |  |

#### x.x.x.xxx Delay measurements for UE performance

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Average RAN delay in UL. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Average RAN delay in DL. Unit: 0.1 millisecond |

#### 2.2.3 Aggregation of delay components

The delay measurements defined in TS 28.558 Sec. 6.3.1.1, and the corresponding MDT measurements defined in TS 37.320 are defined per QoS level, or per S-NSSAI, or at a granularity of per-DRB- per UE.

UE performance may be collected for the whole UE (legacy case) or on a per network slice level (per slice UE performance), or on a per DRB level (UE Performance reporting from SN to MN for NR-DC). RAN3 in meeting #124 agreed that the UE performance per UE is derived by average over the per-DRB measurements of different metrics corresponding to the UE.

UE level performance is derived as the average value of per DRB level measurements of the UE.

Following a similar approach, the delay measurements included in the per slice UE Performance can be derived by averaging the UE performance for all DRBs associated to the same network slice.

Equivalently, the delay measurements included in the UE Performance at the SN (for NR-DC cases) can be derived by averaging the UE performance for all the SN terminated bearers.

Hence, the core of the proposed solution for delay averaging is to measure and signal per-DRB-level delay measurements from the gNB-CU-UP to the gNB-CU-CP. The advantage of signaling per-DRB-level metric is the following:

-	In the context of Network slicing use-case, per-slice UE performance metrics may need to be derived for slice-level performance observability. The gNB-CU-CP is aware of the relationship between the DRBs of a UE to the network slice that the DRB was mapped to. In order to derive a network slice level delay measurement, the gNB-CU-CP may aggregate the delay measurements per DRBs (received from the gNB-CU-UP) corresponding to the particular slice of interest.

-	In the context of the NR-DC, UE performance for the UE at the SN may be required. However, the same mechanism as described in this contribution may be used by the SN to collect UE performance metrics of all SN terminated bearers. The SN may then report the measured values at a per-DRB level granularity to the MN. The MN can, from this information, understand the delay part of the UE performance at the SN.

Observation 2: The proposed mechanism to collect delay measurements for UE performance applies equally well to the Network slicing and NR-DC use cases.

In conclusion, delay measurements included in the UE Performance metrics can be collected on a per DRB basis.

Proposal 6: RAN3 to agree to average on a pr DRB basis the delay measurements included in the UE Performance

# 3. Conclusion

# This paper makes the following observations and proposals:

Observation 1: The following metrics should be made available at the gNB-CU-UP:

- For the Downlink metrics: D1 and D2. D3 and D4 already available at gNB-CU-UP

- For the Uplink metrics: D2.1 and D2.2. D2.3 and D2.4 already available at gNB-CU-UP

Proposal 1: RAN3 to discuss solutions for UE performance according to the following principles:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance case), or (legacy) per-UE UE performance metrics without additional complexity

Proposal 2: Extend the F1-C UE CONTEXT SETUP REQUEST message with two IEs (Delay monitoring for UE performance and Reporting frequency for UE performance)

Proposal 3: Extend the E1 BEARER CONTEXT SETUP REQUEST message with two IEs (Delay monitoring for UE performance and Reporting frequency for UE performance)

Proposal 4: Extend the E1 Resource Status Reporting Initiation message with a new Report Characteristics bit corresponding to UE delay measurements and corresponding configuration for UE performance collection.

Proposal 5: Extend the E1 Resource Status Reporting Update message with IEs corresponding to the delay measurements aggregated at the gNB-CU-UP. This IE shall be a UE associated IE.

Observation 2: The proposed mechanism to collect delay measurements for UE performance applies equally well to the Network slicing and NR-DC use cases.

Proposal 6: RAN3 to agree to average on a pr DRB basis the delay measurements included in the UE Performance

A TP reflecting the above proposals is available in R3-251978 (F1) and R3-251979 (E1).