| 3GPP TSG-RAN WG3 Meeting #127-bis |  |  |  | R3-252055 |
| --- | --- | --- | --- | --- |
| Wuhan, China | 7-11 April 2025 |  |  |  |
| Agenda Item: | 19.2 |  |  |  |
| Title: | CLI mitigation and CLI testing procedures using new Xn and F1 messages and procedures. |  |  |  |
| Document for: | Discussion and decision |  |  |  |

# Introduction

In previous meetings RAN3 has agreed to exchange information over Xn interface and F1 interface regarding:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity

- Measurement resource configuration

- Strongest DL beam information

- CLI mitigation request

Although a gNB is not obligated to respond to a CLI mitigation request, we should have the procedures in place, so that if in a particular deployment or situation, the gNB wants to respond to the CLI mitigation request and to cooperate to improve the interference situation, there is a standardized way of doing that.

This contribution addresses CLI mitigation and testing procedures.  It supports CLI mitigation techniques of beam nulling and beam pairing, and provides capabilities for CLI measurement results to be reported on an event basis, or periodic basis.

# Discussion

This contribution proposes new Xn messages CLI MITIGATION REQUEST, CLI MITIGATION RESPONSE, CLI TEST REQUEST, CLI TEST RESPONSE and CLI MEASUREMENT RESULT and corresponding matching F1 messages of the same name.

## CLI mitigation

The CLI MITIGATION REQUEST message contains the following IE (besides the obvious “Message Name” IE):

- A “Transaction ID” IE which allows a gNB to distinguish between multiple concurrent requests with another gNB

- An “Action” IE, which identifies one or more actions such as “Start”, “Stop”;

- A “Cell_Beam_CLI_Measurement” IE which identifies a particular cell / beam of the victim gNB that is experiencing CLI, informs the aggressor about the CRI that was detected and provides the CLI power that was measured.

- A “My_Cell_Beam_CLI_Measurement_Configuration” IE, which may be sent if so desired to indicate a different CLI measurement configuration from the previously configured CLI measurement configuration sent at the time of XN SETUP REQUEST / XN SETUP RESPONSE and/or NG-RAN NODE CONFIGURATION UPDATE / NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGMENT messages.

Proposal 1: A victim gNB1 can request an aggressor gNB2 to start CLI mitigation action by sending a  CLI MITIGATION REQUEST message with:

- a “Transaction ID”, which allows the victim gNB to distinguish among multiple concurrent requests (possibly for multiple cells / beams of the victim gNB1 that are experiencing CLI from the aggressor gNB2).

- the “Action” IE set to “Start”,

- the “Cell_Beam_CLI_Measurement” IE which identifies the particular cell / beam of the victim gNB1, the CRI (NZP-CSI-RS Indicator) that was detected in the CLI, and provides the CLI power measured.

- the “My_Cell_Beam_CLI_Measurement_Configuration” IE, if different from the previously one configured.

Proposal 2: A gNB2 receiving a CLI MITIGATION REQUEST message with the “Action” IE set to “Start” shall respond with a CLI MITIGATION RESPONSE with:

- the “Transaction ID” IE set to the same value as in the received CLI MITIGATION REQUEST,

- the “Action” IE set to “Start” if it agrees to start applying CLI mitigation actions.

- It shall also include a “Reporting Interval” IE, to request the victim gNB1 to report CLI measurement based on an event such as the CLI measurement power reaching a particular value, or based on periodic intervals of time.  This will allow the aggressor to be aware of how effective (or ineffective) the measures it has taken have been.

- It may also include the “My_Cell_Beam_CLI_Measurement_Configuration” IE, to report its CLI measurement configuration if different from the previously one configured.

Proposal 3: If a gNB2 receives a CLI MITIGATION REQUEST message with the “Action” IE set to “Start”, but cannot or does not want to cooperate with the requesting gNB1, it shall respond with a CLI MITIGATION RESPONSE message with:

- the “Transaction ID” IE set to the same value as in the received CLI MITIGATION REQUEST,

- with the “Action” IE set to the value “Denied”.

Proposal 4: If gNB1 receives a CLI MITIGATION RESPONSE message with the “Action” IE set to “Start”, it shall consider that gNB2 has agreed to carry out the mitigation request associated with the matching “Transaction ID” IE received. It shall take note of the received “Reporting Interval” IE, to prepare for sending CLI measurement results, as requested. It shall also take note of the information possibly sent by gNB2 informing about any change of gNB2’s CLI measurement configuration.

Proposal 5: At the reporting interval requested by a previously received Xn CLI MITIGATION RESPONSE, gNB1 shall send a Xn CLI MEASUREMENT RESULT message with:

- the “Transaction ID” IE set to the same value associated with the corresponding Xn CLI MITIGATION REQUEST

- the “Cell_Beam_CLI_Measurement” IE.

Proposal 6: A gNB1 that is satisfied with the CLI mitigation provided by gNB2, shall send a CLI MITIGATION REQUEST message with:

- a “Transaction ID” IE matching the “Transaction ID” of the original request to start mitigation.

- the “Action” IE set to the value “stop”.

Proposal 7: A gNB2 that has started applying mitigation measures, may at any point indicate that it will stop any further mitigation measures by sending a CLI MITIGATION RESPONSE message with:

- the “Transaction ID” ID matching the “Transaction ID” of the original request to start mitigation.

- the “Action” IE set to “Stop”.

### Applying CLI mitigation in different scenarios

If we assume that both a gNB1 that is a victim, and a gNB2 that is an aggressor, have been previously configured by Xn SETUP REQUEST / Xn SETUP RESPONSE with the information of the particular NZP-CSI-RS configuration being used by their respective cells, then both are already transmitting those signals.

In particular, gNB1, could already be measuring the CLI interference in its cells, and realized that it needs to request gNB2 to apply CLI mitigation techniques to lessen the effect on a particular cell/beam of gNB1. So it sends a CLI MITIGATION REQUEST message to gNB2 providing information on which of its cell / beam the interference is excessive, even providing the actual measured CLI power, and the CRI index.  If gNB2 decides to cooperate with gNB1, it responds with a CLI MITIGATION RESPONSE message to start the process and indicates to gNB1 that it wants to receive CLI measurement reports every time that CLI interference power is reduced by 3dBm, for example.  gNB2 could decide to apply different mitigation techniques.  It could for example decrease its transmission power, or it could decide to apply beam pairing, or beam nulling.

#### gNB2 decides to use beam pairing

gNB2 avoids DL scheduling the identified CSI-RS beam during SBFD slots.  Using the feedback provided by gNB1 with the CLI measurement results, gNB2 can progressively increase the number of SBFD slots that it avoids DL scheduling, so that perhaps at a certain point gNB1 is satisfied with the level of mitigation, and stops the CLI mitigation request.

#### gNB2 decides to use beam nulling

gNB2 estimates the victim-to-aggressor channel by measuring the CLI-RS of the victim gNB.  If the DL transmission uses the identified aggressor beam, gNB2 adjusts the DL precoder. Again, the CLI measurement results being provided by the victim gNB (as requested by the aggressor) help the aggressor understand how effective (or ineffective) the steps taken are.

## CLI test

Proposal 8: A gNB2 that wants to be proactive and wants to start a CLI testing with a neighboring gNB1 in order to find out how a particular configuration of its power level, antenna setting, etc might impact gNB1, can request a CLI testing by sending a CLI TEST REQUEST message with:

- a “Transaction ID”, which allows gNB1 to distinguish among multiple concurrent requests with  gNB2.

- the “Action” IE set to the value “start”,

- the “Cell_Beam_to_Report” IE indicating the particular cell and beam of the neighbor gNB1 that gNB2 is requesting to measure and report the CLI interference,

- possibly including the “CLI measurement configuration” IE to indicate to gNB2 any different, CLI measurement configuration previously configured.

- Including a “CLI Reporting Interval” IE, indicating if the reporting will be event-based, or every indicated period of time.

Proposal 9: If a gNB1 receives a CLI TEST REQUEST message from gNB2 and it agrees to carry out the test, it shall take note of the received “Reporting Interval” IE, to prepare for sending CLI measurement results, as requested. It shall also take note of the information possibly sent by gNB2 informing about any change of gNB2’s CLI measurement configuration. It shall respond with a CLI TESTING RESPONSE message with:

- the “Transaction ID” IE set to the same value as that of the CLI TEST REQUEST,

- the “Action” IE set to “Start”.

Proposal 10: If a gNB1 receives a CLI TEST REQUEST message from gNB2 and it does not agree to carry out the test, it shall respond with a CLI TEST RESPONSE message with:

- the “Transaction ID” IE set to the same value as that received in the CLI TEST REQUEST,

- the “Action” IE set to the value “Denied”.

Proposal 11: If a gNB2 receives a CLI TEST RESPONSE message from gNB1 with the “Transaction ID” IE, and the “Action” IEs matching a previously sent CLI TEST REQUEST, it shall start the testing.

Proposal 12: At the reporting interval requested by gNB2, gNB1 shall send to gNB2 a CLI MEASUREMENT RESULT message that includes the “Cell_Beam_CLI_Measurement” IE.

Proposal 13: If gNB2 receives a CLI TEST RESULT message from gNB1 containing test results for a previously requested CLI testing campaign sent to gNB1, it can consider the test results for deciding the next action it takes regarding the reported level of CLI interference.

Proposal 14: If gNB2 decides to stop a testing that it had previously started, it shall send to gNB1 a CLI TEST REQUEST with the “Action” IE set to “stop”.

Proposal 15: If gNB1 receives a CLI TEST REQUEST message with a “Transaction ID” IE and the “Action” IE with the value “Stop”, it shall stop sending any further testing results back to gNB2.

## Handling split gNB architecture

New F1 messages corresponding to the new Xn messages, are defined in Annex B to cater for the case in which a gNB with split architecture is involved.

### General

Proposal 16: In the case of a split architecture gNB, a gNB-CU that has received a Xn CLI MITIGATION REQUEST, CLI MITIGATION RESPONSE, CLI TEST REQUEST, CLI TEST RESPONSE, CLI MEASUREMENT RESULT message, shall send to the proper gNB-DU the corresponding F1 message of the same name, respectively, with the proper F1 “Transaction ID” IE, and with the rest of IEs of the message set to the same values of those received in the corresponding Xn message.

Proposal 17: In the case of a split architecture gNB, a gNB-CU that has received a F1 CLI MITIGATION REQUEST, CLI MITIGATION RESPONSE, CLI TEST REQUEST, CLI TEST RESPONSE, CLI MEASUREMENT RESULT message from a gNB-DU, shall send to the appropriate gNB, the corresponding Xn message of the same name, respectively, with the proper Xn “Transaction ID” IE, and the rest of the IEs of the message set to the same values as in the corresponding F1 received messages.

### CLI Mitigation

Proposal 18: A victim gNB1-DU can request an aggressor gNB2 to start CLI mitigation action by sending a F1 CLI MITIGATION REQUEST message with:

- a “Transaction ID”, which allows the victim gNB1-DU to distinguish among multiple concurrent requests (possibly for multiple cells / beams of the victim gNB1-DU that are experiencing CLI from the aggressor gNB2).

- the “Action” IE set to “Start”,

- A “Cell_Beam_CLI_Measurement” IE which identifies a particular cell / beam of the victim gNB1-DU that is experiencing CLI, informs the aggressor about the CRI that was detected and provides the CLI power that was measured.

- the “My_Cell_Beam_CLI_Measurement_Configuration” IE, if different from the previously one configured.

Proposal 19: A gNB2-DU receiving a F1 CLI MITIGATION REQUEST message with the “Action” IE set to “Start” shall respond with a F1 CLI MITIGATION RESPONSE with:

- the “Transaction ID” IE set to the value of the “Transaction ID” received in the F1 CLI MITIGATION REQUEST,

- the “Action” IE set to “Start” if it agrees to start applying CLI mitigation actions.

- It shall also include a “Reporting Interval” IE, to eventually request the victim gNB1 to report CLI measurement based on an event such as the CLI measurement power reaching a particular value, or based on periodic intervals of time.  This will allow the aggressor to be aware of how effective (or ineffective) the measures it has taken have been.

- It may also include the “My_Cell_Beam_CLI_Measurement_Configuration” IE, to report its CLI measurement configuration if different from the previously one configured.

Proposal 20: If a gNB2-DU receives a F1 CLI MITIGATION REQUEST message with the “Action” IE set to “start”, but cannot or does not want to cooperate with the requesting gNB1, it shall respond with a F1 CLI MITIGATION RESPONSE message, with the proper “Transaction ID” IE and the “Action” IE set to the value “Denied”.

Proposal 21: If gNB1-DU receives a F1 CLI MITIGATION RESPONSE message with the “Action” IE set to “Start”, it shall consider that gNB2 has agreed to carry out the mitigation request associated with the matching “Transaction ID” IE received. It shall take note of the received “Reporting Interval” IE, to prepare for sending CLI measurement results, as requested. It shall also take note of the information possibly sent by gNB2 informing about any change of gNB2’s CLI measurement configuration.

Proposal 22: A gNB1-DU that is satisfied with the CLI mitigation provided, shall send a F1 CLI MITIGATION REQUEST message with

- a “Transaction ID”, matching the “Transaction ID” of the original request to start mitigation.

- the “Action” IE set to the value “stop”,

Proposal 23: A gNB2-DU that has started applying mitigation measures, may at any point indicate that it will stop any further mitigation measures by sending a F1 CLI MITIGATION RESPONSE message with the appropriate “Transaction ID” and the “Action” IE set to “Stop”.

### CLI TESTING

Proposal 24: A gNB2-DU that wants to be proactive and wants to start a CLI testing with a particular cell / beam of a neighboring gNB1 in order to find out how a particular configuration of its power level, antenna setting, etc might impact gNB1, can request a CLI testing by sending a F1 CLI TESTING REQUEST message with:

- a “Transaction ID”, which allows gNB2-DU to distinguish among multiple concurrent requests with  gNB2-CU,

- the “Action” IE set to the value “Start”,

- the “Cell to Report” IE indicating the particular cell / beam of the neighbor gNB1 (identified by its Cell Global ID and SSB index) that gNB2-DU is requesting to measure and report the CLI interference,

- possibly including the “CLI measurement configuration” IE to indicate any different gNB2-DU CLI measurement configuration than what was previously configured.

- Including a “CLI Reporting Interval” IE, indicating if the reporting will be event-based, or every indicated period of time.

Proposal 25: If a gNB2-DU receives a F1 CLI TESTING REQUEST message from gNB2-CU and it agrees to carry out the test, it shall take note of the received “Reporting Interval” IE, to prepare for sending CLI measurement results, as requested. It shall also take note of the information passed along by gNB2-CU informing about any change of gNB1’s CLI measurement configuration. The gNB2-DU shall respond to gNB2-CU with a F1 CLI TESTING RESPONSE message with:

- the “Transaction ID” IE set to the same value as that of the received F1 CLI TESTING REQUEST,

- the “Action” IE set to “Start”.

Proposal 26: If a gNB2-DU receives a F1 CLI TESTING REQUEST message from gNB2-CU and it does not agree to carry out the test, it shall respond with a F1 CLI TESTING RESPONSE message with:

- the “Transaction ID” IE set to the same value as that received in the F1 CLI TESTING REQUEST,

- the “Action” IE set to the value “Denied”.

Proposal 27: If a gNB2-DU receives a F1 CLI TESTING RESPONSE message from gNB1 with the “Transaction ID” IE matching a previously sent F1 CLI TESTING REQUEST, and the “Action IE” set to “Start” it shall start the testing.

Proposal 28: At the reporting interval requested by a previously received F1 CLI TESTING REQUEST, gNB1-DU shall send a F1 CLI TEST RESULT message with:

- the “Transaction ID” IE set to the same value as received in the F1 CLI TESTING REQUEST

- the “Action” IE set to “Report”

- the “Cell_Beam_CLI_Measurement” IE, with the measured CLI power.

Proposal 29: If gNB2-DU receives a F1 CLI MEASUREMENT RESULT message from gNB2-CU containing test results for a previously requested CLI testing campaign, it can consider the test results for deciding the next action it takes regarding the reported level of CLI interference.

Proposal 30: If gNB2-DU decides to stop a CLI testing that it had previously requested, it shall send  to gNB2-CU a F1 CLI TESTING REQUEST with:

- The “Transaction ID” IE set to the “Transaction ID” of the original  F1 CLI TESTING REQUEST,

- the “Cell_Beam_CLI_Measurement” IE with the information about the cell / beam and the respective CLI power measured.

Proposal 31: If gNB2-DU receives a F1 CLI TESTING REQUEST message with a “Transaction ID” IE and the “Action” IE with the value “Stop”, it shall stop sending any further testing results back to gNB2-CU for the test associated with the received “Transaction ID”.

Annex A – Changes to XnAP (TS38.423)

9.1.3.a	CLI MITIGATION REQUEST

This message is sent by a victim gNB1 to an aggressor gNB2 to initiate the requested CLI mitigation procedures.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  | YES | ignore |
| Action | M |  | ENUMERATED(Start, Stop, Deny) |  | YES | ignore |
| Cell_Beam_CLI_Measurement | C-ifStart |  | 9.2.2.a |  | YES | ignore |
| My_Cell_Beam_CLI_Measurement_ Configuration | C-ifStart_&_ConfigChange |  | 9.2.2.b |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart | This IE shall be present if the “Action” IE has the value “Start” |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.b CLI MITIGATION RESPONSE

This message is a response to a CLI_MITIGATION_REQUEST message to indicate to the victim that the aggressor has accepted the request for mitigation.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  | YES | ignore |
| Action | M |  | ENUMERATED (Start, Stop, Deny) |  | YES | ignore |
| My_Cell_Beam_CLI_ Measurement_ Configuration | C-ifStart_&_ConfigChange |  | 9.2.2.b |  | YES | ignore |
| Reporting Interval | C-ifStart |  | 9.2.2.c |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart | This IE shall be present if the “Action” IE has the value “Start” |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.c	CLI TEST REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  | YES | ignore |
| Type | M |  | ENUMERATED (gNB_to_gNB, UE_to_UE, …) |  | YES | ignore |
| Action | M |  | ENUMERATED(Start, Stop, Deny) |  | YES | ignore |
| Cell_to_report | M |  | 9.2.2.e |  | YES | ignore |
| My_Cell_Beam_CLI_ Measurement_Configuration | C-ifStart_&_ConfigChange |  | 9.2.2.b |  | YES | ignore |
| Reporting Interval | C-ifStart |  | 9.2.2.c |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart | This IE shall be present if the “Action” IE is set to the value “Start” |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.d CLI TEST RESPONSE

This message is a response to a CLI TEST REQUEST message to indicate that the request was accepted.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  | YES | ignore |
| Action | M |  | ENUMERATED( Start, Stop, Deny) |  | YES | ignore |
| My_Cell_Beam_CLI_ Measurement_Configuration | C-ifStart_&_ConfigChange |  | 9.2.2.b |  | YES | ignore |
| Reporting Interval | M |  | 9.2.2.c |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.e CLI MEASUREMENT RESULT

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the CLI measurement.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  |  | ignore |
| Cell_Beam_CLI_Measurement | M |  | 9.2.2.a |  | YES | ignore |

--------------------------------Next Change--------------------------------

9.2.2.a Cell_Beam_CLI_Measurement

This IE contains information about the CLI power measured at a particular cell and beam (optionally) of the victim gNB, as well as the respective detected CRI and SSB index.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |
| CRI | M |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value associated with the detected CLI. CRI is an index indicating a CSI-RS resource within a set of resources |
| CLI Power detected | M |  | INTEGER (0..X) | Number of dBm of CLI power |

9.2.2.b My_Cell_Beam_CLI_Measurement_Configuration

This IE identifies the particular NZP-CSI-RS configuration associated with the cell / beam of the gNB sending this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-CGI | M |  | 9.2.2.7 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |
| NZP-CSI-RS Configuration | M |  | 9.2.2.d |  |

9.2.2.c Reporting Interval

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Type | M |  | ENUMERATED (Event-based, Periodic) |  |
| CLI Power Initial Triggering Value | C-ifEvent-based |  | INTEGER (1 .. N) in dBm | For fist time,trigger event if current value >= initial triggering value |
| CLI Power Upper Histerisys Value Delta | C-ifEvent-based |  | INTEGER (1 .. N) in dBm | Trigger event if current value = last value+upper histerisys |
| CLI Power Lower Histerisys Value Delta | C-ifEvent-based |  | INTEGER (1 .. N) in dBm | Trigger event if current value = last value-lower histerisys |
| Period | C-ifPeriodic |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) |  |

| Condition | Explanation |
| --- | --- |
| ifEvent-based | This IE shall be present if the “Type” IE is set to the value “Event-based” |
| ifPeriodic | This IE shall be present if the “Type” IE is set to the value “Periodic” |

9.2.2.d NZP-CSI-RS Configuration

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  |  |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resoures. Value is 64. |

9.2.2.e Cell_to_report

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

Annex B – Changes to F1 AP (TS38.473)

9.2.1.a	CLI MITIGATION REQUEST

This message is sent by gNB-CU to gNB-DU to request Cross-Link Interference (CLI) mitigation according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| Action | M |  | ENUMERATED(Start, Stop, Deny) |  | YES | ignore |
| Cell_Beam_CLI_Measurement | C-ifActionStart |  | 9.3.1.a |  | YES | ignore |
| My_Cell_Beam_CLI_Measurement_Configuration | C-ifStart_&_ConfigChange |  | 9.3.1.b |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifActionStart | This IE shall be present if the Action IE is set to the value “Start” |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.2.1.b CLI MITIGATION RESPONSE

This message is a response to a previous CLI-related request indicating the request was accepted.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| Action | M |  | ENUMERATED (Start, Stop, Deny) |  | YES | ignore |
| My_Cell_Beam_CLI_Measurement_Configuration | CifStart_&_ConfigChange |  | 9.3.1.b |  | YES | ignore |
| Reporting Interval | C-ifStartDifConfig |  | 9.3.1.c |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStartDifConfig | This IE shall be present if the “Action” IE has the value “Start”, and a reference signal different from what had been previously configured (either at Xn setup, or using Xn NG-RAN NODE CONFIGURATION UPDATE) is desired. |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.c	CLI TEST REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| Type | M |  | ENUMERATED (gNB_to_gNB, UE_to_UE, …) |  | YES | ignore |
| Action | M |  | ENUMERATED(Start, Stop, Deny) |  | YES | ignore |
| Cell_to_report | M |  | 9.3.1.e |  | YES | ignore |
| My_Cell_Beam_CLI_ Measurement_Configuration | C-ifStart_&_ConfigChange |  | 9.3.1.b |  | YES | ignore |
| Reporting Interval | C-ifStart |  | 9.3.1.c |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart | This IE shall be present if the “Action” IE is set to the value “Start” |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.1.3.d CLI TEST RESPONSE

This message is a response to a CLI TEST REQUEST message to indicate that the request was accepted.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | INTEGER (0…32767) |  | YES | ignore |
| Action | M |  | ENUMERATED(Start, Stop, Deny) |  | YES | ignore |
| My_Cell_Beam_CLI_ Measurement_Configuration | C-ifStart_&_ConfigChange |  | 9.3.1.b |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifStart_&_ConfigChange | This IE shall be present if the “Action” IE has the value “Start”, and a new CLI measurement configuration is desired. |

9.2.1.e	CLI MEASUREMENT RESULT

This message is sent by gNB-DU to gNB-CU to report the result of a CLI test measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| Cell_Beam_CLI_Measurement | M |  | 9.3.1.a |  | YES | ignore |

--------------------------------Next Change--------------------------------

9.3.1.a Cell_Beam_CLI_Measurement

This IE contains information about the CLI power measured at a particular cell and optionally a beam, as well as the detected CRI and SSB index

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |
| CRI | M |  | INTEGER (1..64) | Includes the CSI-RS Resource Indicator (CRI) value associated with the detected CLI. CRI is an index indicating a CSI-RS resource within a set of resources |
| CLI Power detected | M | [0..X] | INTEGER (0..X) | Number of dBm of CLI power |

9.3.1.b My_Cell_Beam_CLI_Measurement_Configuration

This IE identifies the particular NZP-CSI-RS configuration associated with the cell / beam of the gNB sending this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |
| NZP-CSI-RS Configuration | M |  | 9.3.1.d |  |

9.3.1.c Reporting Interval

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Type | M |  | ENUMERATED (Event-based, Periodic) |  |
| CLI Power Initial Triggering Value | C-ifEvent-based |  | INTEGER (1 .. N) | For fist time,trigger event if current value >= initial triggering value |
| CLI Power UpperHisterisys Value Delta | C-ifEvent-based |  | INTEGER (1 .. N) | Trigger event if current value = last value+upper histerisys |
| CLI Power LowerHisterisys Value Delta | C-ifEvent-based |  | INTEGER (1 .. N) | Trigger event if current value = last value-lower histerisys |
| Period | C-ifPeriodic |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) |  |

| Condition | Explanation |
| --- | --- |
| ifEvent-based | This IE shall be present if the “Type” IE is set to the value “Event-based” |
| ifPeriodic | This IE shall be present if the “Type” IE is set to the value “Periodic” |

9.3.1.d NZP-CSI-RS Configuration

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  |  |
| >NZP-CSI-RS-Resource Item |  | 1 .. <maxnoofNZP-CSI-RS-RESOURCE> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNZP-CSI-RS-RESOURCE | Maximum no. of NZP-CSI-RS-Resources.Value is 64. |

9.3.1.e Cell_to_report

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 | NR Cell Global Identifier |
| SSB Index | O |  | INTEGER (0..63) | Identifier of the SSB beam. |

--------------------------------End of Changes--------------------------------