3GPP TSG-RAN WG3 Meeting #127bis	R3-252077

Wuhan, China, 7 – 11 April, 2025

Agenda item:	11.2

Source:	Nokia

Title:	AI/ML Slicing Open Issues

Document for:	Discussion and Decision

# 1	Introduction

In the last meeting we made the following agreement with respect to the situation where predicted radio resource status per slice or predicted slice available capacity cannot be initiated:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

In addition another open point was captured based on contributions in the previous meeting namely:

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

Despite the progress on slice-level load reporting, Stage 3 details of slice-level UE Performance are still missing. The following open aspect remains:

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

In addition, even though an agreement was made that UE Performance per S-NSSAI as defined in SA5 can be used in RAN3 for UE Performance feedback other granularity of UE Performance is not excluded and may be further discussed:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

In this contribution we provide our views regarding remaining open aspects such as:

- Slice-level load reporting

- Slice-level UE Performance including triggering the reporting and the respective granularity

# 2	Discussion

## 2.1	Slice-Level Load Reporting

In this section, we discuss some remaining aspects pertaining to slice-level load reporting. In RAN3 #126 we agreed to introduce Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message. We chose to reuse the existing Slice Available Capacity IE for encoding of the reported predictions. However, one issue that arises with this encoding is the lack in the reported value of the Cell Capacity Class Value IE which is used to scale the slice capacity with respect to the total capacity of the cell in case multiple slices are available for a cell.

One simple way to address this issue without standards impacts is to request Slice Available Capacity (using legacy Resource Status Procedures) every time Predicted Slice Available Capacity (using Data Collection Reporting procedures) is requested. Since in legacy Resource Status Reporting procedures Slice Available Capacity is always reported together with Composite Available Capacity the reported Cell Capacity Class Value will provide the available resources of the cell in the uplink and downlink directions to be used for the Predicted Slice Available Capacity.

Observation 1: One way to determine the Cell Capacity Class Value used to scale the cell capacity of the Predicted Slice Available Capacity IE is to also request Slice Available Capacity using legacy Resource Status Reporting Procedures (which is reported together with the Composite Available Capacity).

Even though this is a feasible solution it requires to always trigger both Data Collection Reporting and Resource Status Reporting procedures in order to deduce the used Cell Capacity Class Value required for the scaling of the reported Predicted Slice Available Capacity. This is not an efficient solution.

Observation 2: Triggering both Data Collection Reporting and Resource Status Reporting procedures in order to determine the reported Predicted Slice Available Capacity is inefficient.

Other more efficient approaches exist, but they would require making some changes into the agreements from the previous meeting. One approach is to append in the reporting of Predicted Slice Available Capacity the Predicted Composite Available Capacity, in order to imitate legacy behavior. This would provide the needed information to the receiving node to determine the scaling factor used in the calculation of the Predicted Slice Available Capacity. This approach introduces some redundancy in the reporting as it introduces a new IE (Predicted Composite Available Capacity IE) that contains additional information, not needed to interpret Predicted Slice Available Capacity is the Cell Capacity Class Value. We are open to discuss this approach due to its simplicity and similarities to legacy reporting.

Proposal 1: RAN3 to discuss pros and cons of the approach where Predicted Slice Available Capacity is reported together with Predicted Composite Available Capacity in the DATA COLLECTION UPDATE message.

An alternative approach is to introduce a new IE for Predicted Slice Available Capacity and within this IE introduce the missing Cell Capacity Class Value IEs hence making the new IE self-contained.

Proposal 2: Define a new self-contained IE for Predicted Slice Available Capacity including Cell Capacity Class Value Downlink IE and Cell Capacity Class Value Uplink IE.

A final aspect relates to the criticality of some of the introduced IEs. In the last meeting, we agreed to introduce the Slice Measurement Initiation Result IE to indicate that the predicted radio resource status per slice or predicted slice available capacity measurements cannot be initiated. No criticality was assigned to this IE but in our view, this is due to omission. Since Slice Measurement Initiation Result IE is inside Cell Measurement Initiation Result List IE which has criticality reject, we believe that Slice Measurement Initiation Result IE should have the same criticality.

Proposal 3: Update the assigned criticality of Slice Measurement Initiation Result IE to “reject”.

## 2.2	Slice-Level UE Performance

### 2.2.1	Slice-Level UE Performance Configuration

The second topic we discuss in this paper pertains to slice-level UE Performance reporting configuration. In earlier meetings, we have agreed to support finer granularity of UE performance feedback in a handover scenario to determine UE performance in the target cell for a certain slice. However, so far there haven’t been any stage 3 details agreed. One aspect that remains open is how a source triggers slice-level UE Performance reporting towards a target node. Two approaches have been discussed in the previous meeting:

- Option 1: Introduce new bits in the Report Characteristics for Data Collection IE to request slice level UE Performance measurements from a target node: In this approach, instead of 4 bits indication in the Report Characteristics for Data Collection IE corresponding to Average Throughput UL, Average Throughput DL, Average Packet Delay and Average Packet Loss, the proponents of this solution introduce 4 additional bits to request the corresponding slice-level UE Performance measurements.

- Option 2: Introduce a new IE to indicate that slice level UE Performance is requested from a target node: In this case, the 4 bits in the Report Characteristics for Data Collection IE and an additional indication is used to indicate that the requested UE Performance measurements from a target node (i.e., Average Throughput UL, Average Throughput DL, Average Packet Delay and Average Packet Loss) are on a per slice granularity. This additional indication can be done by enhancing the UE Performance Collection Configuration IE with an indication that Slice Level UE Performance Reporting is requested.

We think that different granularities of UE Performance measurements do not really correspond to new measurements (but rather to different counters being measured). So being exact it is not strictly correct to assign a bit for different counter configurations in the Report Characteristics for Data Collection IE, and such approach could also lead to an exploding number of bits to be used for configuration if support of additional granularities is added in the future.

On the other hand, there is another alternative indicated through Option 2. In this case, the existing bits of UE performance are reused, but the granularity of the reporting is provided through an additional IE in the UE Performance Collection Configuration IE. Different encodings of this IE can be envisioned.

- It can be an enumerated type taking values true or false i.e., {true,…}. When the value is true, slice level UE Performance Reporting is requested.

- It can be of enumerated type taking values {slice level, UE level, both, …}. In this case, the requesting node can request from a target node different granularities of UE Performance including both UE level and slice level granularities.

Proposal 4: Introduce Slice Level UE Performance Reporting IE to indicate that slice level UE Performance Reporting is requested from a target node when a UE Performance-specific bit in Report Characteristics for Data Collection IE is set.

A UE requested to report slice-level UE Performance, reports UE Performance for all its active slices. A question that arises is on the behaviour when a UE performance cannot be calculated per slice at the target node. If UE Performance on a per slice-level granularity is requested but UE does not have any active slices then slice-level UE Performance is not reported.

Proposal 5: If UE at the target node has no active slices, slice-level UE Performance is not reported from target node to source node.

### 2.2.1		Granularity of slice-level UE Performance Reporting

One aspect that remains open from several meetings pertains to the granularity of slice-level UE Performance reporting.

In the last meeting we made the following agreement:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback.

In addition, an open point was captured: Other granularity of UE performance are not excluded and to be continued based on cases ...

Checking SA5 specification TS 28.558 we notice that indeed measurements for UE Performance can be reported on a slice level granularity namely

- Delay Measurements

  - Average delay DL air-interface: This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface in the downlink direction. It is measured at the gNB-DU. The measurement name has the form DRB.AirIfDelayDlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.1.

  - Average delay DL in gNB-DU: This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. The measurement name has the form DRB.RlcSduDelayDlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.2.

  - Average delay DL on F1-U: This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpF1DelayDlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.3.

  - Average delay DL in CU-UP: This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpSduDelayDlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. It is described in TS 28.558, clause 6.3.1.1.4.

Similarly, in the uplink:

  - Average delay UL on over-the-air interface: This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2 (and in TS 28.558, clause 6.3.1.1.6). It is measured at the gNB-DU. The measurement name has the form DRB.AirIfDelayUlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average RLC packet delay in the UL: This measures RLC packet delay in the UL, i.e., the delay within the gNB-DU. It is defined in TS 38.314, clause 4.2.1.2.3 (and in TS 28.558, clause 6.3.1.1.7). It is measured at the gNB-DU. The measurement name has the form DRB.RlcDelayUlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured using the same metric as the average delay DL on F1-U defined in TS 28.558, clause 6.3.1.1.3. It is measured by the gNB-CU-UP. The measurement name has the form DRB.PdcpF1DelayUlUe.Filter,  where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

  - Average PDCP re-ordering delay in the UL: It measures PDCP re-ordering delay in the UL, i.e., delay within the gNB-CU-UP. It is defined in TS 38.314, clause 4.2.1.2.4 (and in TS 28.558, clause 6.3.1.1.8). It is measured at the gNB-CU-UP. The measurement name has the form DRB.PdcpReordDelayUlUe.Filter, where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

- Average Throughput UL/DL

- Average UL UE throughput in gNB: This measurement provides the average UE throughput in DL. It is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU. This measurement is measured by the gNB-DU. It is described in TS 28.558, clause 6.3.1.4.2.

The measurement name has the form

DRB.UEThpUl,or optionally 
DRB.UEThpUl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

- Average DL UE throughput in gNB: This measurement provides the average UE throughput in DL. It is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU. It is described in TS 28.558, clause 6.3.1.4.1.

The measurement name has the form 
DRB.UEThpDl, or optionally 
DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

- Average Packet Loss DL

- DL F1-U Packet Loss Rate: This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU. It is a measure of the DL packet loss on the F1-U interface. This measurement is measured by the gNB-DU. It is defined in TS 28.558, Clause 6.3.1.3.3.

The measurement name has the form DRB.F1UpacketLossRateDlUe, and optionally DRB.F1UPacketLossRateDlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

From the above, we notice that UE Performance measurements may be referred in TS 28.558 with slice granularity, however the corresponding reporting is done on a per DRB per slice basis and not per full (aggregated) S-NSSAI.

Observation 3: UE Performance measurements in TS 28.558 are measured on a per DRB per slice granularity and not on a per (aggregated) S-NSSAI granularity.

The problem that was identified with per DRB reporting during previous meetings was that the target node may remap the DRBs. To address this issue we propose to report UE Performance over the group of QoS flows corresponding to the DRB in the reporting (target) node.

Proposal 6: For the Rel-19 slicing use case, upon request for slice level UE Performance Feedback from the source, the target node reports UE Performance per group of QoS flows (comprising a DRB) for all PDU sessions associated to one or more S-NSSAIs.

# 3	Conclusion

In this paper we make the following observations and proposals:

Observation 1: One way to determine the Cell Capacity Class Value used to scale the cell capacity of the Predicted Slice Available Capacity IE is to also request Slice Available Capacity using legacy Resource Status Reporting Procedures (which is reported together with the Composite Available Capacity).

Observation 2: Triggering both Data Collection Reporting and Resource Status Reporting procedures in order to determine the reported Predicted Slice Available Capacity is inefficient.

Proposal 1: RAN3 to discuss pros and cons of the approach where Predicted Slice Available Capacity is reported together with Predicted Composite Available Capacity in the DATA COLLECTION UPDATE message.

Proposal 2: Define a new self-contained IE for Predicted Slice Available Capacity including Cell Capacity Class Value Downlink IE and Cell Capacity Class Value Uplink IE.

Proposal 3: Update the assigned criticality of Slice Measurement Initiation Result IE to “reject”.

Proposal 4: Introduce Slice Level UE Performance Reporting IE to indicate that slice level UE Performance Reporting is requested from a target node when a UE Performance-specific bit in Report Characteristics for Data Collection IE is set.

Proposal 5: If UE at the target node has no active slices, slice-level UE Performance is not reported from target node to source node.

Observation 3: UE Performance measurements in TS 28.558 are measured on a per DRB per slice granularity and not on a per (aggregated) S-NSSAI granularity.

Proposal 6: For the Rel-19 slicing use case, upon request for slice level UE Performance Feedback from the source, the target node reports UE Performance per group of QoS flows (comprising a DRB) for all PDU sessions associated to one or more S-NSSAIs.