3GPP TSG-RAN WG3 #127bis	R3-252147

Wuhan, CN, 7 – 11 Apr. 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Discussion on LP-WUS

Document for:	Discussion, agreement

Introduction

In past meetings, RAN3 achieved following:

[RAN3#125]

- New IEs should be introduced to support LP-WUS on NG, Xn and F1.

[RAN3#125bis]

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

[RAN3#126]

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

[RAN3#127]

- For the following issues, including Extended UE Identity Index、The maximum value of CN subgroup ID、Emergency call back、the terminology of “LP-WUS”、 the LP-WUS node capability need more inputs from other WGs.

This document discusses the issues captured at the previous meeting.

Discussion

Extended UE Identity Index

In the last RAN2 meeting, the following agreement was reached on how to derive the subgroup ID for UE_ID based subgrouping.

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

However, how many bits the Extended UE Identity Index should be extended is not decided in RAN2, and also, the extension of UE_ID may have security issues which was raised in the previous meeting [1]. Therefore, RAN3 should wait for the final decision and LS from RAN2 on the extension of the Extended UE Identity Index before introducing a new IE in RAN3 specs. If RAN2 does not have any requirement on how many bits the Extended UE Identity Index should be extended but just requires extending UE_ID, whether this IE should be extended some bits only for LP-WUS or more bits for future proof.

The potential security issue with further extension of the Extended UE Identity Index is that each UE has a finer UE_ID, which makes it more unique and can track a specific UE. In our understanding, if the uniqueness of UE_ID is an issue, it comes from the number of subgroups for LP-WUS and formula of subgroup ID rather than Extended UE Identity Index. Also, even if this issue is valid, extension of Extended UE Identity Index itself is not an issue because this IE is exchanged only within the NW and it is transferred via secure F1/Xn/NG interfaces. If attacker can track a specific UE, it would be done based on tracking energy saving behavior of the UE including eDRX, PEI, and LP-WUS, and estimate the subgroup ID of the UE, but not track Extended UE Identity Index directly. Therefore, this should be discussed in RAN2 (or RAN1) and there is no issue from RAN3 perspective.

Proposal 1:	RAN3 waits to work on extension of Extended UE Identity Index IE until we receive LS from RAN2 regarding final decision on subgroup ID formula and extension of UE_ID.

Proposal 2:	Once extension of Extended UE Identity Index IE is confirmed, RAN3 to discuss whether the new IE should be extended some bits only for LP-WUS or more bits for future proof.

Emergency call back during LP-WUS

Over several meetings, the wake up delay that occurs when UE goes to low power state and has an active emergency PDU session was discussed. In addition, RAN2 sent an LS [2] to confirm SA2's understanding of the operation of emergency call during eDRX, and SA2 sent reply LS [3]. This LS exchange concerns eDRX, which allows the NW to indicate whether to use eDRX to the UE via RRC. On the other hand, LP-WUS is a function in which the UE autonomously evaluates the entry/exit condition and turns the function on/off according to the SIB. This is one of the differences between eDRX and LP-WUS. In addition, LS is a confirmation of how the gNB recognizes the PDU session associated with the emergency service when the UE have it. This is a different scenario than emergency call back during LP-WUS. It was found in the reply LS from SA2 that the gNB can recognize the presence of the emergency PDU session based on the appropriate deployment/implementation and the current spec. However, whether a mechanism to avoid the transition to low power state during active emergency PDU session is necessary is still an open issue.

In an emergency call, the paging message is sent via normal PDU session, and the NW does not know whether the UE maintains the emergency PDU session, and UE can not predict the emergency call back in advance. Therefore, it is impossible for the NW to make it sure that UE is not in low power state and latency for emergency call back would be avoided.

The issue we want to address here is whether the NW should introduce a solution to avoid the wake up delay when an emergency PDU session has been established and the UE is expected to maintain it and an emergency call back may occur. Regardless of whether CN based subgrouping or UE_ID based subgrouping, the UE cannot not avoid the wake up delay if UE’s implementation does not care about possibility emergency call back and goes to low power state in such case. One solution is that gNB sends paging for such time sensitive call at the first PO regardless of the UE’s LP-WUS capability based on NW implementation and the UE avoid entering low power state based on UE implementation. Another solution is to activate/deactivate the LP-WUS function of the UE from the NW (RAN or CN) explicitly and let the NW make it sure that UE is not in low power state. The SIB broadcasts whether LP-WUS can be used in the cell, but since this problem is UE dedicated, it is necessary to introduce a new UE dedicated signaling. This would be discussed in RAN2 first because main impacts on specs are expected to be on signaling between NW and UE and on UE behavior. RAN3 can work on this issue if it is acknowledged by RAN2 and they decide to introduce any solution involving RAN3 spec impact.

Proposal 3:	The issue on emergency call back during LP-WUS should be still pending on RAN2 and RAN3 cannot work based on the reply LS from SA2.

LP-WUS capability issue within non-homogeneous deployment

In the previous meeting, LS [4] from SA2 raised the issue of mixed environments with gNBs of Rel-19 or later and gNBs of Rel-18 or earlier. RAN2 discussed this in the previous meeting, and it was agreed that this issue does not occur in environments where only gNBs of Rel-17 or later are mixed, and a reply LS was sent to SA2 [5]. On the other hand, there was no conclusion regarding environments where gNBs of Rel 15/16 are mixed.

In our view, environments where gNBs of Rel 15/16 are mixed should also be considered. This problem can be avoided if all gNBs can be guaranteed to be updated with Rel-19 ASN.1 encoding regardless of the release version of the gNB. On the other hand, especially in multi-vendor environments, it may be difficult to guarantee that all gNBs can encode ASN.1 of Rel-19 because the release update schedule and the ASN.1 encoding upgrade method are different among vendors. Therefore, if an initial registration is performed on a gNB of Rel 15/16 and then the UE canp on a LP-WUS capable cell, the UE may transition to the low power state, but the gNB may not know that the UE has LP-WUS capability.

The main directions to resolve this issu are following:

- gNB sends WUS and then performs paging in LP-WUS cell regardless of UE capability.

- In addition to SIB indication and entry/exit condition, new conditions are added to the conditions under which UE transitions to low power state (e.g. LP-WUS can be turned only in case that the last serving cell is LP-WUS capable.).

- No standardized solution

At least from the operator's point of view, we believe that some solution should be specified. The first solution is possible based on NW implementation without any specification impact, but the LP-WUS capable UE may frequently wake-up due to the WUS transmissions, which is not for the UE, thus this option leads degradation of the energy saving performance of LP-WUS. The second solution requires further discussion on the new conditions, which should be discussed in RAN2 since they are mainly related to Uu signaling and UE behavior.

Proposal 4:	The issue on emergency call back during LP-WUS should be still pending on RAN2 and RAN3 cannot work based on the reply LS from SA2.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 waits to work on extension of Extended UE Identity Index IE until we receive LS from RAN2 regarding final decision on subgroup ID formula and extension of UE_ID.

Proposal 2:	Once extension of Extended UE Identity Index IE is confirmed, RAN3 to discuss whether the new IE should be extended some bits only for LP-WUS or more bits for future proof.

Proposal 3:	The issue on emergency call back during LP-WUS should be still pending on RAN2 and RAN3 cannot work based on the reply LS from SA2.

Proposal 4:	The issue on emergency call back during LP-WUS should be still pending on RAN2 and RAN3 cannot work based on the reply LS from SA2.

References

- R3-250598, “Potential RAN3 impacts of LP-WUS”, CATT

- R3-250006, “LS on emergency services and eDRX”, RAN2

- R3-251521, “Reply LS on emergency call back and paging”, SA2

- R3-250021, “LS Reply on LP-WUS subgrouping”, SA2

- R3-251515, “Reply LS on LP-WUS subgrouping”, RAN2