3GPP TSG-RAN WG3#127 	R3-250059

Athens, Greece, February 17-21, 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-CU LTM Dual Connectivity

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. RAN3#126: RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details…

This paper discusses additional considerations to take into account for the Inter-CU LTM procedure and support for NR-DC.

# 2	Discussion on Inter-CU LTM DC

## 2.1	General Aspects

Scenarios for NR-DC with LTM

The following are NR-DC scenarios applicable for Rel-19 LTM DC work scope:

- Inter-CU SCG LTM with unchanged MCG

- Inter-CU MCG LTM with SCG retained or released

- Inter-CU MCG LTM with SCG addition (after SCG release)

Note:

- RAN2 has confirmed the support of Inter-CU MCG LTM with SCG Addition.

- RAN2 has also confirmed the support of coexistence of a) Inter-CU MCG LTM and Intra-CU SCG LTM and b) Inter-CU SCG LTM and Intra-CU MCG LTM, which needs to be considered when defining RAN3 signaling.

Observation 1: RAN3 needs to consider co-existence of a) inter-CU MCG LTM with intra-CU SCG LTM and b) inter-CU SCG LTM with Intra-CU MCG LTM.

Simultaneous Inter-CU MCG LTM and inter-CU SCG LTM Configuration

There was an agreement at RAN3#123bis meeting that inter-CU MCG LTM and inter-CU SCG LTM are not to be configured simultaneously.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Given that Inter-CU LTM is configured by a gNB-CU based on L3 measurements, there is no guarantee that e.g., the SN is aware that Inter-CU MCG LTM has been configured at the MN, since the SN addition may have occurred prior to Inter-CU MCG LTM being configured. Thus, the SN may independently decide to configure Inter-CU SCG LTM unless it is explicitly aware that Inter-CU MCG LTM is already configured at the MN. A similar scenario can occur in the opposite direction. Therefore, Inter-CU LTM being configured at both MCG and SCG simultaneously needs to be avoided.

One approach for avoiding simultaneous Inter-CU LTM configuration at the MN and SN is to have an indication over Xn that informs the other NG-RAN node that Inter-CU LTM is to be configured. With this information, the receiving node can avoid configuring Inter-CU LTM simultaneously at both the Cell Groups. Likewise, a similar indication would be signalled upon Inter-CU LTM being released.

Proposal 1: In case of NR-DC, MN indicates to SN about Inter-CU MCG LTM configuration/de-configuration.

Proposal 2: In case of NR-DC, SN indicates to MN about Inter-CU SCG LTM configuration/de-configuration.

## 2.2	Inter-CU SCG LTM DC

Support for Scenario: Inter-CU SCG LTM with unchanged MCG

RAN2#127 agreed to support only SN initiated Inter-CU SCG LTM in Rel-19.

Unchanged MCG may include a) PCell unchanged during Inter-CU SCG LTM, or b) Intra-CU PCell change during Inter-CU SCG LTM. In both cases, it is likely that each candidate SN prepares a different SCG configuration with reference to the  SCG configuration at the source, which may result in bearer remapping (e.g. remapping of SCG bearers to the MCG). However, in our view the existing Rel-18 framework should be able to handle the bearer remapping aspect.

Observation 2: The support of Inter-CU SCG LTM with unchanged MCG includes the coexistence of Inter-CU SCG LTM with Intra-CU MCG LTM, which may result in bearer remapping (e.g. remapping of SCG bearers to the MCG), can be handled by the existing Rel-18 framework.

Inter-CU SCG LTM preparation

RAN2 has agreed to only support SN initiated inter-CU SCG LTM preparation. Thus, MN initiated Inter-CU SCG LTM is excluded from the scope. The signalling flow for SN initiated Inter-CU SCG LTM scenario is captured in the figure below.

As part of existing S-CPAC functionality, the S-CPAC Request IE was introduced in the SN ADDITION REQUEST message to indicate that the SN change is for S-CPAC. A similar approach can be used to enable inter-CU SCG LTM, where a specific IE within the SN ADDITION REQUEST message indicates to the candidate target SN that the preparation is for inter-CU SCG LTM.

Proposal 3: Introduce a new IE in SN CHANGE REQUIRED and SN ADDITION REQUEST message to indicate that the SN change is for Inter-CU SCG LTM.

Figure 1: SN Initiated Inter-CU SCG LTM procedure

## 2.3	Inter-CU MCG LTM DC

Inter-CU MCG LTM with SCG retained or released or SCG added (after earlier SCG release)

Regarding coexistence of Inter-CU MCG LTM with Intra-CU SCG LTM, the Inter-CU LTM PCell change will result in security configuration update at the SN, requiring the UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Observation 3: Inter-CU LTM PCell change results in security configuration update at the SN and requires UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Given that it is the decision of candidate MN during HO preparation to retain or release the SCG, some of the Inter-CU MCG LTM candidate nodes during HO preparation may prepare a MCG+SCG configuration (when source SN is retained) or a MCG only configuration (when source SN is released). Hence, this may result into a combination of inter-CU MCG LTM preparation with SCG retained and SCG released.

If the Inter-CU LTM PCell change results in PCell HO with SCG release at the target, then SCG may be added during the subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either full SCG configuration or SCG reference + delta configuration.

Proposal 4: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either a full SCG configuration or SCG reference + delta configuration.

RAN2 has confirmed the support of SCG Addition (after SCG release) in the case of Inter-CU MCG LTM. An SCG Addition or a PSCell Addition during subsequent Inter-CU MCG LTM execution (after SCG release during earlier Inter-CU MCG execution) may result into a potential blind PSCell addition if the network does not consider latest PSCell measurements to identify the suitable PSCell at the time of inter-CU MCG LTM execution. Further, if there is more than one candidate target PSCells prepared associated with the Inter-CU candidate target PCell, considering the latest UE reported radio measurements for candidate PSCells is essential to identify the suitable candidate PSCell and the associated LTM Candidate Configuration ID to avoid user experience and throughput degradation after SCG addition.

Proposal 5: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

Inter-CU MCG LTM when candidate node prepares a different PSCell

In Inter-CU MCG LTM, it is possible that a candidate MN prepares a different PSCell even if it retains the source SN. Thus, both scenarios should be considered in the scope of Inter-CU MCG LTM. Likewise, handling would vary as follows.

Scenario 1: Candidate MN retains the source PSCell: Even though the PSCell remains unchanged after Inter-CU MCG LTM execution, the UE must perform reconfiguration with sync as SN security key changes due to MN change.

Scenario 2: Candidate MN prepares a different PSCell: Intra-CU PSCell HO is triggered, as a result of Inter-CU MCG LTM PCell change.

In scenario 2 above, given that the Intra-CU PSCell change (or Intra-CU SCG LTM) may be resulting from Inter-CU MCG LTM execution, the network must ensure it considers the latest PSCell measurements while triggering PSCell HO.

Proposal 6: In the case of Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

## 2.4	Other Issues

Data Forwarding in Inter-CU LTM DC scenarios

In Rel-18 S-CPAC, data forwarding may be realized either as direct data forwarding or as indirect data forwarding. The same principles and framework may be reused for Inter-CU SCG LTM scenarios. However, since the new serving SN is unaware of the configuration at other candidate SNs, some considerations are needed to enable data forwarding for Inter-CU SCG LTM with subsequent cell change.

Transferring the Admitted PDU session resources configuration of each prepared candidate node to all the prepared candidate Inter-CU SCG LTM nodes via XnAP to enable data forwarding after each Inter-CU SCG LTM PSCell change.

- Option 1: At LTM preparation, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes to all the prepared candidate SNs.

- Option 2: At the time of inter-CU SCG LTM PSCell change execution, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes only to the next serving SN. The new serving SN would carry out the same procedure at the next inter-CU SCG LTM mobility execution.

Observation 3: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 7: To support direct and indirect data forwarding for Inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared Inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Configuration Mismatch during LTM Cell Switch

During initial preparation of candidate target node, the UE’s source cell configuration is used to prepare the target cell configuration for the prepared candidate target nodes. Given that the configuration at the UE after the initial preparation may vary, by the time of LTM cell switch or HO execution, the candidate target cell configuration prepared during the initial LTM preparation may not be aligned with the latest configuration at the UE. The handling of such a problem may require reconfiguring the UE at the time of cell switch or HO execution. However, to support subsequent LTM, the UE reconfiguration after cell change must be avoided. We propose RAN3 to discuss this issue and consider signalling enhancements for handling mismatch in the PDU session and DRB configuration between the serving and the target cells.

Proposal 8: RAN3 to discuss possible solutions to address the issue related to the handling of configuration mismatch between the serving and the target cells after initial preparation and at the time of HO execution to support subsequent LTM cell change.

# 3	Conclusions

General considerations

Observation 1: RAN3 needs to consider co-existence of a) inter-CU MCG LTM with intra-CU SCG LTM and b) inter-CU SCG LTM with Intra-CU MCG LTM.

Proposal 1: In case of NR-DC, MN indicates to SN about Inter-CU MCG LTM configuration/de-configuration.

Proposal 2: In case of NR-DC, SN indicates to MN about Inter-CU SCG LTM configuration/de-configuration.

Inter-CU SCG TLM

Observation 2: The support of Inter-CU SCG LTM with unchanged MCG includes the coexistence of Inter-CU SCG LTM with Intra-CU MCG LTM, which may result in bearer remapping (e.g. remapping of SCG bearers to the MCG), can be handled by the existing Rel-18 framework.

Proposal 3: Introduce a new IE in SN CHANGE REQUIRED and SN ADDITION REQUEST message to indicate that the SN change is for Inter-CU SCG LTM.

Inter-CU MCG LTM

Observation 3: Inter-CU LTM PCell change results in security configuration update at the SN and requires UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Proposal 4: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either a full SCG configuration or SCG reference + delta configuration.

Proposal 5: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

Proposal 6: In the case of Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

Other issues

Observation 3: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 7: To support direct and indirect data forwarding for Inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared Inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Proposal 8: RAN3 to discuss possible solutions to address the issue related to the handling of configuration mismatch between the serving and the target cells after initial preparation and at the time of HO execution to support subsequent LTM cell change.

# References

- 3GPP TS 38.401

- RP-242356 NR mobility enhancements Phase 4, Intel (moderator)