3GPP TSG-RAN WG3 Meeting #127	R3-250089

Athens, Greece, 17-21 February 2025

Agenda Item:		11.4

Source:		Ofinno

Title:	Mobility Optimization for NR-DC

Document for:		Discussion and decision

# Introduction

After RAN3 #126 meeting, the following progress has been achieved on the Mobility Optimization for NR-DC within the agenda item 11.4:

Mobility Optimization for NR-DC

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

RAN3#126:

Mobility Optimization for NR-DC

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Other information, e.g., predicted SCG bearer?

This contribution discusses introducing PSCell ID in DC procedure.

# Discussion

In RAN3 #126 meeting, introducing the predicted PSCell ID into DC procedures (e.g., SN addition, SN change) has been discussed. No consensus has been reached so far.

The MN may use its AI/ML model to predict the UE trajectory. The MN may use such prediction to select a target SN for SN addition or change. UE trajectory prediction is cell-level granularity, which assumes that the MN has very good idea about potential PSCell and/or SCG configuration for the UE. Once such inference results are available, it is not clear why they shall be discarded instead of sending them to the SN to assist in selecting PSCell for the UE / configuring SCG for the UE.

Providing this and maybe additional assistance information from the MN to the SN may be a future compatible solution taking into account that more inference results may be available at the MN in future as RAN2 progresses with AI/ML for mobility work (e.g., SHO, RLF, ToS, measurement predictions).

In RAN3 #126 meeting discussion, it was mentioned that in legacy behaviour, PSCell selection / SCG configuration is done by SN, not MN. As a possible compromise, the MN may send to the SN several PSCell / SCell predicted candidates for the SN to take into account during PSCell/SCell selection. This may be further enhanced by adding priority/preference information to each predicted candidate PSCell/SCell to assist SN in selecting PSCell / configuring SCG for the UE.

Proposal 1: MN sends to SN predicted PSCell / SCell candidates during DC procedures (e.g., SN addition, MN initiated SN change) for a UE. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

Proposal 2: For predicted PSCell / SCell candidates, MN may send to SN priority/preference of selecting a predicted candidate PSCell / SCell. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

# Proposals

Proposal 1: MN sends to SN predicted PSCell / SCell candidates during DC procedures (e.g., SN addition, MN initiated SN change) for a UE. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

Proposal 2: For predicted PSCell / SCell candidates, MN may send to SN priority/preference of selecting a predicted candidate PSCell / SCell. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

# Reference

[1] R3-250001, RAN3#127 Meeting Agenda

[2] R3-250002, RAN3#126 Meeting Report

[3] R3-247788, Discussion on AI/ML Mobility Optimization