3GPP TSG RAN WG3 Meeting #127	R3-250118

Athens, Greece, 17 – 21 Feb, 2025

Title: 	Discussion on RAN Architecture

Source: 	ZTE Corporation

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN #106 plenary meeting, the WID (New Work Item: Solutions for Ambient IoT (Internet of Things) in NR) has approved in RP-243326.

The RAN3’s scope is as below.

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This paper addresses to discuss the general content for RAN Architecture and Procedures

- Discussion

Issue 1: AIoTF information in either NGAP or by carrying a new protocol layer over NGAP

TR38.769 The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued. NOTE 2: Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision.

In our view, AIoTF information in NGAP (like SMF or MB-SMF) is simpler than by carrying a new protocol layer over NGAP (like NRPPa). More, for 6G future proof, we also think both QUIC and NGAP can be reused in 6G network.
Proposal 1: RAN3 to enhance NGAP other than to carry a new protocol layer for support of A-IoT.

Proposal 1a: The following Interface Management Procedures can be enhanced for A-IoT

- NG Setup

- RAN Configuration Update

- AMF Configuration Update

- NG Reset

- Error Indication

Issue 2: Whether to deploy one kind of AIoT RAN supporting AIoT function only

For this kind of AIoT RAN, it can only support AIoT function, but it cannot support NR function, e.g., Handover function, UE context manage function.

Proposal 2: RAN3 to confirm whether to deploy one kind of AIoT RAN which supports AIoT function only.

Furthermore, for these AIoT dedicated RAN nodes, if the existing Interface Management Procedures are enhanced, then more or less “mandatory” information elements used for NR service shall be ignored.

In our view, for the first release of AIoT function, it is better to introduce a clean Interface Management Procedures to support AIoT function only.

Proposal 2a: If the RAN supporting AIoT only is existed, the following new Interface Management Procedures are introduced.

- AIoT NG Setup

- AIoT RAN Configuration Update

- AIoT AMF Configuration Update

- AIoT NG Reset

- AIoT Error Indication

Issue 2: Radio resource coordination

TR38.769 The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers. NOTE 4: How to perform coordination of the usage of the A-IoT radio resources among readers needs further discussion.

In the WID, only T1 is included. So that, within a certain area, all of the readers (e.g., TRP) controlled by an AIoT RAN node. It means that the AIoT RAN can fully control and allocate radio resources for each of readers.

Proposal 3: RAN3 does not need to specify the radio resource coordination for T1, which is left to network implementation.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: RAN3 to enhance NGAP other than to carry a new protocol layer for support of A-IoT.

Proposal 1a: The following Interface Management Procedures can be enhanced for A-IoT

- NG Setup

- RAN Configuration Update

- AMF Configuration Update

- NG Reset

- Error Indication

Proposal 2: RAN3 to confirm whether to deploy one kind of AIoT RAN which supports AIoT function only.

Proposal 2a: If the RAN supporting AIoT only is existed, the following new Interface Management Procedures are introduced.

- AIoT NG Setup

- AIoT RAN Configuration Update

- AIoT AMF Configuration Update

- AIoT NG Reset

- AIoT Error Indication

Proposal 3: RAN3 does not need to specify the radio resource coordination for T1, which is left to network implementation.