3GPP TSG-RAN WG3 Meeting #127	R3-250126

Athens, GR, 17-21 Feb, 2025

Title: 	(TP for TS 38.413 and TS 38.423) Discussion on Low-power wake-up signal and receiver for NR

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# 1. Introduction

The previous RAN3-126 meeting made the following agreements on the LP-WUS signal and receiver in [1].

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

In this contribution, we continue to discuss the RAN3 related issues on the procedure and configuration for RRC_IDLE/INACTIVE UEs.

# 2. Discussion

## 2.1 LP-WUS Subgrouping

- In RAN1-118bis meeting [2], the maximum number of subgroups for LP-WUS was set to 32 as Working Assumption.

Working Assumption at RAN1# 128bis The maximum number of subgroups per PO supported in Rel-19 is 32.

Once RAN1 confirms this WA to be agreement, RAN3 can set the maximum number of LP-WUS CN-based subgroup ID as 32.

RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as 32 if this WA is agreed by RAN1.

In the current spec, the gNB will be provided by the core network or other gNB with an IE of Extended UE Identity Index Value carried in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, or in the XnAP RAN Paging message.

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

The Extended UE Identity Index Value IE comprises 16 least significant bits of 5G-S-TMSI (i.e., UE ID), in which up to 12 bits are to determine the PF and PO (if configured with eDRX) and up to 15 bits (if configured with eDRX) are used for PEI subgrouping as specified in TS 38.304 [4].

In the previous RAN2-128 meeting, RAN2 discussed the design of UE ID based subgrouping, and concluded three options as follows.

Agreement at RAN2#128 FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

For Option 3, if the formula of UE_ID based subgrouping for LP-WUS is reused, and the maximum number of subgroups is finalized to be 32, i.e., 5 bits are required in UE ID for LP-WUS subgrouping, then at least 17 bits would be required for UE ID if considering eDRX is supported when using LP-WUS.

In Option 1 and 2, it can be ensured that the bits in UE ID used for LP-WUS will not overlap with those used for PEI. In such cases, at least 18 bits (if not configured with eDRX) and 20 bits (if configured with eDRX) are used in UE ID for PO/PF determination, PEI subgrouping and LP-WUS subgrouping, the current Extended UE Identity Index Value IE with 16 bits is definitely insufficient.

Therefore, RAN3 should wait for further progress from RAN1/2 regarding whether to support LP-WUS with eDRX and how to design UE ID-based subgrouping for LP-WUS, e.g., to introduce a new IE with larger size.

Observation 1: The current size of Extended UE Identity Index Value IE may not be sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 is pending on the extending the size of Extended UE Identity Index Value IE, based on RAN1/2 progress.

## 2.2 Last used cell only

The previous RAN3-125bis meeting made the working assumption on the last used cell only, which impacts whether to introduce the LP-WUS subgrouping capability related information into the XnAP RAN paging message.

For Rel-17 PEI, each cell may broadcast the lastUsedCellOnly IE in SIB1, to indicate that the PEI can only be used in the last used cell. Typically, for an inactive UE moved from the anchor cell to the new cell, if the new cell broadcasts lastUsedCellOnly IE, the UE will not monitor the PEI, and the new gNB will not send PEI for this UE when it receives Xn RAN paging message from the anchor cell. On the contrary, if the new cell does not broadcast the lastUsedCellOnly IE, the UE should monitor the PEI instead.

Due to the fact that the anchor cell does not know whether the neighbor cells broadcast this lastUsedCellOnly IE or not, it should always include the PEIPS Assistance Information in the RAN paging message. Then it is up to the receiving cell to decide to use PEI or not. Hence for LP-WUS, a very same procedure can be adopted as PEI, the LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, should be introduced over Xn RAN paging message, no matter the last used cell feature is introduced for LP-WUS or not. Of course, this can be further revisited after RAN2 final decision.

RAN3 to keep the working assumption on the LP-WUS assistance information IE in Xn RAN paging message, and agree the XnAP BLCR with editor’s note pending to RAN2 final decision.

## 2.3 LP-WUS and emergency PDU session

In previous meeting, RAN2 discussed that it should be further considered whether/how LP-WUS subgrouping indication can be used when UE has an emergency PDU session.

Agreement at RAN2#128 FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In fact, handling of subgrouping indication when UE has an emergency PDU session was discussed during R17 PEI. According to CT1 specification in TS 24.501, the UE does not need to report the relevant capability to the core network, hence the CN will not send the CN based subgrouping in the NG Paging message for UE with emergency PDU session. But for the UE ID based subgrouping, the PEI is still enabled (see the RAN3 reply LS in R3-233313) since the additional latency introduced by PEI is considered tolerable.

The LP-WUS design would take a very similar approach as PEI. However, for LP-WUS, the latency cannot be negligible due to the ramp up time triggered by LP-WUS. Hence both the CN based subgrouping and the UE_ID based subgrouping of LP-WUS should not be allowed for UE with emergency PDU session to further reduce the potential latency.

For LP-WUS, both CN assigned and UE_ID based subgrouping are not allowed when UE has an emergency PDU session.

## 2.4 Discussion on SA2 LS

SA2 sent an LS to RAN3 on LP-WUS subgrouping, the following two questions are included, which needs RAN3 feedback [5].

Additionally, SA2 discussed the following aspects: 1) SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile. SA2 invites feedback on appropriate terminology and naming of the “LP-WUS” feature. SA2 will then align the terminology and name, based on the feedback. 2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this. 2. Actions: To RAN2, RAN3, CT1 group. ACTION: SA2 asks RAN2, RAN3 and CT1 group to take the above into account and provide feedback if any. ACTION: SA2 asks RAN3 and RAN2 to take the above into account and provide feedback on the terminology and name SA2 should use, and provide feedback on updating UE Radio Capability for Paging Information for the scenario mentioned above.

### 2.4.1 “LP-WUS” terminology

Low Power-Wake Up Signal (LP-WUS) is a wake-up signal that can be received by a receiver with low power consumption, since a specific Low Power-Wake Up Receiver (LP-WUR) is introduced for LP-WUS reception. Per our understanding, LP-WUS is not mandatorily forced to be transmitted with high power. Higher power (e.g., power boosting) is just one possible way to ensure the coverage performance, there are also other ways to improve coverage performance (see the following RAN1 agreement). Therefore, there is no ambiguity caused by “LP-WUS”, it is suggested to use “LP-WUS” as the terminology.

RAN3 to confirm that the “LP-WUS” terminology can be used, since LP-WUS is received with low power consumption by LP-WUR.

### 2.4.2 UE radio capability for paging information

In the RAN1’s LS [5], it is described that the UE may not support LP-WUS reception on all the bands supported by the UE, which means that the LP-WUS capability would be a band-specific capability for the UE. Therefore, similar as pei-SubgroupingSupportBandList for Rel-17 PEI, a new Rel-19 capability of LP-WUS for paging information needs to be introduced, e.g., LP-WUS related capability is added in UE-RadioPagingInfo-r17, or added in UERadioPagingInformation but outside of UE-RadioPagingInfo-r17.

According to the current mechanism, the UE shall report its LP-WUS capability via the UE capability reporting procedure. After receiving the UECapabilityInformation message, the gNB decodes it and accordingly generates the UERadioPagingInformation inter-node RRC message which contains UE’s paging related capabilities including the LP-WUS capability. Then the gNB transfer it to the CN or other gNBs.

Observation 2: LP-WUS related capability needs to be introduced in UE radio capabilities over the Uu interface and UE radio paging capabilities over the network interfaces.

The LS from SA2 discussed the scenario involving the pre-Rel 19 gNB and Rel-19 gNB supporting LP-WUS. As illustrated in Fig.1, if the UE reports the LP-WUS capability to a legacy gNB (i.e. the gNB cannot recognize LP-WUS capability), the legacy gNB cannot include LP-WUS capability in UERadioPagingInformation inter-node message, so the LP-WUS capability is actually not maintained by the CN. However, when the UE moves to the cells where LP-WUS is supported, it does not know this situation and will use LP-WUS for paging monitoring later in these cells. However, the gNB does not get any LP-WUS capability from CN for the paged UE, which results in the UE monitoring LP-WUS while the gNB directly sends paging message to the UE in its POs. In such case, the UE will miss the paging message as it does not wake up in its PO, which means the UE is already unreachable.

Fig.1 An example for the LP-WUS capability maintenance and mismatch issue

Observation 3: If the UE reports LP-WUS capability to an old gNB that does not forward LP-WUS capability to CN, the UE will be unreachable under new gNBs, since the UE will use LP-WUS for paging, while the new gNBs does not use LP-WUS due to the missing LP-WUS capability in the UE Radio Capability for Paging.

RAN3 to confirm the non-homogeneous scenario mentioned in the LS, i.e., the UE will be unreachable due to the LP-WUS mismatch between UE and gNB, if UE does not know the gNB fails to be forward the LP-WUS capability to CN.

To further consider the solution to address the capability mismatch issue between the UE and the network, we provide the following possible solutions.

- Option 1: OAM based - Homogeneous support of LP-WUS

The simplest approach is to ensure the homogenous support of LP-WUS in a given area, e.g., across the entire TA area. Once all gNBs in a given area are upgraded simultaneously, the capability mismatch issue would no longer exist, and this solution does not have any impact on the specifications. However, multi-vendors scenario was emphasized in SA2 discussion, achieving simultaneous upgrades for multi-vendors may be challenging.

Observation 4: Leaving to network deployment does not have any impact on the specifications, but may be challenging for multi-vendors scenario.

- Option 2: UE based solution- UE gets information from gNB whether LP-WUS capability can be forwarded to the CN

As another way, if the UE can know unambiguously whether the reported LP-WUS capability will be forwarded by the gNB and maintained by the CN, the mismatch of LP-WUS capability can be solved. For example, only if the gNB indicates the UE to report LP-WUS capability, or indicates the UE that LP-WUS capability forwarding is supported, the UE can report the LP-WUS capability and can use LP-WUS in a cell where LP-WUS is supported by the gNB. Otherwise, the LP-WUS capable UE cannot use LP-WUS. This option is a universal solution to support the non-homogeneous deployment. Given this solution would have UE impact, the details should be left to RAN2 decision.

RAN3 to discuss the following two options:

- Option 1: OAM based - Homogeneous support of LP-WUS

- Option 2: UE based - UE gets information from the gNB whether LP-WUS capability can be forwarded to the CN

- It is suggested for RAN3 to provide the following feedback to SA2.

RAN3 to reply to SA2:

- the OAM based solution is that last resort to resolve the issue if the UE based solution is not introduced.

- RAN3 would like to leave the UE based solution to RAN2.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: The current size of Extended UE Identity Index Value IE may not be sufficient if both PEI and LP-WUS are supported by UE and gNB.

Observation 2: LP-WUS related capability needs to be introduced in UE radio capabilities over the Uu interface and UE radio paging capabilities over the network interfaces.

Observation 3: If the UE reports LP-WUS capability to an old gNB that does not forward LP-WUS capability to CN, the UE will be unreachable under new gNBs, since the UE will use LP-WUS for paging, while the new gNBs does not use LP-WUS due to the missing LP-WUS capability in the UE Radio Capability for Paging.

Observation 4: Leaving to network deployment does not have any impact on the specifications, but may be challenging for multi-vendors scenario.

- RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as 32 if this WA is agreed by RAN1.

- RAN3 is pending on the extending the size of Extended UE Identity Index Value IE, based on RAN1/2 progress.

- RAN3 to keep the working assumption on the LP-WUS assistance information IE in Xn RAN paging message, and agree the XnAP BLCR with editor’s note pending to RAN2 final decision.

- For LP-WUS, both CN assigned and UE_ID based subgrouping are not allowed when UE has an emergency PDU session.

- RAN3 to confirm that the “LP-WUS” terminology can be used, since LP-WUS is received with low power consumption by LP-WUR.

- RAN3 to confirm the non-homogeneous scenario mentioned in the LS, i.e., the UE will be unreachable due to the LP-WUS mismatch between UE and gNB, if UE does not know the gNB fails to be forward the LP-WUS capability to CN.

- RAN3 to discuss the following two options:

- Option 1: OAM based - Homogeneous support of LP-WUS

- Option 2: UE based - UE gets information from the gNB whether LP-WUS capability can be forwarded to the CN

- RAN3 to reply to SA2:

- the OAM based solution is that last resort to resolve the issue if the UE based solution is not introduced.

- RAN3 would like to leave the UE based solution to RAN2.

The TPs to NGAP and XnAP are provided in the Annex to capture the above agreements. And the F1AP CR is provided in [7].

# 4. Reference

- RAN3#126 chairman notes.

- RAN1#118bis chairman notes.

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- S2-2412876 LS on LP-WUS subgrouping ;

- R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE model;

- S2-2411297/R2-2409226, LS Reply on LP-WUS subgrouping.

- RP‑241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

# 5. TP to TS 38.413

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

LP-WUS	Low Power Wake UP Signal

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.3	UE Context Management Procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, replace the previously provided Core Network Assistance Information for RRC INACTIVE and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.5.1	Paging

#### 8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

If the LP-WUS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for LP-WUS paging subgrouping of the UE, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.4	Paging Messages

#### 9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.3.1.138 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.3.1.139 | If this IE is present, the Paging DRX IE is ignored. | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| WUS Assistance Information | O |  | 9.3.1.143 |  | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.3.62 |  | YES | ignore |
| CN MT Communication Handling | O |  | ENUMERATED (supported, …) | This IE indicates the CN support of CN based MT communication handling. | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	LP-WUS Assistance Information

This IE provides the LP-WUS information related to CN assigned subgrouping for a particular UE, as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..31) [FFS] |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<ASN.1 to be added when the above is stable>

# 6. TP to TS 38.423

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LBT	Listen-Before-Talk

LP-WUS	Low Power Wake UP Signal

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE Specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.2.3.144a |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.172 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.2.3.195 |  | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.2.3.aaa |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.aaa	LP-WUS Assistance Information

This IE provides the LP-WUS information related to CN assigned subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..31) [FFS] |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<ASN.1 is to be added when the above is stable>