3GPP TSG-RAN WG3 #127	R3-250160

Greece, Athens, 17th – 21st February 2025

Agenda item:		13.2 Support for inter-CU LTM

Title:		 (TP to BL CR for TS 38.423 on Inter-CU LTM) Rel-19 inter-CU LTM issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, we continue to discuss and proposal to progress the project.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-242356 (target: RAN #109) [TU: 1 ( 1, 1, 1, 0.5)] QUOTA: 3

13.2. Support for inter-CU LTM Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: As secondary priority, support the case where CU is acting as SN and MN is unchanged As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support RAN3#126: RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details…

2. Discussion

2.1	LTM Preparation

2.1.1	Solution to support semi-persistent CSI-RS

Last RAN3#126 has replied to RAN1 [R3-247911] that in addition to periodic CSI-RS, support of semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN3 confirmed that it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details for intra-CU inter-DU LTM as well as for inter-CU LTM.

Let see first for the intra-CU inter-DU LTM, to have coordination between serving cell and candidate cells, there at least can have some options to discuss: (refer to R3-247824: Summary of offline discussion on MobilityEnh)

Figure 1. Procedures to support SP CSI-RS measurement in intra-CU inter-DU case - Option 1: The source gNB-DU determines the CSI-RS resources that to be transmitted, and sends the CSI-RS resource (set) IDs to the candidate gNB-DU via the gNB-CU to request the candidate gNB-DU to transmit the CSI-RS. In this option, the source gNB-DU needs to know the SSB and CSI-RS association of the candidate cell, to determine the suitable CSI-RS resources. - Option 2: The source gNB-DU forwards the SSB measurement report to the candidate gNB-DU and the candidate gNB-DU determines the CSI-RS resources. The candidate gNB-DU sends that information back to the source gNB-DU. - Option 3: The gNB-CU decides to trigger semi-persistent CSI-RS measurement and sends the selected CSI-RS resource (set) IDs to the candidate gNB-DU and the source gNB-DU based on L3 measurement results. The candidate gNB-DU starts to transmit CSI-RS following the indication of the gNB-CU, and the source gNB-DU triggers the UE measurement.

In addition to the periodic CSI-RS transmission, e.g. based on the L1 measurements, the node can decide whether additional CSI-RSs need to configure and activate for some candidate cells to perform beam measurements with more refinement. Since for the trigger of LTM is basically by the received L1 report in the source DU, it would be more appropriate for the source DU to have at least the trigger of additional semi-persistent CSI-RS transmission.

For the intra-CU case, the procedure can be like (roughly):

1. Source DU receives of L1 report (e.g. SSB based or periodic CSI-RS based)

2. Source DU sends a SP CSI-RS transmission request to the CU. The SP-CSI-RS transmission configuration (CSI-RS resource, periodicity etc.) is configurated by the gNB-CU and indicates to the candidate DUs. The candidate DUs responds back to the source CU meaning the SP CSI-RS transmission is activated.

3. Source CU sends a SP CSI-RS Transmission Response to the source DU.

4. The source CU included the SP CSI-RS configuration information in the RRCReconfiguration message then send to the UE.

Similarly for the inter-CU case, the procedure can be like (roughly)

1. Source DU receives of L1 report (e.g. SSB based or periodic CSI-RS based)

2. Source DU sends a SP CSI-RS transmission request to the source CU.

3. source CU sends the LTM Configuration Update request message to the candidate CUs indicate the request of SP CSI-RS transmission. The SP CSI-RS transmission configuration (CSI-RS resource, periodicity etc.) is configurated by the gNB-CU, and indicate to the candidate DUs. The candidate DUs response back to the gNB-CU meaning the SP CSI-RS transmission is activated.

4. the candidate CU respond back the LTM Configuration Update Acknowledge message with CSI-RS configuration information.

5. Source CU sends a SP CSI-RS Transmission Response to the source DU.

6. The source CU included the SP CSI-RS configuration information in the RRCReconfiguration message then send to the UE.

Proposal 1: For the network Semi-Persistent CSI-RS coordination, it is proposed the source gNB-DU trigger the procedure via the control by the gNB-CU.

2.1.2	Initial LTM configuration request

For inter-CU LTM mobility, we agreed that a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. Still a question is remaining.

Question: For the same UE when multiple preparation procedures is done for different cells in the same candidate CU, wow the candidate CU to identify the same UE in parallel LTM request?

More exactly, this question has been raised that how does the candidate CU know the multiple HANDOVER REQUEST messages (each message per cell) is for the same UE. However this was more or less confirmed that there is no problem for the candidate CU to associate the different HANDOVER REQUEST messages to the same UE because there is a mandatory NG-C UE associated signalling reference IE which is the AMF UE NGAP ID allocated in AMF, will unique for the UE. The candidate gNB when receive two Handover Request message with different AP ID respectively, it will be able to identify only from the NG-C UE associated signalling reference IE whether these multiple Handover Request message is for the same UE. If multiple Handover Request messages each has different GUAMI, the candidate gNB can only identify these Handover Request messages are for different UEs.

Proposal 2: To confirm that the existing mandatory NG-C UE associated signalling reference IE in the XnAP HANDOVER REQUEST can be used to identify the same UE from multiple Handover preparation procedure.

2.1.3	LTM configuration update

Last RAN3#125bis meeting we agreed a new procedure LTM Configuration Update is introduced with LTM CONFIGURATION UPDATE, LTM CONFIGURATION UPDATE ACKNOWLEDGE and LTM CONFIGURATION UPDATE FAILURE messages, these have been reflected in BL CR to 38.300 and BL CR to 38.423.

The BL CR to 38.423 for Inter-CU LTM in the LTM CONFIGURATION UPDATE message now contains:

- Target Cell Global ID (FFS)

- Early Sync Information  (detail is FFS)

- Early Sync Information for SUL (detail is FFS).

- LTM Information Request

- CSI-RS Response Configuration

The Target Cell Global ID IE can be a single cell provided that there are no multiple cells in the handover preparation phase.

Early Sync Information can simply reuse what has been in Rel-18 intra-CU LTM, provided by the candidate DU as in 9.3.1.328 of F1AP:

38.473v18.4.0 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8..

Proposal 3: for the Early Sync Information from the candidate CU, can reuse what has been in Rel-18 intra-CU LTM provided by candidate DU.

2.2	LTM Execution

2.2.1	TA Information Transfer message

For the PDCCH ordered early RACH procedure for inter-CU LTM, in the RAN3#124 meeting, it was already agreed to introduce a new procedure on Xn interface to transfer the TA information. But whether it is a UE associated message or non-UE associated message is still FFS, and in RAN3#125/#126 meeting there is still no consensus. A simple way is to align with Rel-18 LTM in F1, no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 4: In order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

We still assume the allocated Early UL Sync Configuration is valid for a list of DU in the source gNB-CU, and the Preamble Index List is similar to intra-CU LTM case that it is valid for a DU. The information in the Handover related messages and LTM configuration Update related message each is per UE associated, the candidate CU can easily identify the source DU when receive the information from the candidate DUs in F1AP.

Only when transfer the TA Information from the candidate DU to source CU, we should consider such case provided the TA Information will be using non-UE associated signaling message, then an ID to identify the source CU will be needed to be informed to gNB-DU.

Proposal 5: In order to correctly forward the TA information from candidate DU to source CU, consider introducing an ID to identify the source CU e.g. source CU-ID to identify the source CU.

The information to be in the XNAP TA Information Transfer message, can be same as what in the existing F1AP e.g. DU-CU TA Information Transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- Tag ID Pointier

- SSB Positions in Burst

- Early UL Sync Configuration Provider ID

- source gNB-DU ID

Proposal 6: The information to be in the XNAP TA Information Transfer message, can be same as what in the existing F1AP e.g. DU-CU TA Information Transfer message.

2.3	Support of subsequent LTM

Sync up of LTM information between candidate gNBs:

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, and data forwarding addresses, etc.) may need to sync up among candidate gNBs.

In Rel-18 intra-CU inter-DU LTM in F1, the following note is captured in stage 2 (38.401):

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

For the Rel-19 inter-CU LTM, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via LTM Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

Considering to align with Rel-18 Intra-CU case that “Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.”, it would be then feasible to do the same, i.e. all three Option 1, 2 and 3 may be done by implementation. The message to be used can be LTM Configuration Update message.

Proposal 7a: To support subsequent inter-CU LTM, to sync up among candidate gNBs the LTM information, to align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3, which then can be done by implementation. The message to be used can be LTM Configuration Update message.

Proposal 7b: The LTM information to be sync up are e.g. early sync configuration, LTM configuration ID, and data forwarding addresses of each candidate gNBs.

Data forwarding address transfer among candidate gNBs for subsequent LTM

For the purpose of subsequent LTM, the prepared data forwarding addresses may need to transfer among candidate gNBs. The timing of the transferring, some options to be further discussed (from R3-247824: moderator summary in RAN3#126):

- Option 1: The data forwarding address is sent to the candidate node during LTM preparation phase.

- Option 2: The data forwarding address is sent to the candidate node in the XnAP CELL SWITCH NOTIFICATION message

- Option 3: The data forwarding address is sent together with the collected LTM information during the LTM preparation phase or late steps (up to implementation)

- Option 4: The new serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Similar to the case of sync up configurations among candidate gNBs for subsequent LTM, the data forwarding address of each candidate gNB can be included in the LTM Configuration Update message, the timing of the transferring can be during the LTM preparation phase or during the cell switch execution, or after the successful cell switch, up to implementation.

Proposal 8: For the purpose of subsequent LTM, the prepared data forwarding addresses of each candidate gNB can be included in the LTM Configuration Update message, the timing of the transferring can be during the LTM preparation phase or during the cell switch execution, or after the successful cell switch, up to implementation.

3. Conclusion and proposals

Proposal 1: For the network Semi-Persistent CSI-RS coordination, it is proposed the source gNB-DU trigger the procedure via the control by the gNB-CU.

Proposal 2: To confirm that the existing mandatory NG-C UE associated signalling reference IE in the XnAP HANDOVER REQUEST can be used to identify the same UE from multiple Handover preparation procedure.

Proposal 3: for the Early Sync Information from the candidate CU, can reuse what has been in Rel-18 intra-CU LTM provided by candidate DU.

Proposal 4: In order to align with Rel-18 LTM in F1, and with no change to Rel-18 LTM, to introduce TA information transfer in XN with non-UE associated message.

Proposal 5: In order to correctly forward the TA information from candidate DU to source CU, consider introducing an ID to identify the source CU e.g. source CU-ID to identify the source CU.

Proposal 6: The information to be in the XNAP TA Information Transfer message, can be same as what in the existing F1AP e.g. DU-CU TA Information Transfer message.

Proposal 7a: To support subsequent inter-CU LTM, to sync up among candidate gNBs the LTM information, to align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3, which then can be done by implementation. The message to be used can be LTM Configuration Update message.

Proposal 7b: The LTM information to be sync up are e.g. early sync configuration, LTM configuration ID, and data forwarding addresses of each candidate gNBs.

Proposal 8: For the purpose of subsequent LTM, the prepared data forwarding addresses of each candidate gNB can be included in the LTM Configuration Update message, the timing of the transferring can be during the LTM preparation phase or during the cell switch execution, or after the successful cell switch, up to implementation.

# References

[1] RP-242356 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-247824, Summary of offline discussion on MobilityEnh

[3] R3-250008== R2-2411134, RAN2 agreements on Inter-CU LTM and Conditional LTM

[4] R3-250050 == R3-247932 (BL CR to 38.300) Inter-CU LTM

[5] R3-250051 == R3-247933 (BL CR to 38.423) Xn Support for Inter-CU LTM

[6] R3-250048 == R3-247590 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[7] R3-250049 == R3-247514 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

(TP to TS 38.423 on Inter-CU LTM) Inter-CU LTM on top of R3-247933

///////////////////////////////////////////////////////////////////////skip nrelated///////////////////////////////////////////////////////////////////////

9.1.1	Messages for Basic Mobility Procedures

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. NOTE: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| >NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| >LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.2.3.108 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditional Handover Time Based Information | O |  |  | This IE only applies to NTN. | YES | reject |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigNR IE as defined in TS 38.331 [10] | – |  |
| >Maximum Number of Conditional Reconfigurations to Prepare | O |  | INTEGER (1..8 , …) | Indicates that the target NG-RAN node may prepare for CHO with candidate PSCell(s) configuration and indicates the maximum number of conditional reconfigurations that the target NG-RAN node may prepare. | YES | reject |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED(authorized, not authorized, ...) | This IE indicates the authorization status of the IAB node. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) | This IE indicates that information on DL LBT Failures occurring at the target gNB during handover execution that results in mobility failure is requested | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.2.3.175 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.2.3.176 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.2.3.177 |  | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.2.3.178 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.180 | This IE contains information about cells that a UE is predicted to be connected to. | YES | ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | ignore |
| Candidate Relay UE Info List | O |  | 9.2.3.188 |  | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC configuration information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Mobile IAB Authorization Status | O |  | 9.2.2.105 |  | YES | reject |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | reject |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x2		TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Editor’s note: Details on IEs need to be continued.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| CSI-RS Resource Configuration | O |  | OCTET STRING |  | YES | ignore |

9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| CSI-RS Report Configuration | O |  | OCTET STRING (FFS) |  | YES | ignore |

9.1.1.x5 LTM CONFIGURATION UPDATE FAILURE

This message is sent by the NG-RAN node2 to indicate to NG-RAN node1 about LTM Configuration Update failure.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.1.x6	 LTM CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared LTM handover.

Direction: target NG-RAN node  source NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | O |  | 9.2.3.2 |  | YES | ignore |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.xx1	LTM Information Request

This IE contains a request for LTM information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Configuration | O |  | OCTET STRING |  |
| CSI Resource Configuration | O |  | OCTET STRING |  |
| LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  |
| Request For CSI-RS Configuration | O |  | ENUMERATED (true, …)(FFS) |  |

9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING |  |
| Reference Configuration | O |  | OCTET STRING |  |
| CSI-RS Configuration | O |  | OCTET STRING |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.xx3	Early Sync Information Request

This IE contains a request for Early synchronization information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Request for RACH Configuration | M |  | ENUMERATED (true, …) |  |
| ID to identify the source CU | FFS |  |  |  |
| LTM gNB-DUs List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |
| >LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |

9.2.1.xx4	Early Sync Information Response

This IE contains Early synchronization related information to be provided in response to information provided in the Early Sync Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| Early UL Sync Configuration | O |  | 9.2.1.xxY |  |
| Early UL Sync Configuration for SUL | O |  | 9.2.1.xxY | This IE applies for SUL carrier. |

9.2.1.xx5	LTM Configuration ID Mapping List

This IE indicates the list of LTM candidate cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [10]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

9.2.1.xxY	Early UL Sync Configuration

This IE indicates the early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8]. |
| LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |
| >LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.2.1.yy1 |  |
| >>Preamble Index List | O |  | 9.2.1.yy2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

9.2.1.yy1	gNB-DU ID

The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU ID | M |  | INTEGER (0 .. 236-1) | The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers. |

9.2.1.yy2	Preamble Index List

This IE indicates the list of preamble indexes to be used for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Preamble Index Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >Preamble Index | M |  | INTEGER (0..63) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

///////////////////////////////////////////////////////////////////////skip nrelated///////////////////////////////////////////////////////////////////////

Skip unchanged part