3GPP TSG-RAN WG3 #127	R3-250202

Athens, Greece, 17th- 21st February 2025

Agenda Item:	17.3

Source:	Fujitsu

Title: 	On-demand SIB1 for UEs in idle/inactive mode

Document for:	Discussion and Approval

1	Background

In RAN#102 meeting, a work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2] in RAN#105 meeting. The objectives of the work item on on-demand SIB1 for UEs in idle/inactive are the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

At RAN3#125 meeting, there are following agreements:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

At RAN3#125bis meeting, there are following agreements:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

Besides, RAN3 has an offline discussion for the following open issues but no conclusions are made.

Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast. Further check on first activation of UL WUS configuration broadcast in Cell A.

At RAN3#126 meeting, there are following agreements:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In this contribution, we would like to discuss the interaction of NES Cell and Cell A in case of UL WUS configuration update.

2	Discussion

In RAN2#127 meeting, it was agreed that once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. That is to say, the NES cell will apply the updated UL WUS configuration from the next SI modification period of the NES cell.

Observation 1: In cases of WUS configuration update, the updated UL WUS configuration will take effect from the next SI modification period of the NES cell.

As mentioned above, RAN3 has the following agreements on the details of Case 2:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Based on the RAN3 agreements, when UL WUS configuration is updated in the NES cell, the updated UL WUS configuration would be sent from the RAN node of the NES cell to the RAN node of Cell A, and then Cell A would update its SIB including the updated UL WUS configuration. By the legacy SI modification procedure, the updated SIB will be transmitted from the next SI modification period of Cell A.

Observation 2: In cases of WUS configuration update, the updated UL WUS configuration will take effect from the next SI modification period of Cell A.

An issue that can be discussed is the UL WUS configuration mismatch between NES cell and Cell A in case of UL WUS configuration update. As shown in Figure 1, if the next SI modification period at the NES cell is not aligned with the next SI modification period at Cell A, the UEs in Cell A may transmit UL WUS using the UL WUS configuration not aligned with the UL WUS configuration applied in the NES cell, and thus the UL WUS would not be received by the NES cell and the on-demand SIB1 request procedure would fail. Since the SI modification period can be as long as 4096 frames (40.96s), the UL WUS transmission failure of UEs in Cell A may happen frequently.

RAN node of Cell A

RAN node of NES cell

UL-WUS config update

Next SI modification period @NES cell

Next SI modification period @Cell A

OD-SIB1 request from UEs in Cell A may fail due to mismatch of UL WUS configuration

RAN node of Cell A

RAN node of NES cell

UL-WUS config update

Next SI modification period @NES cell

Next SI modification period @Cell A

OD-SIB1 request from UEs in Cell A may fail due to mismatch of UL WUS configuration

Figure 1 UL WUS configuration update

Observation 3: If the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, UL WUS transmission from UEs in Cell A may fail.

To reduce the UL WUS transmission failure from UEs in Cell A, there can be following options:

- Option 1: Align the SI modification periods of Cell A and the NES cell.

- Option 2: Cell A sends an indication to NES cell to indicate it is time to broadcast UL WUS configuration in Cell A.

- Option 3: NES cell sends an indication to Cell A to indicate it is time to broadcast UL WUS configuration in the NES cell.

- Option 4: NES cell notifies Cell A about the effective time of the (updated) UL WUS configuration in the NES cell during the new class1 defined procedure.

Among the above options:

Option 1 is straightforward, and it has least signalling overhead and least specification impact compared with other options but may require some configuration limitation on the SI modification periods between the two RAN nodes. In our understanding, the configuration limitation can be done by OAM which is simple.

Option 2 and Option 3 have been discussed for UL WUS configuration broadcast in the offline discussions last meeting but currently companies have no consensus. In our assumption, Option 2 or Option 3 can also be used to align the UL WUS configuration update timing if the ETWS/CMAS like notification scheme is used in Cell A (Option2) or the NES cell (Option 3). Moreover, on receiving the paging short message, the UEs in Cell A (Option2) or the NES cell (Option 3) should acquire the SIB including the updated UL WUS configuration immediately. Therefore, if RAN3 would agree Option 2 or Option 3 for UL WUS configuration, the agreed option could be used to resolve the above issue.

Option 4 can reduce the signalling exchange without transmitting the additional indication in Option 2 and Option 3. How to indicate the effective time can be further discussed if this option is agreed.

From our perspective, as each option can work, we have no strong preference. Therefore, RAN3 is proposed to discuss these options for the case of UL WUS configuration update.

Proposal 1: RAN3 to discuss the following options for aligning the UL WUS configuration update timing in case of UL WUS configuration update:

- Option 1: Align the SI modification periods of Cell A and the NES cell.

- Option 2: Cell A sends an indication to NES cell to indicate it is time to broadcast UL WUS configuration in Cell A.

- Option 3: NES cell sends an indication to Cell A to indicate it is time to broadcast UL WUS configuration in the NES cell.

- Option 4: NES cell notifies Cell A about the effective time of the (updated) UL WUS configuration in the NES cell during the new class1 defined procedure.

3	Proposals

In this contribution, we discussed the issue when UL WUS configuration is updated. We have the following observations and proposals:

Observation 1: In cases of WUS configuration update, the updated UL WUS configuration will take effect from the next SI modification period of the NES cell.

Observation 2: In cases of WUS configuration update, the updated UL WUS configuration will take effect from the next SI modification period of Cell A.

Observation 3: If the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, UL WUS transmission from UEs in Cell A may fail.

Proposal 1: RAN3 to discuss the following options for aligning the UL WUS configuration update timing in case of UL WUS configuration update:

- Option 1: Align the SI modification periods of Cell A and the NES cell.

- Option 2: Cell A sends an indication to NES cell to indicate it is time to broadcast UL WUS configuration in Cell A.

- Option 3: NES cell sends an indication to Cell A to indicate it is time to broadcast UL WUS configuration in the NES cell.

- Option 4: NES cell notifies Cell A about the effective time of the (updated) UL WUS configuration in the NES cell during the new class1 defined procedure.

5	References

[1]	RP-234065	New WID: Enhancements of network energy savings for NR

[2]	RP-242354	Revised WID: Enhancements of network energy savings for NR