3GPP TSG-RAN WG3 Meeting #127	R3-250255

Athens, GR, 17-21 Feb, 2025

Title:	Authorization for multi-hop relay

Agenda Item:	22.2

Source:	Huawei

Document for:	other

# 1	Introduction

RAN2 has had two meetings (RAN2#127 and RAN2#128) to discuss the multi-hop relay, which covers the main part of the work item objectives [1]. For the first RAN3 meeting for multi-hop relay, we discuss the authorization for multi-hop relay in this paper.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

# 2	Discussion

SA2 has sent an LS [S2-2501296] to RAN3 with the following content:

SA2 would like to inform RAN2 and RAN3 that SA2 agreed to provide the following service authorization information from AMF to NG-RAN for Layer-2 multi-hop UE-to-Network relay operations as attached CR:

- whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

The LS also includes the associated CR [S2-2500386] with the following agreed changes:

- -	5G ProSe Layer-2 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay, as 5G ProSe Layer-2 Intermediate UE-to-Network Relay, as 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay).

In addition to this, the CR also contains contains previously agreed authorization related to Layer-3 multi-hop UE-to-Network Relay:

- 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay)

Based on the above, RAN3 should provide this possibility over our interfaces, similarlry as was done in the past.

Agree to include authorsiztion information for 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 Intermediate UE-to-Network Relay, and 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay

Agree to include authorsiztion information for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay

Since the latter proposal is not within the scope of the LS from SA2, we also suggest to send a reply LS to inform them that we also captured support for this authorization.

Send a response LS to inform SA2 that RAN3 also agreed to include authorization for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay

# 3	Conclusion

This document proposes:

Proposal 1:	Agree to include authorsiztion information for 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 Intermediate UE-to-Network Relay, and 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay

Proposal 2:	Agree to include authorsiztion information for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay

Proposal 3:	Send a response LS to inform SA2 that RAN3 also agreed to include authorization for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay

# Annex3	Response LS to SA2

3GPP TSG-RAN WG3 Meeting #127	TDoc <TDoc#>

Athens, GR, 17-21 Feb, 2025

Title:	[DRAFT] Reply LS on Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN

Response to:	LS S2-2501296

Release:	Rel-19

Work Item:	5G_ProSe_Ph3

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2

Contact person:	Henrik Olofsson

Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 would like to thanks SA2 for the LS on authorization information from AMF to NG-RAN for Layer-2 multi-hop UE-to-Network relay.

RAN3 agreed to capture the autorization for layer-2 multi-hop U2N relaying.

In addition to this, RAN3 also agreed to include authorization for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above information into account in their future work.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127	2025-02-17 - 2025-02-21		Athens, GR

RAN3#127-bis	2025-04-07 - 2025-04-11		China, CN

RAN3#128	2025-05-19-  2025-05-23		Malta, MT

RAN3#129	2025-08-25 - 2025-08-29		India (TBC), IN

RAN3#129-bis	2025-10-13 -  2025-10-17		Prague, CZ

RAN3#130	2025-11-17 -  2025-11-21		Dallas, US