3GPP TSG-RAN WG3 Meeting #127	R3-250268

Athens, Greece, 17-21 Feb, 2025

Agenda item:	8.1

Source: 	Lenovo

Title: 	Discussion on LS on XR UL bit rate control

Document for:	Discussion and Approval

- Introduction

This paper discusses the response to the LS on XR UL bit rate control [1]:

RAN2 has made the following agreement regarding a solution on UL bit rate control Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

- Discussion

From our perspective, the main issue is whether the per QoS flow level rate control can work or not in case of CU-DU split architecture. It mainly depends on the following aspects:

-	GRB or non-GBR QoS flow

-	whether only one QoS flow within a DRB if which bit rate can be adapted

Currently, the mapping of QoS flows to DRBs is performed in the SDAP layer, which resides in the gNB-CU. The gNB-DU, on the other hand, carries out scheduling on a per-DRB basis.

In the F1-AP, the gNB-CU sends the following information to the gNB-DU:

-	DRB-level QoS parameters

-	QoS flow list associated with the DRB

-	Corresponding QoS flow-level QoS parameters of each QoS flow.

This information enables the gNB-DU to understand the mapping of QoS flows to the DRB and to know both DRB-level and QoS flow-level parameters.

Observation 1: gNB-DU knows the mapping of QoS flows to the DRB, DRB-level QoS parameters and QoS flow-level QoS parameters of each QoS flow.

In the F1-U, the gNB-DU cannot directly know the QFI of a PDCP PDU. This limitation arises due to the following reasons:

For UL Data Transmission:

-	The UE may include the QFI in the SDAP header of a packet, especially when there is a many-to-one mapping of QoS flows to a single DRB. However, the gNB-DU cannot understand the SDAP header because it is secured by the PDCP layer, which encrypts and integrity-protects the data.

For DL (Downlink) Data Transmission:

-	Before Release-18, the gNB-CU did not provide the QFI of a PDCP PDU to the gNB-DU.

-	Starting from Release-18, the gNB-CU may optionally include the QFI in the GTP-U extension header, specifically within the PDU Set Information container. However, this feature is limited to the F1-U PDU Set Information container and is not universally applicable to all scenarios.

In summary, the gNB-DU's ability to identify QFI of a QoS flow is constrained by the security mechanisms of the PDCP layer and the optional nature of QFI inclusion in the GTP-U extension header.

Observation 2: gNB-DU's ability to identify QFI of a packet is constrained by the security mechanisms of the PDCP layer and the optional nature of QFI inclusion in the GTP-U extension header.

If the QoS flow is a GBR QoS flow, the gNB-DU can determine the bit rate of the QoS flow since the CU already sends the QoS parameters via F1-AP signaling as stated in observation 1.

However, for a non-GBR QoS flow, it is challenging for the gNB-DU to determine the bit rate of the QoS flow. This is because the bit rate of a non-GBR QoS flow can vary dynamically, when there is a many-to-one mapping of QoS flows to a single DRB. And the gNB-DU is not able to identify QFI of a QoS flow as stated in observation 2. In contrast, if there is a one-to-one mapping of a QoS flow to a DRB, the gNB-DU can measure the accurate bit rate of that QoS flow.

Observation 3: For a non-GBR QoS flow, it is challenging for the gNB-DU to determine the bit rate of a QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB.

As discussed above, in CU-DU split architecture, per QoS flow rate control can only works for GBR QoS flow and when there is only one QoS flow within a DRB needs to be adapted. In this case, both per QoS flow rate indication by MAC CE and QoS flow indicated by RRC while MAC CE is per DRB can work well. The specific approach can rely on RAN2’s decision.

Proposal 1: In CU-DU split architecture, per-QoS-flow-based rate control can work effectively for a GBR QoS flow when bit rate of only one QoS flow within a DRB needs to be adapted.

However, for non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, the gNB-DU is unable to determine the bit rate of an individual QoS flow. Consequently, it is not feasible for the gNB-DU to determine the recommended bit rate for the QoS flow.

Proposal 2: For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-DU to determine the recommended bit rate for the QoS flow.

Some companies may argue that gNB-CU instead of gNB-DU determines the recommended bit rate. However, gNB-CU is unable to know per QoS flow congestion status since gNB-DU is in charge of scheduling.

In Rel-18, for ECN marking, the UL Congestion Information and DL Congestion Information were introduced in TS 38.425:

5.5.3.61 UL Congestion Information Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB. For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a DRB. As an example, value 9574 corresponds to a percentage of 95.74%. Value range: {0..10000}. Field length: 2 octets.

The UL Congestion Information and DL Congestion Information are currently provided on a per-DRB basis in F1-U. In the previous discussion, it was assumed that there was a one-to-one mapping between QoS flows and DRBs for ECN marking. Specifically, the gNB-DU supplies UL/DL Congestion Information for each DRB to the gNB-CU, which then either performs ECN marking itself or passes the information to the CN for UPF-based ECN marking. However, when multiple QoS flows are mapped to a single DRB (i.e., many-to-one mapping), the existing UL/DL Congestion Information in F1-U cannot be directly reused.

Proposal 3: For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-CU to determine the recommended bit rate for the QoS flow as gNB-CU cannot know congestion status per QoS flow basis.

3. Conclusion

This paper discusses the response to the LS on XR UL bit rate control. The following observations and proposals are made.

Observation 1: gNB-DU knows the mapping of QoS flows to the DRB, DRB-level QoS parameters and QoS flow-level QoS parameters of each QoS flow.

Observation 2: gNB-DU's ability to identify QFI of a packet constrained by the security mechanisms of the PDCP layer and the optional nature of QFI inclusion in the GTP-U extension header.

Observation 3: For a non-GBR QoS flow, it is challenging for the gNB-DU to determine the bit rate of a QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB.

Proposal 1: In CU-DU split architecture, per-QoS-flow-based rate control can work effectively for a GBR QoS flow when bit rate of only one QoS flow within a DRB needs to be adapted.

Proposal 2: For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-DU to determine the recommended bit rate for the QoS flow.

Proposal 3: For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-CU to determine the recommended bit rate for the QoS flow as gNB-CU cannot know congestion status per QoS flow basis.

In the Annex, we provide the draft response LS.

Reference

[1] R3-250012 LS on XR UL bit rate control	RAN2(Meta)

Annex

Draft Response LS to RAN2:

1. Overall Description:

RAN3 thanks RAN2 on the LS on XR UL bit rate control. RAN3 discussed how it works in the CU-DU split architecture and made the following agreements:

- In CU-DU split architecture, per-QoS-flow-based rate control can work effectively for a GBR QoS flow when bit rate of only one QoS flow within a DRB needs to be adapted.

- For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-DU to determine the recommended bit rate for the QoS flow.

- For a non-GBR QoS flow, when there is a many-to-one mapping of QoS flows to a single DRB, it is not feasible for gNB-CU to determine the recommended bit rate for the QoS flow as gNB-CU cannot know congestion status per QoS flow basis.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take above agreements into consideration.