3GPP TSG-RAN WG3 Meeting #127	R3-250281

Athens, Greece, 17th – 21th Febuary, 2025

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Discussion on remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, some remaining issues are discussed based on the latest BL CR.

- Discussion

NR Femto Architecture

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW. For NR Femto, the NG-C interface is defined as the interface: - Between the NR Femto GW and the Core Network; - Between the NR Femto and the NR Femto GW; - Between the NR Femto and the Core Network. The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not. The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC. Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW. One NR Femto serves one or more cells.

Based on the current conclusions for NR Femto architecture, the overall NG RAN architecture with deployed NR Femto GW is shown below.

Figure 1: Overall Architecture with deployed NR Femto GW

Proposal 1: Agree the overall RAN architecture with deployed NR Femto GW.

NR Femto node ID

For LTE, the ECGI only has 28 bits to globally identify a 4G cell, and it also need to use 28 bits to identify a Home eNB, then a Home eNB only support one cell, otherwise, the length of ECGI needs to be extended to support multi-cell of Home eNB. And for NR Femto, it needs to support multiple cells for certain frequency bands. In addition, NCGI has 36 bits, then an NR Femto node can support more than one cell in case we still use 28 bits to identify an NR Femto node.

And in LTE, because the length of Macro eNB ID is 20 bits, and it needs 28 bits to identify a Home eNB, then separate eNB ID IEs were used for Macro eNB and Home eNB. As in NR, the gNB ID has been extended to at most 32 bits, if NR Femto node also uses 28 bits as the Home eNB, the NR Femto at most occupies 1/16 of the space of gNB ID. Then, to minimize the impacts, the gNB ID can be reused directly to identify the NR Femto.

Proposal 2: The gNB ID can be reused directly to identify the NR Femto.

9.2.1.37 Global eNB ID This information element is used to globally identify an eNB (see TS 36.401 [2]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.2.3.8. CHOICE eNB ID; M. >Macro eNB ID. >>Macro eNB ID; M; BIT STRING (SIZE(20)); Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Home eNB ID. >>Home eNB ID; M; BIT STRING (SIZE(28)); Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB.. >Short Macro eNB ID. >> Short Macro eNB ID; M; BIT STRING (SIZE(18)); Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Long Macro eNB ID. >> Long Macro eNB ID; M; BIT STRING (SIZE(21)); Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB.. 9.2.1.38 E-UTRAN CGI This information element is used to globally identify a cell (see TS 36.401 [2]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.2.3.8. Cell Identity; M; BIT STRING (SIZE(28)); The leftmost bits of the Cell Identity correspond to the eNB ID (defined in subclause 9.2.1.37)..

9.3.1.6 Global gNB ID This IE is used to globally identify a gNB (see TS 38.300 [8]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. CHOICE gNB ID; M. >gNB ID. >>gNB ID; M; BIT STRING (SIZE(22..32)); Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB.. 9.3.1.7 NR CGI This IE is used to globally identify an NR cell (see TS 38.300 [8]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. NR Cell Identity; M; BIT STRING (SIZE(36)); The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.3.1.6)..

Function split – NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto, except the UE CONTEXT RELEASE REQUEST message received from the NR Femto with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the NG UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by an NR Femto attached to it. Otherwise it ignores the message.

-	In case of NG INITIAL CONTEXT SETUP REQUEST message and NG HANDOVER REQUEST message, informing the NR Femto about any GUAMI corresponding to the serving AMF, the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE. In case of NG PATH SWITCH REQUEST ACKNOWLEDGE message, informing the NR Femto about the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

-	Upon receiving an OVERLOAD START/STOP message, the NR Femto GW should send the OVERLOAD START/STOP message towards the NR Femto(s) including in the message the identities of the affected AMF(s). The NR Femto uses this information received from the OVERLOAD START message to identify to which traffic the above defined rejections shall be applied. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between the NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally concentration NG-U interface with the NR Femto and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

-	Routing the NG PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto.

Proposal 3: Agree the functions above for NR Femto GW.

# Conclusion

This contribution aims to discuss remaining issues for NR Femto. And following observations and proposals are concluded.

Proposal 1: Agree the overall RAN architecture with deployed NR Femto GW.

Proposal 2: The gNB ID can be reused directly to identify the NR Femto.

Proposal 3: Agree the functions above for NR Femto GW.

Based on the proposals, a TP is drafted to capture the aspects related to NR Femto in another paper [3].

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).

- R3-250282. (TP to BL CR 38.300) Architecture and function split for NR Femto. Lenovo.