3GPP TSG RAN WG3 #127	R3-250290

Athens, Greece, Feb 17th – 21th, 2025

Agenda item:	17.3

Source: 	Lenovo

Title: 	Discussion on On-demand SIB1 for Idle/Inactive UE

Document for:	Discussion and Decision

Introduction

In RAN#105, a revised work item “Enhancements of network energy savings for NR” was approved [1]. The one objective of the work item is the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN3 has discuss the on demand SIB1 for Case 2 and reached the agreement：

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). - The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. - Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. RAN3#126： - The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. - Cell A stores the UL WUS configuration information after it has received it. - A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. - Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In this contribution, we further analyse potential RAN3 impact based on RAN1 and RAN2 progress.

Discussion

2.1 RAN1 and RAN2 progress

In RAN1#116, RAN1 has on-demand SIB1 operation, and it was agreed:

- For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - For study of UL WUS design, consider at least PRACH as a starting point - FFS: Whether there is dedicated PRACH resource for SIB1 request - For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS. - For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

In RAN1#116bis discussion on the on demand SIB1 with the following agreements:

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

In RAN1#117 discussion on the on demand SIB1 with the following agreements:

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact. From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

In RAN1#118b discussion on the on demand SIB1 with the following agreements:

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. (working assumption)ULSubCarrierSpacing. For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

Based on RAN1 agreement, we have following observations.

Observation 1: R19 focus on case 2 on on-demand SIB1 for idle/inactive mode UE:

- Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

In RAN2#125bis initial discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements on on-demand SIB1:

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

In RAN2#126 discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES The same Cell A may assist more than one NES Cells.Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In RAN2#127 discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on OD-SIB1

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

In RAN2#127b discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

- We will inherit all agreements made during SI phase to WI phase. - A NES cell can include neighbouring NES cell’s WUS configuration. - NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration). - In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1. - The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred. - The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell. - A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. - If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. - Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. - NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities). - RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In RAN2#128discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

Based on RAN2 agreement, we have following observations on case 2(Option 1+B+X in RAN1 agreement).

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Observation 6: A NES cell can include neighboring NES cell’s WUS configuration. NES cell’s WUS configuration is included in a new SIB (including its own WUS configuration).

2.2 on-demand SIB1 procedure

Based on RAN3 agreement, DU of NES cells generates UL WUS config and then transmits it to CU. Before DU transmission UL WUS config to CU, one possibility is that DU determines which cells can use on-demand SIB1 by OAM. Before NES cell selects the CELL A for providing the UL WUS config for NES cell, the NES cell may acquire the capability of neighbour cell by OAM or informed by the neighbour cell. After the CU of NES cell transmits UL WUS config of a list of NES cells to the CU of Cell A, if receiving accept response for a list of NES cells from Cell A, then CU of NES cell determines to enter NES mode for the list of NES cells, and transmits on-demand SIB1 activation to DU of the OD-SIB1 for the NES cells, the NES cells starts the OD-SIB1 operation, e.g., indicating UE the OD-SIB1 of the cell is enabled.

Proposal 1:  If CU of NES cells receive accept response for providing the WUS config for a NES cell from Cell A, CU of the NES cell transmits On-demand SIB1 operation activation command to DU of the NES cell, the NES cell starts the OD-SIB1 operation.

If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of NES cells, Cell A should be considered it starts providing WUS config for the NES cells, i.e., no need further instruction from NES cell. In addition, Cell A should be able to reject for providing WUS config for an NES cell even after having provided WUS config for the NES Cells for a period. If Cell A decides to stop providing WUS config for the NES cells later, it transmits the decision to CU of NES cells, so that NES cells can decide to stop the OD-SIB1 operation by taking into account this decision.

Proposal 2:  If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of the NES cells, Cell A should be considered it starts providing WUS config for the NES cells.

Based on RAN2 agreement, a NES cell can include neighbouring NES cell’s WUS configuration, NES cell’s WUS configuration is included in a new SIB (including its own WUS configuration). Therefore, the WUS configuration needs to be exchanged between NES cells as that between Cell A and NES cell. The agreed class 1 procedure about UL WUS configuration transmission should be extended to the Xn between the NES cells. In case of CU-DU split case, after the CU of Cell A or NES cell accepted the WUS configuration, it includes the received WUS configuration in a new SIB and transfers it to the DU of Cell A or NES cell.

Proposal 3: The agreed class 1 procedure about UL WUS configuration transmission is extended to the Xn between the NES cells.

Proposal 4:  If the CU of Cell A or NES cell accepted the WUS configuration, the CU includes the received WUS configuration in a new SIB and transfers it to the DU of Cell A or NES cell.

The OD-SIB1 mechanism is similar as on-demand other SI. RAN2 agreed to use the existing Msg 1 based on-demand procedure for on-demand SIB1 acquisition. If receiving request via MSG1, DU can response via MSG2 and broadcasts SIB1. This procedure has no impact on RAN3. Since CU control a cell to use on-demand SIB1, DU may further inform CU the SIB1 is broadcasting if DU broadcasts SIB1 triggered by UE. Whether this notification has no RAN3 impact is FFS, it depends on RAN1 design how to indicate SIB1 is broadcasting.

Observation 7: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB1 operation to the Cell A. If the OD-SIB1 for an NES cell is not needed, the NES cell can request Cell A to release the WUS configuration for the NES cell by WUS configuration exchange message. In addition, RAN3 has agreed that A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. If Cell A gNB releases the UL WUS configuration after signalling the stopping the UL WUS configuration broadcast, NES gNB transmits the WUS configuration request to Cell A gNB again when the WUS configuration transmission is needed. If Cell A gNB does not releases the UL WUS configuration after signalling the stopping the UL WUS configuration broadcast, the explicit request indication for UL WUS configuration transmission is needed. From our view, the decision for stopping the WUS configuration transmission for the NES cells is semi-static, it may be simpler to release the WUS configuration by avoiding the additional release signalling from NES gNB.

Observation 8: Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB operation to the Cell A.

Proposal 5: If the OD-SIB1 for an NES cell is not needed, NES cell can request Cell A to release the WUS configuration for the NES cell.

Proposal 6: RAN3 to discuss whether Cell A gNB releases the UL WUS configuration after signaling the stopping the UL WUS configuration broadcast in its SIB.

Conclusion

In summary, we have following observations and proposals for on-demand SIB1 for Idle/Inactive UE:

Observation 1: R19 focus on case 2 on on-demand SIB1 for idle/inactive mode UE:

Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Observation 6: A NES cell can include neighboring NES cell’s WUS configuration. NES cell’s WUS configuration is included in a new SIB (including its own WUS configuration).

Observation 7: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Observation 8: Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB operation to the Cell A.

Proposal 1:  If CU of NES cells receive accept response for providing the WUS config for a NES cell from Cell A, CU of the NES cell transmits On-demand SIB1 operation activation command to DU of the NES cell, the NES cell starts the OD-SIB1 operation.

Proposal 2:  If the CU of Cell A receives UL WUS config of a list of NES cells from CU of NES cells and transmits accept response to CU of the NES cells, Cell A should be considered it starts providing WUS config for the NES cells.

Proposal 3: The agreed class 1 procedure about UL WUS configuration transmission is extended to the Xn between the NES cells.

Proposal 4:  If the CU of Cell A or NES cell accepted the WUS configuration, the CU includes the received WUS configuration in a new SIB and transfers it to the DU of Cell A or NES cell.

Proposal 5: If the OD-SIB1 for an NES cell is not needed, NES cell can request Cell A to release the WUS configuration for the NES cell.

Proposal 6: RAN3 to discuss whether Cell A gNB releases the UL WUS configuration after signaling the stopping the UL WUS configuration broadcast in its SIB.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-247441, Discussion on On-demand SIB1 for Idle/Inactive UE, Lenovo