3GPP TSG-RAN WG3 Meeting #127	R3-250447

Athens, Greece, 17th – 21st February 2025

Agenda Item:	16.3

Source:	Ericsson

Title:	[TP for BL CR 38.413] On Location Reporting at reader ID granularity

Document for:	Discussions & Approval

# 1	Introduction

Discussion and text proposal in this document builds upon documents on architecture and NGAP signalling in R3-25xxxx and R3-25xxxx and concerns the unimpressive task for RAN3 in the approved WID (RP-243326):

    - Device location reporting at reader ID granularity

# 2	Discussion

The task to introduce reporting the A-IoT device’s location on reader ID granularity resembles the existing User Location Information reporting defined which is based on the identiy of the cell(s) via which a UE consumes radio resources.

User Location Information reporting relies on attaching respective information to certain groups of messages (related to PDU Session management, UE Context management and mobility) and does not need an explicit procedure.

It should be rather straightforward to agree to follow the same principle for A-IoT location reporting in Rel-19, i.e. to incorporate A-IoT device location reporting on reader ID granularity within the A-IoT Inventory Report and A-IoT Command Report procedures.

Proposal 1:	For A-IoT device location reporting follow the approach for User Location Information reporting and incorporate A-IoT device location reporting information within A-IoT Inventory Report and A-IoT Command Report procedures.

There is the possibility that more than one A-IoT reader can communicate and actually communicates with the A-IoT device. A-IoT device location information reporting should allow A-IoT enabled gNBs to report multiple (e.g. up to 16) reader IDs to the 5GC.

Proposal 2:	A-IoT device location information reporting allows A-IoT enabled gNBs to report multiple (e.g. up to 16) reader IDs to the 5GC.

# 3	Text Proposal for TS 38.413 (based on TP submitted in R3-250446, additions highlighted in yellow)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.x1.2	A-IoT Inventory Report

#### 8.x1.2.1	General

The purpose of the A-IoT Inventory Report procedure is to report to the AMF inventory results. The procedure uses non-UE associated signalling.

#### 8.x1.2.2	Successful Operation

Figure 8.x1.2.2-1: Broadcast Session Release Required, successful operation.

The NG-RAN node initiates the procedure by sending a AIOT INVENTORY REPORT message to the AMF.

The NG-RAN node shall, if supported, report in the AIOT INVENTORY REPORT message location information for each reported A-IoT device in the A-IoT Device Location Information IE.

If the A-IoT Device Location Information IE is included in the AIOT INVENTORY REPORT message, the AMF shall handle this information as specified in TS 23.xxx [FFS].

#### 8.x1.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.x1.4	A-IoT Command Report

#### 8.x1.4.1	General

The purpose of the A-IoT Command Report procedure is to report to the AMF command results. The procedure uses non-UE associated signalling.

#### 8.x1.4.2	Successful Operation

Figure 8.x1.4.2-1: A-IoT Command Report, successful operation.

The NG-RAN node initiates the procedure by sending a AIOT COMMAND REPORT message to the AMF.

The NG-RAN node shall, if supported, report in the AIOT COMMAND REPORT message location information for each reported A-IoT device in the A-IoT Device Location Information IE.

If the A-IoT Device Location Information IE is included in the AIOT COMMAND REPORT message, the AMF shall handle this information as specified in TS 23.xxx [FFS].

#### 8.x1.4.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y5	A-IoT Device Location Information

This IE contains location information for an A-IoT device.

Editor’s Note:	Further details on A-IoT Device Location Information FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Device Location Information List |  | 1 |  |  |
| >A-IoT Device Location Information Item |  | 1..<maxnoofAIoTReaderIDs> |  |  |
| >>A-IoT Reader ID | M |  | 9.3.1.y6 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTReaderIDs | Maximum no. of A_IoT reader identifiers. Value is 4. (FFS) |

#### 9.3.1.y6	A-IoT Reader ID

This IE is the A-IoT reader identification.

Editor’s Note:	Further details on A-IoT Reader ID FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Reader ID | M |  | FFS |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x3.4	A-IoT Inventory Report Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Inventory Report List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Inventory Report Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |
| >>A-IoT Device Location Information | M |  | 9.3.1.y5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x3.8	A-IoT Command Report Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Command Report List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Command Report Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |
| >>A-IoT Command PDU | M |  | OCTET STRING |  | - |  |
| >>A-IoT Device Location Information | M |  | 9.3.1.y5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>