3GPP TSG-RAN WG3 Meeting #127	R3-250458

# Athens, Greece, 17th – 21st February, 2025

Title: 	(TP to 37.483) - Split architecture support for Release 18 use cases

Source: 	Ericsson, Nokia, JIO Platforms (JPL), Deutsche Telekom, FiberCop, Vodafone

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this paper we present a TP to 37.483 for Split architecture support for Release 18 use cases, based on the proposals in R3-250455.

# 2. TP to E1AP – Split architecture support for Release 18 use cases

-------------------------------------- CHANGES START HERE ----------------------------------

### 8.2.9	Resource Status Reporting Initiation

#### 8.2.9.1	General

This procedure is used by an gNB-CU-CP to request the reporting of load measurements to gNB-CU-UP.

The procedure uses non UE-associated signalling.

----

Unmodified text omitted

----

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the RESOURCE STATUS UPDATE message:

-	the HW Capacity Indicator IE, if the second bit, "HW Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Available Capacity Indicator IE, if the first bit, "TNL Available Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Average Packet Delay IE, if the third bit, "Average Packet Delay" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". The reported Average Packet Delay is per DRB.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the RESOURCE STATUS REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the RESOURCE STATUS REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the UE Performance Collection Configuration IE is present in the RESOURCE STATUS REQUEST message, the gNB-CU-UP  shall, if supported, take it into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall, if supported, terminate the collection when at least one of the following conditions is fulfilled:

-	the time since Bearer context was successfully setup at the gNB-CU-UP is equal to the value of the Collection Time Duration for UE Performance IE;

-	Bearer context is released from the gNB-CU-UP

The result of the UE performance collection when at least one of the above conditions is fulfilled is reported at the next available RESOURCE STATUS UPDATE message.

----

Unmodified text omitted

----

#### 9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| Transaction ID |  | M |  |  |  | 9.3.1.53 |  |  |  | YES |  | reject |  |
| gNB-CU-CP Measurement ID |  | M |  |  |  | INTEGER (1..4095,...) |  | Allocated by gNB-CU-CP |  | YES |  | reject |  |
| gNB-CU-UP Measurement ID |  | C-ifRegistrationRequestStop |  |  |  | INTEGER (1..4095,...) |  | Allocated by gNB-CU-UP |  | YES |  | ignore |  |
| Registration Request |  | M |  |  |  | ENUMERATED(start, stop, …) |  | Type of request for which the resource status is required. |  | YES |  | reject |  |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(36)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Periodic. Third Bit = Average Packet Delay, Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity |  | O |  |  |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) |  | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. |  | YES |  | ignore |  |
| UE Performance Collection Configuration |  | O |  |  |  | 9.3.1.xxx. |  |  |  | YES |  | ignore |  |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

----

Unmodified text omitted

----

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  | YES | ignore |
| HW Capacity Indicator | O |  | 9.3.1.73 |  | YES | Ignore |
| UE Associated Info Result list |  | 0..1 |  |  | YES | Ignore |
| >UE Associated info Result Item (FFS) |  | 1 .. < maxnoofUEReports > |  |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

----

Unmodified text omitted

----

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.xxx	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful UE context setup within which the UE performance measurements are collected. Unit: millisecond |

#### 9.3.1.yyy	UE Performance

This IE indicates the UE performance measurements metrics. Structure of this IE is FFS

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.zzz |  |
| Average UE Throughput DL | O |  | Bit Rate 9.3.1.20 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.3.1.20 |  |

#### 9.3.1.zzz	Average Packet Delay

- This IE indicates the gNB-CU-UP part of the average packet delay in the UL and DL directions for a DRB in a UE. Individual components of the delay, e.g., Average delay in DL in CU-UP, Average delay in UL over F1-U, are summed up to generate an Average UL and DL packet delay measurements. Description of this IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314. Unit: 0.1 millisecond |

----

End of changes

----