3GPP TSG-RAN WG3 Meeting #127	R3-250468

Athens, Greece, 17-21 Feb 2025

Title: 	AI/ML enabled CCO across multiple nodes

Source: 	Ericsson, Jio Platforms (JPL), InterDigital

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

To further progress the solution for AI/ML enabled CCO, in this contribution we discuss the following aspects:

- adapt to a future CCO state at neighbour gNB

- handling of multiple CCO issues (predicted and/or detected)

Agreements and open points captured at last RAN3 meeting on AI/ML enabled CCO are captured below.

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

The direction of the information over F1 can be further discussed.

WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

FFS on whether to send time information related to coverage modification cause over Xn.

Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

# 2. Discussion

## Adapt to a future CCO state at neighbour gNB

For AI/ML enabled CCO, according to the agreements reached so far, the following steps are included:

- CU sends to the DU the type of predicted CCO issue, the predicted affected cells and beams and the time when the CCO issue will happen (Predicted CCO Assistance Information)

- DU generates the future CCO state and sends it to the CU together with the time when the future CCO state will be applied (Future Coverage Modification Notification)

For legacy CCO, in a split-gNB, the CU can send to a DU a detected CCO issue and the affected cells and beams (CCO Assistance Information). Note that the affected cells and beams may be served by the same DU or not, as specified in the current procedure text:

F1AP procedure text for CCO Assistance Information with affected cells and beams served by the DU:

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

F1AP procedure text for CCO Assistance Information with affected cells and beams not served by the DU:

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.

With the current signaling, if a CCO issue is detected at gNB1 (CU1 + DU1), a neighbour gNB2 (CU2 + DU2) can adjust the coverage of its cells: the CU1 sends the CCO issue and the CCO state to CU2, and CU2 indicates to DU2 the CCO issue at gNB1 and the cells and beams of gNB1 that are affected. The CU2 does not send to DU2 the CCO state generated by gNB1 (DU1), but the solution works, because DU2 can deduce which CCO state has been adopted for cells and beams of gNB1, for example collecting UE measurements and reports. Considering this information and the configured alternative configurations the DU2 derives a matching CCO state for cells and beams under its control.

For a predicted CCO issue, DU2 needs to receive the future CCO state generated by gNB1 (DU1) because the future CCO state has not been applied yet, and therefore DU2 cannot deduce it. So, to enable DU2 to generate a future CCO state that allows to adjust to the future CCO state generated by gNB1 (or DU1), we propose to resolve the open point from last meeting: “The direction of the information over F1 can be further discussed.” and allow CU2 to send to DU2 the future CCO state generated at gNB1.

Observation 1: Sending the future CCO state generated at one (source) gNB to another (target) neighbour gNB allows the neighbour gNB to adjust the future CCO state of its cells and beams.

Proposal 1: In a split gNB that receives via Xn a future CCO state for a CCO issue predicted at another gNB, the CU forwards the future CCO state to the DU to generate a matching future CCO state for cells and beams of the receiving gNB.

Another observation that can be made is that, if CU2 receives one predicted CCO issue from a neighbour gNB1, CU2 does not need to perform any new CCO issue prediction. CU2 will take the issue predicted by the sending gNB1 and use it to derive its own future CCO states. Namely, the only relevant prediction is the predicted CCO issue received from gNB1 (CU1).

Observation 2: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue.

At last meeting, some discussions took place on whether the time information for the predicted CCO issue should be signalled over the Xn from gNB1 to gNB2, so that this timing information can be signalled from CU2 to DU2. In relation to that, we note that this information is essential to determine when to trigger a future CCO state change. In a split-gNB, the DU decides the future CCO state and when to apply it. It is then important that when a gNB1 (CU1) predicts a CCO issue, it signals it to gNB2 together with the CCO issue prediction time. CU2 can therefore signal the gNB1 predicted CCO issue with its prediction time to DU2, so that DU2 can decide when to apply the matching future CCO state changes.

Proposal 2: CU sends to DU the time information for the predicted CCO issue (in a gNB predicting the CCO issue, and in a neighbour gNB).

## Handling of multiple CCO issues

In this section, we discuss our view on how to handle multiple CCO issues (predicted and/or detected). As an example, we discuss how the CCO solution can work in the following scenario:

- a CCO issue is predicted by a first gNB1, the corresponding future CCO state is generated and signalled to a neighbour gNB2. The gNB2 generates a matching future CCO state to adapt – in the future – to the future CCO state that will be used at gNB1. Before the future CCO states are applied, another CCO issue is detected (or predicted) by gNB2 (or by a third gNB3), which impacts the future CCO states generated in relation to the first predicted CCO issue (from gNB1).

Before we analyse the scenario in more details, we summarize the logical steps used to handle one CCO issue and we show how the same process can be reused when handling multiple CCO issues, regardless of whether the multiple CCO issues at hand are detected, predicted, or a mix of both.

In legacy CCO, the logical steps to handle one detected CCO issue can be summarized as follows:

- CU1 detects CCO issue #1.

- DU1 needs to generate a new CCO state. To enable this, CU1 sends to DU1 the CCO issue #1 and the affected cells and beams (CCO Assistance Information).

- DU1 generates the CCO state #1 to apply immediately and sends it to CU1 (Coverage Modification Notification).

- gNB2 (DU2) needs to generate a matching CCO state. To enable this, gNB1 (CU1) sends to gNB2 (CU2) the detected CCO issue (Coverage Modification Cause) and the CCO state #1 at gNB1 (Cell Coverage State, SSB Coverage State).

- If gNB2 is a split gNB, DU2 needs to generate a matching CCO state. To enable this, CU2 sends to DU2 the detected CCO issue and the affected cells and beams, which are not served by DU2 (CCO Assistance Information).

- DU2 generates a CCO state #2 to adjust the coverage of cells and beams of gNB2 to CCO state #1 and sends it to CU2.

In the legacy CCO solution, if another CCO issue is detected, the new CCO issue is handled through the same logical steps 1-6. So, the steps 1-6 can be regarded as a generic process, or transaction to handle a CCO issue (a “CCO transaction”). We can say that a “CCO transaction” starts when a CCO issue is detected (at gNB1) and stops when the new CCO states are used (including, when applicable, the matching CCO states decided by the neighbour gNBs).

Observation 3: In legacy CCO, the handling of a detected CCO issue is done with a process (e.g., a “CCO transaction”) that starts when the CCO issue is detected and stops when the new CCO states are used (including, when applicable, the matching CCO states decided by the neighbour gNBs).

Let’s discuss now the handling of one predicted CCO issue. The agreements reached at last RAN3 meeting and listed below, together with Proposal 1, support the handling of one predicted CCO issue using the same logic just described for legacy CCO:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

Because of the timing aspects, related to the prediction and the future CCO state, the DU at a neighbor gNB (DU2) needs the additional information discussed in Proposal 1, and described in step 5 below. For the handling of a predicted CCO issue, a CCO transaction includes the following:

- CU1 predicts CCO issue #1.

- DU1 needs to generate a new future CCO state. To enable this, CU1 sends to DU1 the predicted CCO issue #1 and the predicted affected cells and beams (Predicted CCO Assistance Information).

- DU1 generates the future CCO state #1 and sends it to CU1 with the time when the future CCO state #1 will be applied (Future Coverage Modification Notification).

- gNB2 (DU2) needs to generate a matching future CCO state. To enable this, gNB1 (CU1) sends to gNB2 (CU2) the predicted CCO issue (Predicted Coverage Modification Cause) and the future CCO state #1 at gNB1 (Future Cell Coverage State, Future SSB Coverage State).

- If gNB2 is a split gNB, DU2 needs to generate a matching future CCO state. To enable this, CU2 sends to DU2 the predicted CCO issue, the predicted affected cells and beams (not served by DU2), and their future CCO state. The sending of the future CCO state of cells and beams in gNB1 corresponds to Proposal 1, not yet discussed in RAN3.

- DU2 generates a future CCO state #2 to adjust the coverage of cells and beams of gNB2 to future CCO state #1 and sends it to CU2.

We observe that if Proposal 1 is agreed, the same approach (with the exception of signalling DU1 CCO states to DU2), i.e., the same type of process and the same implementation, can be used when only one CCO issue is present, whether the CCO issue is detected or predicted, which allows a simpler solution and implementation. For one predicted CCO issue, the process (“CCO transaction”) starts when the predicted CCO issue is recognized (at gNB1) and it stops when the future CCO states (including the matching future CCO states generated at neighbour gNBs) are applied.

Observation 4: If Proposal 1 is agreed, the same process and the same implementation can be used when only one CCO issue is present, whether the CCO issue is detected or predicted.

Observation 5: The handling of one predicted CCO issue can follow the same logic as the handling of one detected CCO issue, with a process (or CCO transaction) that starts when the CCO issue is predicted and stops when the generated future CCO states (including, if used, the matching future CCO state generated at neighbour gNBs) are applied.

We also observe that if there is only one CCO issue predicted, the predicted CCO issue that CU2 sends to DU2 is the very same predicted CCO issue that CU2 receives from CU1.

Observation 6: When only one CCO issue is predicted, CU2 sends to DU2 the same predicted CCO issue that it received from CU1.

Proposal 3: If one CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).

Let’s now consider an example of multiple CCO issues with three gNBs involved, gNB A, gNB B and gNB C, where each CCO issue (predicted or detected) is handled individually, i.e., the same type of process is used. The scenario is shown in the figure below.

At gNB A, CU-A predicts CCO issue A#1, DU-A generates the future CCO state A#1, sends it to CU-A with the time for future CCO state A#1, and CU-A sends to gNB B (CU-B) the predicted CCO issue A#1, the future CCO state A#1 and the time for future CCO state A#1. Following the steps described previously, DU-B generates a matching future CCO state B#1.

Before the future CCO state A#1 is applied, gNB C (CU-C) detects another CCO issue C#1, DU-C derives a CCO state C#1, and the information is sent to gNB A and to gNB B. The gNB A (DU-A) and the gNB B (DU-B) need to generate (and use immediately) a new CCO state that matches the CCO state C#1. Therefore DU-A generates (and applies) CCO state A#2, DU-B generates (and applies) CCO state B#2. 
In addition to this, gNB A (DU-A) needs to consider that, in light of CCO issue C#1 and the new CCO state A#2, the future CCO state A#1 may not be a good choice anymore. For this reason, DU-A has to update its future CCO state to A#3. The future CCO state A#3 is still associated with the predicted CCO issue #1. To enable gNB B to adapt its future CCO state to the A#3, gNB A needs to send the future CCO state A#3 to gNB B, and DU-B derives a new matching future CCO state B#3.

The proposed example does not intend to represent all possible scenarios where multiple CCO issues are detected or predicted at a given point in time. On the other hand, we think it allows to analyse and discuss some general principles.

In the figure, for example, actions and signaling related to a certain CCO issue are highlighted in different colours (green for the predicted CCO issue, red for the detected CCO issue), to remark that each CCO issue is handled independently. Each colour represents the process (CCO transaction) used to handle the individual CCO issue. An observation that can be made is therefore that when multiple CCO issues (detected or predicted) are present, each CCO issue can be handled separately. The same logic previously described to handle a single CCO issue (detected or predicted) can be fully reused, i.e., one single solution and implementation can be used, also to handle the case of multiple CCO issues.

Observation 7: The same solution used to handle one CCO issue can be reused when multiple CCO issues (detected or predicted) are present, where each CCO issue is handled independently.

If multiple CCO issues are handled independently, we think that sending the time information related to a predicted CCO issue (i.e., related to a coverage modification cause) over Xn is an important part of the solution, as it indicates to the receiver what is the point in time in the future when the issue will occur, on the basis of which the future CCO state time can be calculated. In case multiple predicted CCO issues are present, the timing information related to each one of them can be used to understand when it is more appropriate to apply a certain future CCO state or another. Therefore, we propose to resolve the FFS “FFS on whether to send time information related to coverage modification cause over Xn.” and send this information via Xn.

Proposal 4: Send time information related to coverage modification cause over Xn.

As said earlier, in the figure, one CCO issue (predicted or detected) is associated with a unique process: CCO transaction #1 is associated with the predicted CCO issue A#1 and involves two gNBs, CCO transaction #2 is associated with detected CCO issue C#1 and involves three gNBs.

If we increase the number of involved gNBs and the number of CCO issues, multiple processes will be active at the same time, with some gNBs handling multiple CCO issues at the same time. A modular approach is still possible, where each CCO issue is treated individually using the same type of process. Namely, if we consider that a future CCO state can be generated at gNB A (DU-A) and that one future CCO state is tied to one predicted CCO issue, then that CCO state can be adjusted quickly when gNB A receives a new “input” (another predicted CCO issue, or another detected CCO issue). This approach makes AI/ML enabled CCO fully aligned with legacy CCO.

Observation 8: Handling of each multiple CCO issue as an independent process which can be updated (i.e. the current/future CCO states and/or the affected cells and beams can be updated) ensures alignment between the legacy CCO and the AI/ML supported CCO solution.

If the handling of multiple predicted CCO issues is done using independent processes, one for each issue, a natural choice is that when the CU of a node (e.g., CU-B of gNB B) receives a predicted CCO issue from another node (e.g., CU-A of gNB A), the CU forwards the information to its DU (DU-B). We propose to clarify this in the next proposal.

Proposal 5: A predicted CCO issue signalled via Xn is forwarded over F1 from CU to DU.

If multiple CCO issues are present, it becomes quite complex for a gNB to keep track of which CCO state (or future CCO state) is associated to which CCO issue (predicted or detected), and which nodes should be contacted to match the generated (future) CCO state. In addition, an aspect that is also visible in the proposed example, is that when a new future CCO state is generated due to a new CCO issue detected/predicted, some signaling becomes redundant. In the figure, for instance, in step 15, gNB A sends to gNB B almost the same information as in step 3.

To support a gNB that needs to handle multiple CCO issues at the same time, and that needs to update the CCO state or the affected cells and beams for each CCO issue, we see a benefit in explicitly identifying in F1 and Xn “CCO transactions”, where a CCO transaction consists of the CCO state changes/updates corresponding to a CCO issue (predicted or detected). The use of a CCO transaction identifier enables a gNB to understand what is the CCO Issue at the root of the problem. Only with this knowledge it is possible for the gNB to select/update to the appropriate CCO state.
For example: in step 1, CU-A could send the identity of the CCO transaction (e.g., “CCO transaction #1”) to DU-A; in step 2 DU-A sends the generated future CCO state A#1 together with “CCO transaction #1” and in step 3, CU-A sends to CU-B the CCO information together with “CCO transaction #1”. If the gNB1 becomes aware of any other CCO issue (in the example, the detected CCO issue C#1) before the future CCO state A#1 is applied, gNB1 handles the new CCO issue independently, it generates a future CCO state A#3 (step 14) and in step 15 it sends to the relevant nodes (in this case only gNB B) the future CCO state A#3 together with “CCO transaction #1”, to indicate that the new future CCO state is relevant for CCO transaction #1. Note that the update from gNB1 may also involve a new list of affected cells and beams for the new CCO state A#3.

If there was yet another CCO issue that requires the gNB A to further update the future CCO state associated with CCO transaction #1, such CCO state updates could be signalled together with CCO Transaction #1.

With a “CCO transaction identity” it becomes easy to identify which future CCO state (e.g., which future change in cell/beam coverage) refers to which CCO issue. Therefore, to enable a gNB (or CU, or DU) to follow in a good way how a future CCO state (associated to a predicted CCO issue) and the matching future CCO states (generated by neighbour gNBs) may evolve over time due to multiple predicted/detected CCO issue, we propose to introduce the CCO transaction identity in the IE defined for AI/ML enabled CCO in F1 (Predicted CCO Assistance Information, Future Coverage Modification Notification) and in Xn (Future Coverage Modification List).

Using this identity also allows to avoid unnecessary repetitions in the signaling. Back to the example proposed above, the CCO transaction identity can be sent in step 3 from gNB A to gNB B, and in step 12 there is no need to repeat the information related to the predicted CCO issue and the time for future CCO state.

Based on the above, to support the handling of multiple CCO issues, we propose the introduction of a CCO transaction identity in F1 and Xn.

Proposal 6: To handle multiple CCO issues, introduce a CCO transaction identity in F1 and Xn.

Proposal 7: Agree the TPs to F1AP and XnAP provided in R3-250467 and R3-250466.

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Observation 1: Sending the future CCO state generated at one (source) gNB to another (target) neighbour gNB allows the neighbour gNB to adjust the future CCO state of its cells and beams.

Proposal 1: In a split gNB that receives via Xn a future CCO state for a CCO issue predicted at another gNB, the CU forwards the future CCO state to the DU to generate a matching future CCO state for cells and beams of the receiving gNB.

Observation 2: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue. Proposal 2: CU sends to DU the time information for the predicted CCO issue (in a gNB predicting the CCO issue, and in a neighbour gNB).

Observation 3: In legacy CCO, the handling of a detected CCO issue is done with a process (e.g., a “CCO transaction”) that starts when the CCO issue is detected and stops when the new CCO states are used (including, when applicable, the matching CCO states decided by the neighbour gNBs).

Observation 4: If Proposal 1 is agreed, the same process and the same implementation can be used when only one CCO issue is present, whether the CCO issue is detected or predicted.

Observation 5: The handling of one predicted CCO issue can follow the same logic as the handling of one detected CCO issue, with a process (or CCO transaction) that starts when the CCO issue is predicted and stops when the generated future CCO states (including, if used, the matching future CCO state generated at neighbour gNBs) are applied.

Observation 6: When only one CCO issue is predicted, CU2 sends to DU2 the same predicted CCO issue that it received from CU1.

Proposal 3: If one CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).

Observation 7: The same solution used to handle one CCO issue can be reused when multiple CCO issues (detected or predicted) are present, where each CCO issue is handled independently.

Proposal 4: Send time information related to coverage modification cause over Xn.

Observation 8: Handling of each multiple CCO issue as an independent process which can be updated (i.e. the current/future CCO states and/or the affected cells and beams can be updated) ensures alignment between the legacy CCO and the AI/ML supported CCO solution.

Proposal 5: A predicted CCO issue signalled via Xn is forwarded over F1 from CU to DU.

Proposal 6: To handle multiple CCO issues, introduce a CCO transaction identity in F1 and Xn.

Proposal 7: Agree the TPs to F1AP and XnAP provided in R3-250467 and R3-250466.