3GPP TSG-RAN WG3 Meeting #127	R3-250499

Athens, Greece, 17th - 21st February 2025

Agenda Item:	10.3.1

Source:	ZTE Corporation

Title:	(TP to BL CR for 38.300) Further discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1  Introduction

In the previous meetings, some agreements on SON/MDT enhancements for NTN were reached as below.

Logged MDT enhancement based on scenarios to be worked first, work on immediate MDT if time allows. MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later. Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility. MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. - Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. - FFS on whether it matters if RLF happens within the duration or after the duration period. WA: down-select among below three alternatives. Alt 1: the RLF report includes for single time-based trigger condition: - Absolute time (measured UTC time) when RLF occurred Alt 2: a flag to indicate whether the RLF happens before T1 or after T1 Alt 3: based on existing timers in RLF report. Further check the granularity of the current timers. Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved. MRO for CHO with a location-based trigger condition: For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case. WA: There is no need to report any other info in the RLF report for the location based CHO. MRO for CHO with joint trigger conditions: UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN? New finer area scope for MDT: Is this for signaling based MDT or for management-based MDT? Introduce a geographical area (a reference location plus a radius) as the area scope for MDT or a mapped cell ID list? RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In this contribution, we provide our further views on this topic, including the MRO for intra-NTN failure scenarios.

2  Discussion

2.1  MRO for intra-NTN mobility failure

Considering the CHO with a time-based trigger condition, in last meeting, the conclusion is that the existing timers (i.e. timeSinceCHO-Reconfig) can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved.

The available existing timer with smallest granularity is 100ms, and the derived RLF report time is also in form of 100ms, while the T1’s granularity is 10ms. Therefore the granularity is not aligned, and the accuracy cannot be guaranteed. For example, if the derived RLF report time (T_RLF) is 09:30:00:0100, it could be 09:30:00:0050, 09:30:00:0060, 09:30:00:0070, ..., 09:30:00:0130, 09:30:00:0140. If the T1 is  09:30:0090, the actual T_RLF could be either earlier than T1 or later than T1. Then, the gNB cannot judge whether the RLF happens before or after T1. And the maximum accuracy error could be +/- 50ms which cannot be ignored easily.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

In addition, based on the definition of timeSinceCHO-Reconfig in TS 38.331, this timer can be used to indicate the time elapsed between the RLF and the reception of the latest conditional reconfiguration, considering the long transmission delay between the UE and the gNB with NTN access, it may be different for the gNB to derived the accurate reception time of the latest conditional reconfiguration by the UE. In this case, the accuracy issue could be even worse.

As for the flag to indicate whether the RLF happens before or after T1, it seems to be a compromise way, while, after receiving this flag without the accurate RLF time, the gNB may still not be able to adjust the T1 properly. Therefore, the most straightforward way is to enable the UE to report the time when failure occurs in form of absolute time.

Proposal 1: Regarding the MRO for the intra-NTN CHO with time-based trigger condition, the absolute time information when the failure happens should be included in the RLF report.

For the CHO with location-based trigger condition, in last meeting, there has been the WA that there is no need to report any other info in the RLF report. Considering the earth-moving scenario, the condition Event D2 should be applied. As given in below, the movingReferenceLocation and the corresponding epoch time and satellite ephemeris of the serving cell are provided in SIB19, and the referenceLocation and the corresponding epoch time and satellite ephemeris of the neighbor cell are provided in MeasObjectNR.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

With the above time, the UE is able to decide when to trigger the location-based CHO execution. And the gNB is able to derive the specific moving reference location of the serving cell and the reference location of the neighbor cell when the CHO was executed. In this case, based on the existing location info in the RLF report, the NW is able to adjust the configuration of the location-based trigger condition. Therefore, there is no need for the UE to report other location information for earth moving case.

Proposal 2: Regarding the MRO for intra-NTN CHO with location-based trigger condition, turn the WA into agreement.

2.1.2  MRO for RACH-less handover

In Rel-18 NTN, the RACH-less handover was introduced, and the corresponding stage 2 description is given as below.

---------------------------------------------------------TS 38.300-------------------------------------------------------------------------

9.2.3.6	RACH-less handover

During intra-gNB HO procedure, RACH-less handover can be configured for a UE. The RACH-less handover procedure applies the following functionality:

-	The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0.

-	The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell.

-	The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell.

-	The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

---------------------------------------------------------TS 38.300-------------------------------------------------------------------------

For the RACH-less HO and potential subsequent RACH-based HO, the following 4 scenarios could be considered:

Scenario 1: The configured grant for RACH-less HO has been pre-allocated by the NW and accepted by the the UE, the RACH-less HO is executed and succeed.

Scenario 2: The configured grant for RACH-less HO has been pre-allocated by the NW and accepted by the the UE, the RACH-less HO is executed, but failed.

Scenario 3: The configured grant for RACH-less HO has been pre-allocated by the NW and refused by the UE, the RACH-based HO is executed and succeeded.

Scenario 4: The configured grant for RACH-less HO has been pre-allocated by the NW and refused by the UE, the RACH-based HO is executed, but failed.

For Scenario 1, since the RACH-less HO is succeed, there is not any impact on the UE.

For Scenario 2, since the RACH-less HO is failed which means that the pre-allocated UL grant configured by the NW is not appropriate, and the RACH-less HO related configuration should be reported in the RLF report.

For Scenario 3 and 4, although the RACH-less HO is not executed, the NW should be informed of the unsuccessful RACH-less HO and adjust the inappropriate pre-allocated UL grant. In this case, if the RACH-less HO related configuration is reported by the UE, it implicitly indicate that the RACH-less HO in not executed. The only difference is whether this info should be reported in the RACH report or the RLF report.

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

According to the TS 38.331, for configured grant case,the RSRP threshold configured for SSB selection for the CG configuration and the SSB subset for the SSB for the CG configuration of the RACH-less HO should be reported by the UE.

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

---------------------------------------------------------TS 38.331-------------------------------------------------------------------------

For dynamic grant case, the SSB index in the target cell for initial UL transmission could also be reported by the UE.

Proposal 3: Regarding the MRO for RACH-less handover, the RACH-less HO related configuration (e.g., RSRP threshold and SSB subset for CG configuration, SSB index for DG configuration) should be included in the RACH report and RLF report.

2.1.3  MRO for non-conditional intra-NTN handover

Considering the potential stage 3 impact, since the NG based handover between the NTN base stations should be applied in case of no Xn interface between the NTN base stations, some existing MRO mechanisms only supported over Xn should be also applied over NG, e.g., the missing part over NG, the transfer of assistance information for RRC Re-establishment reporting without RLF report. And the details could be discussed later.

Proposal 4: RAN3 shall apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

2.1.4  Stage 2 update

Since it has been agreed that “Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.”, the corresponding stage 2 description could be update to capture the connection failure scenarios for intra-NTN mobility.

Proposal 5: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

2.2  Logged MDT enhancement

Although RAN3 reached the assumption that both the geographical are and the mapped cell ID as Area Scope of logged MDT over NGAP are feasible in last meeting, for the mapped cell ID, it is deployed by implementation with a fixed geographical area, and its coverage could be smaller or larger than the coverage of the Uu cell. In addition, after the UE receiving the LoggedMeasurementConfiguration with the mapped cell ID, how to interpret this cell ID is still a problem, because the mapping rule of mapped cell ID and geographical area is configured in RAN. Therefore, introducing a finer granularity (i.e. geographical area) is a more straightforward solution for the Area scope and help the MDT to be executed more accurately. However, as sent in [3], the details of geographical area for Area Scope of logged MDT over NGAP should waits for the reply from RAN2.

Proposal 6: Wait for RAN2’s reply on the details of geographical area for Area Scope of logged MDT over NGAP.

3  Conclusion

MRO for intra-NTN mobility failure

Proposal 1: Regarding the MRO for intra-NTN CHO with time-based trigger condition, the time information when the failure happens (e.g. absolute time) should be included in the RLF report.

Proposal 2: Regarding the MRO for intra-NTN CHO with location-based trigger condition, turn the WA into agreement.

Proposal 3: Regarding the MRO for RACH-less handover, the RACH-less HO related configuration (e.g., RSRP threshold and SSB subset for CG configuration, SSB index for DG configuration) should be included in the RACH report and RLF report.

Proposal 4: RAN3 should apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

Proposal 5: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

Logged MDT enhancement

Proposal 6: Wait for RAN2’s reply on the details of geographical area for Area Scope of logged MDT over NGAP.

The corresponding TP is given in the Annex.

References

- TS 38.331 v18.2.0

- TS 38.300 v18.2.0

[3] R3-247903, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, RAN3

Annex

TP for TS 38.300 on connection failure scenarios for intra-NTN mobility

--------------------------------------------------------TS 38.300-------------------------------------------------------------------------

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of intra-NTN mobility, the Too Late Handover, Too Early Handover, Handover to Wrong Cell, Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell are all applied.

--------------------------------------------------------TS 38.300--------------------------------------------------------------------------