3GPP TSG-RAN WG3 #127									R3-250542

# Athens, Greece, 17-21 February 2025

Agenda Item:	9.2

Source:	Ericsson, CATT, ZTE, Lenovo, LG Electronics

Title:	Triggering LTM without L1 measurements UE capability

Document for:	Discussion, Approval

# 1	Introduction

RAN2 has discussed decoupling UE capabilities for L1 measurement and LTM. Following the exchange of LSs between RAN2 and RAN4, RAN2 reached additional agreements regarding support for L3 measurement-based LTM in Release 18. An LS [1] has been sent to RAN3 for further action.

1. Overall Description:

In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution.

During RAN2#127 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting the DU to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards.

2. Actions:

RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE  Layer 3 measurements.

In the last meeting, RAN3 sent a response LS to RAN2 as noted in [4] and requested additional clarification. RAN2 then confirmed the following points in [5].

RAN WG2 understands that early synchronisation is also applicable to L3 measurements based LTM.

It is also RAN WG2 understanding that for L3 measurements based LTM, the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation.

This contribution aims to evaluate the corresponding solution.

# 2	Discussion

## 2.1	Background

The RAN2#126 meeting discussed the decoupling of L1 measurement capability from LTM capabilities. Furthermore, RAN2 sent an LS to RAN4, as mentioned in [2], requesting additional input.

RAN2 discussed RAN1 and RAN4 feature lists associated with Rel-18 NR_Mob_enh2.  RAN2 discussed wither the following L1 measurement and reporting features are mandatory to support LTM or whether L3 measurement could be used.

| 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |
| --- | --- |
| 45-1a | Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure |

RAN2 made the following agreement:

RAN2 makes no further assumptions whether L3 measurements can be used or not to trigger LTM.

RAN2 would like to check the following with RAN1 and RAN4:

Question 1 : Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively?

RAN4’s response is provided in [3].

For whether intra-frequency and inter-frequency L1 measurements and reporting are prerequisites to support intra-frequency and inter-frequency LTM, RAN4 analysed RAN4 requirements and achieved following agreements

- For FR1, RAN4 Rel-18 related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells.

- For FR2, RAN4 Rel-18 related LTM requirements are not applicable if L1 measurements for LTM candidate cells are not supported by the UE, or not configured by the network.

After receiving the reply, RAN2 agreed to support L3 measurements triggered by LTM. Additionally, RAN2 and RAN3 confirmed that early synchronization can also be triggered by L3 measurements, with the CU making the decision based on these measurements and informing the DU.

## 2.2	Proposals

The scenario involves the CU informing the DU to trigger a MAC CE command to the UE when the CU decides to execute LTM or trigger early sychronization based on the L3 measurements received.

As well known, The CU processes L3 measurements, including beam-level information, to determine the activation of a TCI state. However, L3 measurements should not be transmitted from the CU to the DU. The CU is responsible for making handover and beam selection decisions based on these measurements, ensuring no performance degradation whether the CU provides the SSB index and Cell ID or retains decision-making at the CU level. This distinction remains an implementation choice.

Throughout the signaling phase, the UE remains unaffected by interruptions as this process occurs entirely within the network. The CU identifies the necessity for triggering a procedure and subsequently instructs the DU. The interruption is only initiated when the UE receives the TCI state activation MAC CE. The DU ensures timely transmission of the MAC CE just before the SSB to optimize synchronization with the LTM candidate cell.

This discussion is independent of RAN4 requirements. Any interruption occurs strictly when the UE receives the LTM cell switch MAC CE or the PDCCH order. The primary consideration is whether the network can send the LTM cell switch MAC CE within 160ms following the PDCCH order or TCI state activation MAC CE. Failure to meet this requirement is unrelated to whether the CU provides the SSB index and Cell ID or L3 measurement to the DU.

The CU should retain responsibility for handling L3 information, ensuring compliance with functional split principles in the gNB architecture.

RAN4 requirements are separate from RAN3's discussion on split architecture and remain unaffected by RAN3 solution.

RAN3 moves forward with CU-based solution.

Several options are considered:

Option 1: Introduce an indicator over the F1 interface.

- Option 1a: Add this indicator to an existing Class-1 procedure.

- Option 1b: Add this indicator to an existing Class-2 procedure.

- Option 1c: Introduce a new Class-2 procedure specifically for this purpose.

Option 2: Propagate part of the measurement results from the CU to the DU.

In a split network architecture, the CU manages non-real-time functions, including higher-layer L2 and Layer 3 processing. Therefore, the CU should take a decisions based on L3 measurements if LTM execution should be triggered and to which of candidate cells it should be performed Option 2 does not adhere to this principle.

Among Options 1a, 1b, and 1c, it is known that the UE Context Modification procedure and the Cell Switch Notification procedure are used for Rel-18 LTM, however those procedures were intended for different functions. Therefore, introducing a new procedure is the most straightforward approach.

RAN3 agrees with Option 1c, allowing the CU to make decisions based on received L3 measurements and tell the DU to execute LTM and/or initiate early synchronization.

Below is an example illustrating Option 1c. A point code is required to specify the purpose of triggering this procedure, such as for a cell switch, early UL/DL synchronization, or both based on the same L3 measurements.

#### 9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to inform the gNB-DU to trigger a cell switch command and/or early synchronization to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Triggering Information |  |  | 1 |  | YES | reject |
| >Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |

Introduce a new Class 2 procedure over F1 to allow the CU to instruct the DU to trigger LTM and/or early synchronization based on the L3 measurements received from the UE.

Agree the CR to TS 38.473 in [6], the CR to TS 38.401 in [7], and the CR to TS 38.470 in [8].

# 3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1	The CU should retain responsibility for handling L3 information, ensuring compliance with functional split principles in the gNB architecture.

Observation 2	RAN4 requirements are separate from RAN3's discussion on split architecture and remain unaffected by RAN3 solution.

Proposal 1	RAN3 moves forward with CU-based solution.

Proposal 2	RAN3 agrees with Option 1c, allowing the CU to make decisions based on received L3 measurements and tell the DU to execute LTM and/or initiate early synchronization.

Proposal 3	Introduce a new Class 2 procedure over F1 to allow the CU to instruct the DU to trigger LTM and/or early synchronization based on the L3 measurements received from the UE.

Proposal 4	Agree the CR to TS 38.473 in [6], the CR to TS 38.401 in [7], and the CR to TS 38.470 in [8].

# 4	References

R2-2407606, L3 measurement based LTM support over F1, RAN2

R2-2404014, LS on LTM L1 intra and inter-frequency measurements, RAN2

R4-2410303, Reply LS on LTM L1 intra and inter-frequency measurements, RAN4

R3-245710, Reply LS on L3 measurement based LTM support over F1, RAN3

R3-247013, L3 measurement based LTM support over F1, RAN2

R3-250543, F1 support for L3 measurements-based LTM, Ericsson, Vodafone, CATT, ZTE, NTT DoCoMo, Nokia, Samsung, LG Electronics, Lenovo

R3-250584, Stage 2 updates for support of L3 measurements based LTM, CATT, Ericsson, Vodafone, ZTE, Google, Samsung, LG Electronics, NTT DoCoMo, Nokia

R3-250115, F1 support for L3 measurements-based LTM, ZTE, Ericsson, CATT, Vodafone, LG Electronics, Samsung