3GPP TSG-RAN WG3 Meeting #127	R3-250586

Athens, GR, 17-21 Feb, 2025

Title: 	Discussion on energy saving indication from CN to RAN

Source: 	Huawei, CATT, NEC

Agenda item:	17.4

Document Type:	 Discussion

# 1. Introduction

The last SA2-166 meeting sent an LS on energy saving indication from CN to RAN as follows [1].

As part of the Rel-19 EnergySys work SA2 approved the attached 23.501CR5739. SA2 would like RAN2 and RAN3 to provide feedback on the Energy Saving Indicator that may be provided from AMF to NG-RAN.

This contribution provides our views on the received LS.

# 2. Discussion

## 2.1 Background

SA2 studied the R19 Energy Efficiency and Energy Saving, and concluded in the TR 23.700-66. The contents within the incoming LS are related to the key issue #2: Subscription and policy control. The related conclusion is excerpted below, where it can be observed that the PCF may make policy decisions based on the energy related information per UE, but not mentioning the NG-RAN node.

The following principles for enhancements on subscription and policy control are agreed for the normative work: - The following information is stored as part of the subscription data in the UDM/UDR: - Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE. - The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

- Observation 1:  The conclusion of TR 23.700-66 only described that the PCF can make policy decisions based on the energy saving subscription information per UE, but not in relation to the NG-RAN node.

Also, the incoming LS attaches the agreed TS 23.501 CR5739, which is copied as follows.

### 5.X.Y Subscription information aspects The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF. Editor’s Note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Editor’s Note: Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS. Editor’s Note: Whether the Energy Saving indicator is also stored in the UDR is FFS. ### 5.X.Z Policy control aspects During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.x.y. Based on the Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy. Editor’s Note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.

From the above, SA2 agreed the subscription information aspects, including:

- the energy saving indication is UE subscription data. The AMF may provide it to the PCF

- the PCF can make policy decisions based on the energy saving indicator.

- It remains FFS whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN.

- Observation 2:  It remains an editor’s note whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN.

## 2.2 Incoming LS analysis

The LS asks RAN feedback on the energy saving indicator from the AMF to the NG-RAN.

First, it is not clear what the NG-RAN node behaviour upon receiving the energy saving indicator at a UE level. One possible NG-RAN’s implementation is that the NG-RAN node could have a different network energy saving handlings towards this kind of UEs, while not for other UEs. However, it is not clear what the exact network energy saving mechanisms the NG-RAN node should take for this particular kind of UEs. It should keep in mind that the main goal of the NG-RAN node is to fulfil the UE service requirements, while saving the NG-RAN node energy consumption as much as possible. On this aspect, even without the energy saving indicator, the NG-RAN node should always try to save its power consumption whenever possible.

- Observation 3: The primary objective of NG-RAN nodes is to meet the service requirements of UE as dictated by the CN, while concurrently optimizing energy consumption to the fullest extent possible, even without the necessity of an explicit network saving indicator.

Second, from the agreed CR above, the AMF would provide the energy saving indicator to the PCF, then the PCF can provide general policy control functions.  As specified in TS 23.501, the PCF holds the following functions:

The Policy Control Function (PCF) includes the following functionality: - Supports unified policy framework to govern network behaviour. - Provides policy rules to Control Plane function(s) to enforce them. - Accesses subscription information relevant for policy decisions in a Unified Data Repository (UDR). - Support PDU Set Handling as defined in clause 5.37.5. NOTE: The PCF accesses the UDR located in the same PLMN as the PCF. The details of the PCF functionality are defined in clause 6.2.1 of TS 23.503 [45].

From the above, it can be observed that it is already possible that the PCF can provide the policy rules to the CP functions. And whenever needed, the CN CP function (e.g., the PCF/SMF) can provide the updated UE service requirements (e.g., in terms of updated AM policies including RFSP, UE-AMBR etc) to the NG-RAN. In this sense, it is already possible for the NG-RAN to take its own proprietary RRM mechanisms to fulfil the requirement without the explicit the network saving indicator.

- Observation 4: It is already possible for the NG-RAN to take its own proprietary RRM mechanisms to fulfil the requirement after the CN (e.g., PCF/SMF) makes policy decisions and updates to the NG-RAN node.

In addition, starting from Rel-18 a set of UE level network energy saving radio capabilities are introduced, including the Cell DTX-DRX, conditional handover etc. Even for Rel-19, it is expected that a set of UE level capabilities would be introduced, to support the on-demand SSB SCell, or the on-demand SIB1 feature. With the set of the UE capabilities, the NG-RAN can take the NES actions already. In this way, there is no need for the CN to provide more information to assist network NES operations. And it seems strange if the UE supports the NES features, while the CN does not provide the network energy saving indicator to the NG-RAN node for this UE.

- Observation 5: Based on the UE radio capabilities, the NG-RAN can take the NES actions already. And it seems strange if the UE supports the NES features, while the CN does not provide the network energy saving indicator to the NG-RAN node for this UE.

Finally, there may have some arguments for the OAM e.g., how many UEs are subscribed with the energy saving function. But it is better to involve SA5 first.

- Observation 6: If there is a requirement for OAM to be informed e.g., the number of UEs utilizing energy-saving features, it would be more appropriate to initially engage SA5 first.

In summary, based on the discussions outlined above, we propose so far there seems no need for the RAN to be aware of the network saving indicator. And it would be prudent to communicate these points back to SA2 for their review and input. This will ensure that all relevant aspects have been thoroughly evaluated before a final decision is made. Meanwhile, it is up to RAN2 to discuss and feedback whether the indicator is needed from their perspective, e.g., the radio configuration aspects.

RAN3 to reply that from RAN3 perspective, there seems no such need to have this energy saving indication. And RAN3 can communicate the above points back to SA2 for their further input before to have the signalling design.

RAN3 can further consider based potential RAN2 and SA2 feedback.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Background

- Observation 1:  The conclusion of TR 23.700-66 only described that the PCF can make policy decisions based on the energy saving subscription information per UE, but not in relation to the NG-RAN node.

- Observation 2:  It remains an editor’s note whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN.

- Analysis of the LS

- Observation 3: The primary objective of NG-RAN nodes is to meet the service requirements of UE as dictated by the CN, while concurrently optimizing energy consumption to the fullest extent possible, even without the necessity of an explicit network saving indicator.

- Observation 4: It is already possible for the NG-RAN to take its own proprietary RRM mechanisms to fulfil the requirement after the CN (e.g., PCF/SMF) makes policy decisions and updates to the NG-RAN node.

- Observation 5: Based on the UE radio capabilities, the NG-RAN can take the NES actions already. And it seems strange if the UE supports the NES features, while the CN does not provide the network energy saving indicator to the NG-RAN node for this UE.

- Observation 6: If there is a requirement for OAM to be informed e.g., the number of UEs utilizing energy-saving features, it would be more appropriate to initially engage SA5 first.

- RAN3 to reply that from RAN3 perspective, there seems no such need to have this energy saving indication. And RAN3 can communicate the above points back to SA2 for their further input before to have the signalling design.

- RAN3 can further consider based potential RAN2 and SA2 feedback.

The draft reply LS is provided in [3].

# 4. Reference

- LS on energy saving indication from CN to RAN, SA2

- 3GPP TR 28.880 V1.1.0 Study on energy efficiency and energy saving aspects of 5G networks and services

(Release 19)

- R3-250587 [draft] Reply LS on energy saving indication from CN to RAN, Huawei, CATT, NEC