3GPP TSG-RAN WG3 #127	          R3-250637

# Athens, Greece, 17th – 21th Feb 2025

Agenda item:	16.3

Source: 	Samsung

Title: 	Discussion on Location report for AIoT

Document for:	Discussion and decision

Introduction

The Ambient IoT SI has been transferred to a WI, with the following RAN3 scope in terms of location report,

Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

- Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we discuss the fundamental aspects to be specified.

Discussion

The Ambient IoT WI has been approved in the last RANP meeting. According to the WID, in general only D1T1 with Device 1 is considered in this WI.

Recall that in TR 38.769, the primary message follow for AIoT inventory and command operations have been agreed and captured as follows,

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

We can observe from the above message flow that, the device location reporting is provided from AIoT RAN to AIoTF in the Inventory Report message and the Command Response message.

In addition, in TR 38.769, for T1 the following text has been captured from the perspective of locating AIoT devices,

For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages.

The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded.

NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion.

NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion.

NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

As observed by the above quoted text, the first question to check is whether an AIoT RAN node can have multiple Readers. In our understanding, there’s no doubt that an AIoT RAN node can have multiple readers because of the limited coverage for each reader.

The next question, as indicated by NOTE 1, whether multiple Reader IDs can be included in the Inventory Report/Command Response message. In our understanding, even though an AIoT device is located in an overlapping coverage area of multiple readers, the AIoT device only needs to reply once during inventory procedure or command procedure; otherwise there will be more chances for collision during potential random access. Under such case, the AIoT RAN node is only able to report one Reader ID to the AIoTF. And till now we see no clear advantage for the AIoT device to reply multiple times to the same inventory procedure or command procedure. Although whether the AIoT device can only reply once or multiple times depends on RAN2 discussion, RAN3 can firstly assume that at most one Reader ID is included in the Inventory Report/Command Response message.

Proposal 1: RAN3 assumes that at most one Reader ID is included in the Inventory Report/Command Response message.

The next question, as indicated by NOTE 2, what is the definition and format of the AIoT RAN node ID and Reader ID. For AIoT RAN node ID, the current Global RAN Node ID IE as defined in TS 38.413 can be reused. While for the Reader ID, the current TRP ID IE as defined in TS 38.455 can be reused.

Proposal 2: The Reader ID included in the Inventory Report/Command Response message can be TRP ID which uniquely identifies a TRP/Reader within an AIoT RAN node.

Conclusion

In this paper, we provide primary discussion on location reporting for Ambient IoT. The following proposals are provided,

Proposal 1: RAN3 assumes that at most one Reader ID is included in the Inventory Report/Command Response message.

Proposal 2: The Reader ID included in the Inventory Report/Command Response message can be TRP ID which uniquely identifies a TRP/Reader within an AIoT RAN node.

Reference