3GPP TSG-RAN WG3 Meeting #127	R3-250647

Athens, Greece, February 17th – 21st, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Keep continuing discussions on Inter-CU LTM signaling design

Document for:	Discussion and Decision

# Introduction

The discussions for Rel-19 Inter-CU LTM have consumed 5 meetings so far (from April 2024). The related RAN3 and RAN2 agreements are provided in Annex A and B, respectively.

In this contribution, we continue providing our views for Rel-19 Inter-CU LTM design.

# Discussion

## Late data forwarding still needs a partner

Regarding Inter-CU LTM data forwarding, RAN3 has progressed as follows:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

While RAN3 has already agreed to support early data forwarding for Inter-CU LTM, some companies attempted to reverse this agreement in the last RAN3 meeting, proposing to specify only late (normal) data forwarding for Inter-CU LTM.

Clearly, a true education is necessary:

Yes, initiating normal (late) data forwarding immediately after cell switch command without waiting for HO SUCCESS works. We don’t disagree. However, it only works from a pure “data forwarding” point of view. To understand deeper, it is important to understand the principle behind the differentiation of early and late data forwarding in the past. Please stay tuned.

The purpose of early data forwarding is to ensure that DL packets are ready for transmission by the target cell as soon as the UE successfully accesses. That’s why it is named “early”. However, if only normal data forwarding right after cell switch command is specified (i.e. early data forwarding is not even an option), there would be no guarantee that the forwarded DL packets are ready for transmission at the target cell (which belongs to different CU), considering F1 delay from S-DU to S-CU, and Xn forwarding delay from S-CU to the target CU.

Moreover, there seems a mis-understanding that this early data forwarding concept only existed for conditional mobility families in previous releases (i.e. where multiple candidate cells are prepared for robustness). In fact, early data forwarding was also specified for Rel-16 DAPS (Dual Active Protocol Stack) handover [1], which involved only one target cell, as its critical component. In Rel-16 DAPS, the source gNB connection is maintained until the UE successfully accesses the target gNB. The goal was to reduce handover interruption to 0ms, and early data forwarding was introduced to ensure that DL packets are available at the target gNB as early as possible. Only after the source gNB received HO SUCCESS from the target gNB (confirming that the UE successfully accessed the target cell), normal data forwarding was triggered, and the PDCP SN assignment role was transferred to the target gNB.

Late data forwarding is closely tied to PDCP SN assignment "freezing". From DL point of view, when normal data forwarding is triggered, the source gNB stops assigning PDCP SNs and its role is handed over to the target gNB. Given this, we understand that it might seem natural for the S-CU, who knows that its S-DU has triggered a cell switch command (from cell switch notification), to immediately stop assigning PDCP SNs and transfer its role to the target CU, even without waiting for HO SUCCESS. We don’t disagree that this process works. But please note that this is not related to the purpose of early data forwarding, which is to ensure packets are ready at the target CU for immediate transmissions to the UE for improved HO performances.

It is right that HO SUCCESS has been a distinguishing point between early and late data forwarding in the past. However, this does not mean that we should follow the same for Inter-CU LTM and normal (late) data forwarding should always wait for HO SUCCESS. Given the network-triggered nature of LTM, for Inter-CU LTM, the exact timing of late (normal) data forwarding can be left to implementation. Overall, data forwarding is an optional feature (that’s why our agreement always say “can be” or “may”) and the timing of when to initiate early or late data forwarding has been left to implementation. For Rel-19 Inter-CU LTM, there is no problem to leave the door more (wide) open for S-CU to initiate late data forwarding immediately after a cell switch command is issued to the UE (i.e. without waiting for HO SUCCESS).

Proposal 1: Honor the previous RAN3 agreements for Inter-CU LTM early data forwarding:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Proposal 2: For Inter-CU LTM, allow normal (late) data forwarding to be initiated right after cell switch command to a UE without waiting for HO SUCCESS (when exactly is up to implementation as usual).

## Still incomplete E1AP BL CR

RAN3 endorsed E1AP BL CR supported by many companies [2], for which the rapporteur appreciates.

While some early data forwarding related procedures have been addressed for LTM, there is still one remaining aspect for the Bearer Context handling:

- LTM shares the similar design philosophy with conditional mobility, where serving cell switch does not happen immediately upon RRC configured. As a result, the CHO Initiation IE in the BEARER CONTEXT SETUP REQEUST message for one-bearer-context handling in CU-UP for multiple CHO requests applies the same to LTM.

- From CU-UP standpoint, no difference observed between traditional LTM and Conditional LTM.

Given these, we think the usage of the CHO Initiation IE in the BEARER CONTEXT SETUP REQUEST message [3] simply needs to be extended to cater for LTM (which was neglected in Rel-18), as follows:

If the CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns conditional handover or conditional PSCell change or conditional PSCell addition or subsequent CPAC or LTM and act as specified in TS 38.401 [2].

Proposal 3: Extend the usage of the CHO Initiation IE in the E1AP BEARER CONTEXT SETUP REQUEST message to cater for LTM as well.

The corresponding TP for E1AP BL CR [2] is also provided in Annex C.

## Inter-CU LTM preparation details

### Per-cell LTM preparation: how to avoid multiple UE-associations

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

While RAN3 finally agreed to move forward with per-cell HO preparation, one important issue remained is how to avoid multiple UE-associations between two CUs over Xn [4].

If we do nothing, then the current XnAP signalling may result in multiple UE-associations for the same UE between two CUs for different candidate cells. In previous releases, this was allowed by the source CU and the target CU treated each UE-association separately, as if they belonged to different UEs. Also, managing these multiple UE-associations was left to the source CU's implementation.

This was acceptable in previous releases because CHO was designed for a one-time cell switch, after which candidate cell configurations were released in both the UE and NW following a successful execution. However, regarding UE-association, Rel-19 Inter-CU LTM introduces more complex interactions over XnAP, not only during the preparation phase but also after each cell switch (for subsequent LTM, shown below):

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

As a result, allowing multiple UE-associations for Inter-CU LTM would create several challenges: (a) It would complicate the identification of the same UE when establishing UE-association in-between C-CUs for subsequent LTM, due to each C-CU having multiple UE associations and multiple pair of UE XnAP IDs associated with S-CU; (b) Needless to say, multiple UE-associations (each tied with different candidate cells) would complicate error handlings throughout LTM operations.

Given these issues, we think it is critical to prevent multiple UE-associations from the initial LTM preparation phase. For that, [5] also provided insights on how to avoid multiple UE-associations:

As a compromise, we propose another way to avoid the complex error handling, and still use one Handover Preparation to prepare only one LTM candidate cell. When the first Handover Preparation is done, to prepare the first LTM candidate cell controlled by the target gNB, the UE association is established and the pair “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID” is created. When the following Handover Preparation procedures are done, to prepare the other LTM candidate cells controlled by the same target CU (second, third, etc.), the source CU signals to the target CU the Target NG-RAN node UE XnAP ID already assigned by the target CU for the UE. With this information, the target CU can understand that there is an already existing UE association for the UE and avoids assigning a new Target NG-RAN node UE XnAP ID. It seems therefore possible, to resolve the error handling concerning multiple UE associations in stage 3.

The suggested approach is to assign and provide the same Target NG-RAN node UE XnAP ID to the same C-CU. While this works when additional LTM preparation is triggered for additional candidate cell under a C-CU that already has an existing UE-association with S-CU, it doesn’t address the case when parallel LTM requests are initiated from the beginning, which is expected to occur frequently in Inter-CU LTM.

Overall, we think the best and simplest solution is to enforce a single UE-association between CUs during Inter-CU LTM preparation phase. This approach would ensure that (a) the risk of multiple UE-associations is blocked from the start; (b) No additional UE XnAP ID needs to be encoded into HO REQ message (avoiding unnecessary stage-3 impact); and (c) Any CU can easily identify and maintain a single UE-association with other CU throughout LTM operations. By adopting this approach, we can effectively eliminate unnecessary complexities and streamline UE-association management over Xn for Inter-CU LTM.

Proposal 4: To ease XnAP signalling management and error handling between CUs throughout LTM operations, during HO preparation, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and a single UE-associated signaling connection is established/maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

### The second round-trip (“LTM Configuration Update”) should be marked mandatory or optional?

6. The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration. 7. The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration. Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

When multiple entities are involved, a two-round-trip interaction between entities is deemed inevitable for subsequent LTM because admission control and finalizing LTM configurations are not expected to be performed simultaneously. The first round-trip involves initial context setup and retrieval of necessary configurations and resources, allowing the source to collect configurations and information from all the admitted candidate cells to generate a common configuration for these cells. The second round-trip is used to exchange the collected information and finalize LTM configurations to the UE from each involved entity.

Having this in mind, RAN3 previously discussed a similar issue at Rel-18 Inter-DU LTM, where optional was decided for the second round-trip to a candidate DU (TS 38.401 Figure 8.2.1.5-1, Steps 7-8). Though it was considered necessary for subsequent LTM, this decision was made from the fact that LTM configurations for subsequent cell switch can be prepared and configured to the UE at a later stage (i.e. after the initial LTM configuration, with the expense of additional RRC reconfiguration).

Another consideration was to cater for some “ideal” scenario (i.e. unusual), where (A) all the candidate cells chosen by CU lie in C-DU(s) (not in S-DU); and (B) those were all successfully admitted by C-DU(s) during Steps 3-4, so that CU doesn’t have to update the (common) CSI resource configuration of candidate cells to update the CSI report configuration of each candidate cell from C-DU(s). In Rel-18, only SSB-based L1 measurement was supported, and the (cell-specific) SMTC information provided to CU via the legacy non-UE associated F1 signaling (via F1 SETUP REQEST or gNB-DU CONFIGURATION UPDATE, through 9.3.1.10 Served Cell Information > Measurement Timing Configuration IE) was enough for CU to collect and generate the common CSI resource configuration for LTM. As a result, CU can provide the (common) resource pool of candidate cells from the beginning.

In Rel-19, “CSI-RS”-based L1 measurement is in the scope [6], whose resource configuration is UE-specific. This means that S-CU who initiates Inter-CU LTM first need to retrieve the UE-specific CSI-RS resource configuration allocated by each C-CU to construct the (common) CSI resource pool from all the candidate cells. However, this does not mean the Steps 6-7 should be mandatory, as it is still true that that those steps with each C-CU can be omitted if S-CU chooses to address subsequent LTM at a later stage.

Moreover, if we focus on “SSB”-based (also in the scope by default), the same logic in Rel-18 applies to Rel-19 Inter-CU LTM. The (cell-specific) SMTC information is already exchanged via the legacy non-UE associated Xn signalling (via Xn SETUP REQUEST/RESPONSE or NG-RAN NODE CONFIGURATION UPDATE/ACKNOWLEDGE, through 9.2.2.11 Served Cell Information NR or 9.2.2.13 Neighbour Information NR). This is why RAN3-125bis agreed the following:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

Then, the “ideal” scenario is still possible for SSB based CSI resource pool, where (A) all the candidate cells chosen by S-CU lie in (other) candidate CU(s); and (B) those were all successfully admitted by C-CU(s) during Steps 3-4, so that S-CU doesn’t have to update the (common) CSI resource configuration of candidate cells to update the CSI report configuration of each candidate cell from C-CU(s).

Proposal 5: The second round-trip (“LTM Configuration Update”) of Steps 6-7 should be marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM. Even for “CSI-RS”-based resources (whose resource configuration is UE-specific), the second round-trip can be skipped if S-CU chooses to address the subsequent LTM at a later stage.

### The second round-trip (“LTM Configuration Update”): per cell vs multiple?

The current XnAP BL CR [7] captured the initial stage-3 structure as per-cell basis:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| CSI-RS Resource Configuration | O |  | OCTET STRING |  | YES | ignore |

#### 9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| CSI-RS Report Configuration | O |  | OCTET STRING (FFS) |  | YES | ignore |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Note that the purpose of the second round-trip is not about modifying radio bearer details that have already been admitted. Instead, it is more like a post-LTM-preparation step to finalize LTM configurations to the UE with each C-CU, based on the common information collected from all the admitted candidate cells. The essence of LTM design lies on common CSI resource configuration and cell-specific CSI report configuration. Any update or change to the common information collected at S-CU (e.g. any modification to a set of candidate cells) may require updating the LTM configuration generated by each C-CU wholistically. If there is more than one candidate cell admitted in a C-CU, the LTM Configuration Update toward that C-CU is better to cater for those multiple candidate cells by one-shot.

Another point is that this new procedure is now being specified from scratch. This is different to the first round-trip procedure, where our beloved HO signalling has been designed per-cell basis from the beginning of NR (from Rel-15). But this new “LTM Configuration Update” procedure permits full freedom to specify.

Proposal 6: Design the stage-3 messages of the second round-trip (“LTM Configuration Update”) procedure to cater for the update of multiple candidate cells configurations simultaneously (i.e. not per-cell basis as in the existing HO procedure).

### The second round-trip (“LTM Configuration Update”): who can initiate?

In [7], the procedure is captured to be initiated by any involved CU. But this needs to be fixed to be triggered from the source. According to the current 38.300 BL CR [8], it is triggered from S-CU during preparation phase. Considering the potential usage of this procedure for subsequent LTM, it is expected to be triggered from the CU who is currently serving the UE (i.e. “source”).

Proposal 7: Fix the second round-trip (“LTM Configuration Update”), to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during preparation phase, and also expected to be initiated from CU who is currently serving the UE (i.e. “source”) during subsequent LTM.

### LTM Cell ID in XnAP: NR CGI (9.2.2.7) vs Target Cell Global ID (9.2.3.5)

It is true that the scope of LTM is in NR, but RAN3 has been using 9.2.3.5 Target Cell Global ID, which includes NR CGI as well:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.2.3.25	Target Cell Global ID

This IE contains either an NR CGI or an E-UTRA CGI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Target Cell | M |  |  |  |
| >NR |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.7 |  |
| >E-UTRA |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.2.8 |  |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

For forward compatibility, we suggest to stick to the Target Cell Global ID.

Proposal 8: For LTM Cell ID in XnAP, use 9.2.3.5 Target Cell Global ID (instead of 9.2.2.7 NR CGI).

### Details of information exchange between S-CU and C-CU

Given the Rel-18 Intra-CU LTM designs serve as baseline, the information exchanged during the first round-trip (the existing HO procedure, triggered per candidate cell) and during the second round-trip (new UE associated class-1 “LTM Configuration Update”, per node) can mirror what we have specified in Rel-18 and also from S-CPAC.

The current works captured in the XnAP BL CR [7] are still far from complete:

Proposal 9: During the first round-trip (the existing HO procedure), the below information should be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - (O) LTM initiation indication (already by LTM Information Request IE, but requires the underlying (M) “LTM Information Setup” as in F1AP to avoid all optionals?)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but “request” is missing)

  - (O) SSB / CSI-RS resource configuration of candidate cells, if available (already by LTM Information Request IE > CSI Resource Configuration IE)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE)

  - (O) A request for CSI-RS resource configuration (already by LTM Information Request IE > Request for CSI-RS Configuration IE)

  - (O) Early Sync Information Request (already by Early Sync Information Request IE, but needs more discussion).

- (per-cell) HO REQ ACK message to include:

  - (O) LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, but the semantic of the mandatory existing container in HO REQ RESP shall be updated to be ignored if included)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE)

  - (O) CSI-RS resource configuration allocated from the admitted cell (already by LTM Information Response IE > CSI-RS Configuration IE)

  - (O) Early Sync Information, which contains DL/UL early sync info allocated from the admitted candidate cell (already by Early Sync Information Response IE, but needs more discussion)

  - (O) Data Forwarding proposal to be used for subsequent LTM (not implemented yet)

Proposal 10: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information should be exchanged:

- (per-node) LTM Configuration Update REQ message to include:

  - (M) Target Cell Global ID (shall be removed / re-designed for multi-cell handling)

  - (O) Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell (already by the respective Early Sync Information / SUL IEs, but needs more discussions)

  - No IE for requesting the updated Early Sync Information List

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but “request” is missing)

  - (O) SSB / CSI-RS resource configuration of all the candidate cells admitted by other CU(s) (already by LTM Information Request IE > CSI Resource Configuration IE –repeated message-level CSI-RS Resource Configuration shall be removed)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE)

  - (O) A request for CSI-RS resource configuration (already by LTM Information Request IE > Request for CSI-RS Configuration IE)

  - (O) Data Forwarding TNLs assigned by other involved CU(s) (not implemented yet)

  - (O) Data Forwarding proposal from the requesting CU (not implemented yet)

- (per-node) LTM Configuration Update ACK message to include:

  - (M) Target Cell Global ID (shall be removed / re-designed for multi-cell handling)

  - No IE for providing the updated Early Sync Information List

  - (O) Updated LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, FFS whether the message-level CSI-RS Report Configuration shall be removed if included into the container)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE)

  - (O) Updated CSI-RS resource configuration (already by LTM Information Response IE > CSI-RS Configuration IE)

  - (O) Data Forwarding TNLs assigned by the requested CU (not implemented yet)

Other information may need further discussions, but the above information exchange over the first/second round-trips are deemed essential for Inter-CU LTM preparation across S-CU and C-CU(s).

## UE-associated Xn signalling connections between C-CUs

The Inter-CU LTM can be viewed as a series of NW-triggered conditional handovers executed sequentially. In Rel-16, CHO was designed only for single execution from the source currently serving the UE toward one of the candidate targets. If executed serially, the selected target becomes the new radio access management point of the UE, and subsequently determines, as a “source”, for the next handover of the served UE toward another RAN node, and so on. This nature inherently allows the XnAP descriptions based on the "source" and "target" to be applied successively for subsequent Inter-CU LTM.

From this sense, RAN3 acknowledged the fact that, once we have direct UE-associations in-between C-CUs, any procedure triggered by a C-CU at any point of time can be made to work as effective as from S-CU for initial PCell switch, and our specification efforts and signaling overhead can be greatly reduced as their stage-2/3 descriptions can remain universally applied regardless of how many CUs are involved for LTM:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

So far, what RAN3 has agreed is (a) the need of direct UE-association between C-CUs; (b) when it should be established and via which Xn signalling (i.e. after each Inter-CU LTM cell switch, using LTM Configuration Update – the first message expected in-between). This is a great milestone indeed, but we still have leftover issues of

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

### (1) How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

First, we would like to point out that this issue doesn’t exist when only one C-CU (other than S-CU) is involved from the beginning (i.e. a total of two CUs). In this case, there is already a direct UE-association between two CUs (S-CU <> C-CU) over Xn, and it can be re-used for subsequent LTM. Only when the number of CUs involved exceeds two (i.e. more than one C-CU), this issue becomes evident for establishing direct UE-associations in-between C-CUs.

Having that in mind, once an initial Inter-CU PCell switch happens from S-CU toward a candidate PCell in one C-CU, we first observe that this C-CU needs to know the existence of another CU(s) involved for the same UE’s LTM. Otherwise, establishing direct UE-associations (or even first seeking for Xn connectivity if not connected beforehand) would not be a question.

Overall, we think that leveraging the LTM Configuration ID Mapping List IE is the best way. This IE is used for assigning the unique identifiers for each candidate cell configuration, allocated and managed by S-CU (RAN3-125bis already agreed: The LTM Configuration IDs are allocated by the source CU). Moreover, similar to the (common) CSI resource configurations of candidate cells, this IE shall also be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations. Currently, this IE is captured at bare minimum in the current BL CR [7], i.e. a list of IDs (1 to 8) for each candidate cell. But if we can extend this IE to also include the information of which candidate cell belongs to which CU, any recipient CU of this IE would be able to know which CUs are involved for the UE’s LTM and which candidate cells are currently planned.

In fact, making involved entities visible with each other is nothing new. In Rel-18 of (Intra-CU) Inter-DU LTM, each involved DU was also visible to other DUs based on early UL sync information handling. The preamble indexes for early UL synchronization are allocated and shared per DU basis, and the early UL sync resource request from CU can indicate a list of involved DU IDs [9]. Though this aspect was not addressed by F1AP LTM Configuration ID Mapping List IE, at least we can say that the similar handling of letting each involved DU know the existence of other involved DU(s) was already in place in Rel-18.

Moreover, one advantage of this approach observed is that timing of information update (i.e. “when”) doesn’t need to be worried about. As mentioned above, the LTM Configuration ID Mapping List IE shall anyway be maintained up-to-date and synched to all the involved entities throughout LTM operations of a UE. Whenever there is a need for update, it will be updated in NW side and UE side (though synch to the UE may not be always required case-by-case).

### (2) How to identify the same UE?

A reference to a UE context should be included in the LTM Configuration Update message (first message) triggered from the new serving C-CU so that the recipient C-CU can associate with the same UE.

In the past, identifying the same UE context already established in one entity upon request from other entity was addressed in RAN3 based on “UE XnAP ID” [10].

- During Inter-MN HO without SN change, the same SN identifies the already established UE context when the target MN decides to add the same SN, by the “SN UE XnAP ID” included in SN ADD REQ, which is forwarded from the source MN via HO REQ.

- Another practice was at Rel-17 CHO with NR-DC, where the same target SN identifies the same UE SN addition request from different target MN(s), based on the pair of the source MN node ID and the MN UE XnAP ID included in each SN ADD REQ, which is also forwarded from the source MN via HO REQ.

Two slightly different approaches were taken for each practice:

- In the first case, an entity requesting for the same UE identification directly provided the UE XnAP ID assigned by the recipient before.

- For the second case, the requesting entities provide the common UE XnAP ID for the recipient to identify that multiple requests (from different entities) are indeed for the same UE, where the common ID was selected by the identifier assigned at the source side.

When it comes to having direct UE-associations in-between C-CUs, the first approach can be applied as:

- Each C-CU would have already established a UE-association with S-CU and the corresponding “Target UE XnAP ID” has been assigned for the interaction with S-CU (during the first round-trip HO preparation). When a new serving C-CU initiating to establish direct UE-association with other C-CU, the requesting C-CU can use the “Target UE XnAP ID” (assigned by the receiving C-CU for interaction with S-CU), to be used as a reference to the UE context established in the receiving C-CU.

- If we extend this approach successively for subsequent LTM, the pre-requisite will be for a new serving C-CU to know the “Target UE XnAP IDs” assigned by other C-CUs (for interaction with the old serving CU), before it initiates direct UE-association with other C-CU(s).

The second approach can be worked out by:

- A new serving C-CU who initiating to establish direct UE-association with other C-CU uses the “Source UE XnAP ID” (assigned by S-CU toward each receiving C-CU during initial preparation).

- If we extend this approach successively for subsequent LTM, then the pre-requisite will be for the new serving CU to know the “Source UE XnAP IDs” assigned by the old serving CU toward other C-CUs. In addition, this may require to include the old serving CU’s node ID as well to refer to the right UE context, because from the receiving CU point of view, the reference to a UE context is the “Source UE XnAP ID” assigned by the other node (i.e. by the old serving CU).

Or, both approaches can be mixed up to provide both UE XnAP IDs assigned by the peer CUs.

But, regardless of which UE XnAP IDs to take as a reference, the reference info shall be provisioned to a new serving CU (after each Inter-CU LTM cell switch), so that it can use those references to make the receiving CU to identify the same UE when initiating direct UE-association in-between.

For that, we think that Cell Switch Notification is a natural choice as it is triggered from the old serving CU to the new serving CU whenever Inter-CU LTM cell switch happens.

However, this alone is not enough due to a failure/re-attempt scenario for which the UE may fail cell switch to the indicated CU and access to another CU. In this case, the old serving CU receives HO SUCCESS from a CU that it didn’t send the Cell Switch Notification to. Moreover, Cell Switch Notification would not be applicable for Conditional LTM, for which the old serving CU knows the Inter-CU LTM execution only when it receives HO SUCCESS from the new serving CU. For provision of UE context reference to this unintended new serving CU, the LTM Configuration Update shall also be able to carry the references.

Considering above analysis and observations, we propose the followings:

Proposal 11: For direct UE-associations between C-CUs, the leftover issues are on

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 12: For (2) to work, after each Inter-CU LTM cell switch, the old serving CU shall provision to the new serving CU, UE context “reference” to be included in the subsequent LTM Configuration Update procedure (first message) toward other CU for establishing direct UE-association in-between.

Proposal 13: For provisioning “reference” from the old serving CU to the new serving CU,

- Cell Switch Notification to include such “reference”, for normal NW-triggered LTM.

- LTM Configuration Update to include such “reference”, for failure/re-attempt and Conditional LTM.

Proposal 14: Considering that the LTM Configuration ID Mapping List IE shall be maintained up-to-date (whenever update happens) and synced to all the involved CUs for correct LTM operations,

- Address both (1) and (2) by including the IE into Cell Switch Notification (Note that this LTM Configuration ID Mapping List IE is already included inside the LTM Configuration Update message).

- Enhance the IE to include

- The information of which candidate cell belongs to which CU (for (1)).

- UE context “reference” associated with each candidate cell, which can be used by the new serving CU for establishing direct UE-association with other CUs (for (2)).

Proposal 15: FFS on which UE XnAP IDs to take as reference, either “Source UE XnAP ID” assigned by the old serving CU, or “Target UE XnAP ID” assigned for the old serving CU, or both.

## Semi-Persistent CSI-RS support

RAN3 received an LS from RAN1 for the support of semi-persistent (SP) CSI-RS resource for LTM candidate cells [11]. RAN3 agreed to help RAN1 and replied it is feasible by [12].

RAN3 also discussed solutions on how to support the signalling coordination between CUs [4] but no meaningful progress was achieved yet:

Issue1: Solution to support semi-persistent CSI-RS: In order to support semi-persistent CSI-RS measurement in LTM, the source node and the target node shall inform each other when the candidate cell active the semi-persistent CSI-RS, which will incur RAN3 signalling design. In [7261, HW], it provides three options to support SP CSI-RS measurement in intra-CU inter-DU case: As illustrated in Figure 1, after receiving SSB measurement report from the UE, the source gNB-DU determines to start semi-persistent CSI-RS measurement for the candidate cell. Before sending the SP CSI-RS measurement indication to the UE, the source gNB-DU has three options to trigger the candidate gNB-DU to transmit the CSI-RS correspondingly. - Option 1: The source gNB-DU determines the CSI-RS resources that to be transmitted, and sends the CSI-RS resource (set) IDs to the candidate gNB-DU via the gNB-CU to request the candidate gNB-DU to transmit the CSI-RS. In this option, the source gNB-DU needs to know the SSB and CSI-RS association of the candidate cell, to determine the suitable CSI-RS resources. - Option 2: The source gNB-DU forwards the SSB measurement report to the candidate gNB-DU and the candidate gNB-DU determines the CSI-RS resources. The candidate gNB-DU sends that information back to the source gNB-DU. - Option 3: The gNB-CU decides to trigger semi-persistent CSI-RS measurement and sends the selected CSI-RS resource (set) IDs to the candidate gNB-DU and the source gNB-DU based on L3 measurement results. The candidate gNB-DU starts to transmit CSI-RS following the indication of the gNB-CU, and the source gNB-DU triggers the UE measurement. In this section, we can discuss the basic signaling framework to support SP CSI-RS coordination for the intra-CU inter-DU case first. The message to be used and the detailed IE design can be further discussed in the next meeting. Question: Which node makes the decision on SP CSI-RS transmission: source DU, candidate DU or CU for the intra-CU inter-DU case? Moderator summary: LG: It is a common understanding that the semi-persistent and periodic CSI-RS are per-UE specific. After we reach a consensus on the intra-CU case, we can further discuss the solution to support SP CSI-RS for inter-CU case. - Option 1: source node-initiated coordination - Option 2: candidate node -initiated coordination Question: Which node makes the decision on SP CSI-RS transmission for inter-CU case: source node or candidate node?

The nature of SP CSI-RS is to reduce constant CSI-RS transmission and reporting overhead over air interface, for which is activated / de-activated by MAC CE. Given CSI-RS is allocated per UE-specific, this requires candidate cells prepare those CSI-RS resources for the UE and pre-configured to the UE. After pre-configured, which CSI-RS resources to start to measure and to stop can then be controlled by MAC CE.

Moreover, the decision whether to start CSI-RS measurement for candidate cells and on which CSI-RS resources, should be based on measurement results from the UE, which is provided to the DU controlling the UE’s serving cell (if measurements via L1), or its CU (if measurements via L3).

Considering these, focusing on L1-based LTM, it is logical that the serving DU (who receives measurements from the UE and can control the UE via MAC CE) should make decisions on SP CSI-RS activation/de-activation and on which CSI-RS resources to measure. Once determined to activate, it should request the candidate cell to start the corresponding CSI-RS transmissions. Once acknowledged, the serving DU can activate the UE via MAC CE. For de-activation, the similar process can be considered where the serving DU decided to deactivate CSI-RS measurement for a specific candidate cell first de-activates the UE via MAC CE and inform the candidate cell to stop those CSI-RS transmissions. There seems no ground-breaking reason to forward the measurement results to a candidate cell (as in Option 2) to decide which CSI-RS resources to activate or de-activate.

The spirit of Option 3 where the decision maker is CU can be also discussed, but we think it should be a low priority given that the main focus of Rel-19 is based upon L1-based LTM for Inter-CU and adding CSI-RS resources and measurements in addition to the existing SSB-based.

Proposal 16: For SP CSI-RS coordination, the serving DU decides activation / de-activation of CSI-RS measurements for each candidate cell and which CSI-RS resources to measure.

Proposal 17: For activation, the serving DU asks the candidate cell’s DU to start the corresponding CSI-RS transmissions and activates the UE once acknowledged from the candidate cell’s DU.

Proposal 18: For de-activation, the serving DU de-activates the UE via MAC CE and informs the candidate cell’s DU to stop the corresponding CSI-RS transmissions for the UE.

Proposal 19: The decision by CU for SP CSI-RS activation/de-activation should be taken as a low priority, given that the main focus of Rel-19 is L1-based LTM for Inter-CU and adding CSI-RS resources and measurements in addition to the existing SSB-based.

Having these, we think we should also not preclude the possibility that a candidate cell (who requested by the serving cell to start or stop CSI-RS transmissions for the UE) may not follow as exactly the serving cell wants. The life of a DU is unexpected. After started, it may be stopped unexpectedly (or may have to be stopped for various reasons) before requested by the serving cell. For future-proof design, we believe it is important to specify a mechanism that a candidate cell who inevitably stopped the requested CSI-RS transmissions for the UE notifies the serving cell, so that the serving cell can subsequently de-activate the UE’s CSI-RS measurement for that candidate cell via MAC CE.

Proposal 20: For SP CSI-RS, RAN3 also considers a mechanism for a candidate cell’s DU to notify the serving DU if the CSI-RS transmissions requested from the serving DU are (or has to be) stopped.

## Others

Regarding TA handling, there were several additions worked out in Rel-18 that CU can share with a DU: (a) a known TA value (i.e. TA value = 0) between the current serving cell and a candidate cell; and (b) whether UE-based TA measurement can be applied (and configured to the UE) in-between the serving and candidate cells. The former was an optimization to cater a special deployment (co-located cells), while the latter was one of early UL synchronization mechanism specified that is utilized only when DU knows so.

While those TA handling perfectly fits within a single CU scope (as specified in Rel-18), a question is whether they can be applicable between cells under different CUs.

It is hard to imagine that either co-located cells are managed under different CUs, or the 260ns TAE (timing alignment error) requirement could be satisfied between cells under different CUs [13].

Before extending, the usage of those TA handlings across CUs shall be validated.

Proposal 21: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Late data forwarding still needs a partner

Proposal 1: Honor the previous RAN3 agreements for Inter-CU LTM early data forwarding:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Proposal 2: For Inter-CU LTM, allow normal (late) data forwarding to be initiated right after cell switch command to a UE without waiting for HO SUCCESS (when exactly is up to implementation as usual).

## Still incomplete E1AP BL CR

Proposal 3: Extend the usage of the CHO Initiation IE in the E1AP BEARER CONTEXT SETUP REQUEST message to cater for LTM as well.

If the CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns conditional handover or conditional PSCell change or conditional PSCell addition or subsequent CPAC or LTM and act as specified in TS 38.401 [2].

## Inter-CU LTM preparation details

Proposal 4: To ease XnAP signalling management and error handling between CUs throughout LTM operations, during HO preparation, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and a single UE-associated signaling connection is established/maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

Proposal 5: The second round-trip (“LTM Configuration Update”) of Steps 6-7 should be marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM. Even for “CSI-RS”-based resources (whose resource configuration is UE-specific), the second round-trip can be skipped if S-CU chooses to address the subsequent LTM at a later stage.

Proposal 6: Design the stage-3 messages of the second round-trip (“LTM Configuration Update”) procedure to cater for the update of multiple candidate cells configurations simultaneously (i.e. not per-cell basis as in the existing HO procedure).

Proposal 7: Fix the second round-trip (“LTM Configuration Update”), to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during preparation phase, and also expected to be initiated from CU who is currently serving the UE (i.e. “source”) during subsequent LTM.

Proposal 8: For LTM Cell ID in XnAP, use 9.2.3.5 Target Cell Global ID (instead of 9.2.2.7 NR CGI).

Proposal 9: During the first round-trip (the existing HO procedure), the below information should be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - (O) LTM initiation indication (already by LTM Information Request IE, but requires the underlying (M) “LTM Information Setup” as in F1AP to avoid all optionals?)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but “request” is missing)

  - (O) SSB / CSI-RS resource configuration of candidate cells, if available (already by LTM Information Request IE > CSI Resource Configuration IE)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE)

  - (O) A request for CSI-RS resource configuration (already by LTM Information Request IE > Request for CSI-RS Configuration IE)

  - (O) Early Sync Information Request (already by Early Sync Information Request IE, but needs more discussion).

- (per-cell) HO REQ ACK message to include:

  - (O) LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, but the semantic of the mandatory existing container in HO REQ RESP shall be updated to be ignored if included)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE)

  - (O) CSI-RS resource configuration allocated from the admitted cell (already by LTM Information Response IE > CSI-RS Configuration IE)

  - (O) Early Sync Information, which contains DL/UL early sync info allocated from the admitted candidate cell (already by Early Sync Information Response IE, but needs more discussion)

  - (O) Data Forwarding proposal to be used for subsequent LTM (not implemented yet)

Proposal 10: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information should be exchanged:

- (per-node) LTM Configuration Update REQ message to include:

  - (M) Target Cell Global ID (shall be removed / re-designed for multi-cell handling)

  - (O) Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell (already by the respective Early Sync Information / SUL IEs, but needs more discussions)

  - No IE for requesting the updated Early Sync Information List

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but “request” is missing)

  - (O) SSB / CSI-RS resource configuration of all the candidate cells admitted by other CU(s) (already by LTM Information Request IE > CSI Resource Configuration IE –repeated message-level CSI-RS Resource Configuration shall be removed)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE)

  - (O) A request for CSI-RS resource configuration (already by LTM Information Request IE > Request for CSI-RS Configuration IE)

  - (O) Data Forwarding TNLs assigned by other involved CU(s) (not implemented yet)

  - (O) Data Forwarding proposal from the requesting CU (not implemented yet)

- (per-node) LTM Configuration Update ACK message to include:

  - (M) Target Cell Global ID (shall be removed / re-designed for multi-cell handling)

  - No IE for providing the updated Early Sync Information List

  - (O) Updated LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, FFS whether the message-level CSI-RS Report Configuration shall be removed if included into the container)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE)

  - (O) Updated CSI-RS resource configuration (already by LTM Information Response IE > CSI-RS Configuration IE)

  - (O) Data Forwarding TNLs assigned by the requested CU (not implemented yet)

## UE-associated Xn signalling connections between C-CUs

Proposal 11: For direct UE-associations between C-CUs, the leftover issues are on

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 12: For (2) to work, after each Inter-CU LTM cell switch, the old serving CU shall provision to the new serving CU, UE context “reference” to be included in the subsequent LTM Configuration Update procedure (first message) toward other CU for establishing direct UE-association in-between.

Proposal 13: For provisioning “reference” from the old serving CU to the new serving CU,

- Cell Switch Notification to include such “reference”, for normal NW-triggered LTM.

- LTM Configuration Update to include such “reference”, for failure/re-attempt and Conditional LTM.

Proposal 14: Considering that the LTM Configuration ID Mapping List IE shall be maintained up-to-date (whenever update happens) and synced to all the involved CUs for correct LTM operations,

- Address both (1) and (2) by including the IE into Cell Switch Notification (Note that this LTM Configuration ID Mapping List IE is already included inside the LTM Configuration Update message).

- Enhance the IE to include

- The information of which candidate cell belongs to which CU (for (1)).

- UE context “reference” associated with each candidate cell, which can be used by the new serving CU for establishing direct UE-association with other CUs (for (2)).

Proposal 15: FFS on which UE XnAP IDs to take as reference, either “Source UE XnAP ID” assigned by the old serving CU, or “Target UE XnAP ID” assigned for the old serving CU, or both.

## Semi-Persistent CSI-RS support

Proposal 16: For SP CSI-RS coordination, the serving DU decides activation / de-activation of CSI-RS measurements for each candidate cell and which CSI-RS resources to measure.

Proposal 17: For activation, the serving DU asks the candidate cell’s DU to start the corresponding CSI-RS transmissions and activates the UE once acknowledged from the candidate cell’s DU.

Proposal 18: For de-activation, the serving DU de-activates the UE via MAC CE and informs the candidate cell’s DU to stop the corresponding CSI-RS transmissions for the UE.

Proposal 19: The decision by CU for SP CSI-RS activation/de-activation should be taken as a low priority, given that the main focus of Rel-19 is L1-based LTM for Inter-CU and adding CSI-RS resources and measurements in addition to the existing SSB-based.

Proposal 20: For SP CSI-RS, RAN3 also considers a mechanism for a candidate cell’s DU to notify the serving DU if the CSI-RS transmissions requested from the serving DU are (or has to be) stopped.

## Others

Proposal 21: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

The corresponding TP for Proposal 3 for E1AP BL CR [2] is also provided in Annex C.

# Reference

[1] TS 38.300 NR; NR and NG-RAN Overall description; Stage-2, v18.4.0

[2] R3-250048, “(BL CR to 37.483) Introducing Rel-19 Mobility enhancement”, LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC

[3] TS 37.483, NG-RAN; E1 Application Protocol (E1AP), v18.3.0

[4] R3-247824, “Summary of offline discussion on MobilityEnh”, China Telecom (moderator)

[5] R3-247483, “Comprehensive analysis of inter-CU LTM”, Ericsson

[6] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[7] R3-250051, “(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics

[8] R3-247031, “(BL CR to 38.300) Support for Inter-CU LTM”, Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung

[9] TS 38.473, NG-RAN; F1 Application Protocol (F1AP), v18.4.0

[10] TS 37.340, E-UTRA and NR; Multi-connectivity; Overall Description; Stage 2, v18.4.0

[11] R3-247006/ R1-2409283 LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

[12] R3-247911 Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

[13] R3-235019/R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

# Annex A (RAN3 agreement for Inter-CU LTM)

## RAN3-126 (Orlando, November 2024)

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

## RAN3-125bis (Hefei, October 2024)

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## RAN3-125 (Maastricht, August 2024)

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

## RAN3-124 (Fukuoka, May 2024)

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## RAN3-123bis (Changsha, April 2024)

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

# Annex B (RAN2 agreement for Inter-CU LTM)

## RAN2-128 (Orlando, November 2024)

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

## RAN2-127bis (Hefei, October 2024)

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Agreements related to Inter-SN LTM

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

## RAN2-127 (Maastricht, August 2024)

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

## RAN2-126 (Fukuoka, May 2024)

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

## RAN2-125bis (Changsha, April 2024)

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

# Annex C (TP for E1AP BL CR [2])

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.1	Bearer Context Setup

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.1.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the DAPS Request Information IE is included for a DRB to be setup in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns a DAPS handover for that DRB and, if admitted, act as specified in TS 38.300 [4].

If the CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns conditional handover or conditional PSCell change or conditional PSCell addition or subsequent CPAC or LTM and act as specified in TS 38.401 [2].

If the MCG Offered GBR QoS Flow Information IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account when two cell groups are served by the gNB-CU-UP.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////