# 3GPP TSG-RAN WG3 Meeting #127	R3-250716

# Athens, GR, February 17th – 21st, 2025

# Agenda Item:	8.1

# Source:	ZTE Corporation

# Title:	Further discussion on the available data rate

# Document for:	Discussions & Decision

- Introduction

SA2 sent LS S2-2407351[1] to RAN3 and asked RAN3 on whether RAN3 is flexible to support the available data rate for GBR QoS Flow via UP. Based on the RAN3 feedback R3-244844[2] sent in RAN3#125, SA2 further clarify the definition for the available data rate in their LS S2-2412720[3]. This paper is used to further explain our view based on the received SA2 LS.

# 2 Discussion

In the received reply LS[3], SA2 firstly confirmed our understanding on the in-band signalling of the periodicity and agreed to exclude the in-band signalling of the periodicity in Rel-19. RAN3 may note SA2 decision and do nothing.

Observation 1: SA2 decision on in-band signalling of the periodicity can be noted by RAN3.

For another question in SA2 reply LS on the available data rate, SA2 explained its understanding in detail. More specifically, the following content can be found in the reply LS:

==== content in SA2 reply LS ====

Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR).

Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

==== content in SA2 reply LS ====

In the reply LS, SA2 not only clarified the definition of available data rate, but also further explain  how does this information be used by NW. It further pointed out that the usage logic of available data rate information is similar to the existing description in the TS 23.501. Hence, from our point of view, RAN3 may support transmitting the available data rate via UP, as required by SA2. A CR has also be provided for our side  to further show our view on how to make such enhancement. Detail can be further discussed by RAN3 when TU is allowed in XR topic.

Proposal 1: RAN3 confirms SA2 clarification for the available data rate.

Proposal 2:  Introduce available data rate information in UP header. Detail enhancement will be discussed when TU is available.

In addition, RAN3 may also need to reply SA2 our decision and next steps(e.g. how to enhance will be further checked.). A draft reply LS(R3-250715) is prepared from our side and can be considered as baseline.

Proposal 3: RAN3 sends reply LS to SA2 and considers draft reply LS (R3-250715) as baseline.

# 3 Conclusion

Based on above analysis, we provide the following proposals.

Proposal 1: RAN3 confirms SA2 clarification for the available data rate.

Proposal 2:  Introduce available data rate information in UP header. Detail enhancement will be discussed when TU is available.

Proposal 3: RAN3 sends reply LS to SA2 and considers draft reply LS (R3-250715) as baseline.