3GPP TSG-RAN WG3 Meeting #127	R3-250728

Athens, Greece, 17th- 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1431 | rev | 1 | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Xn Setup message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI15 |  |  |  |  |  | Date: |  |  | 2025-2-4 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In LTE, Duplicated X2 setup request user case has been covered by the specification. As excepted below, when receiving a duplicated X2 setup request, the receiving node will erase any existing application level configuration data and replaces it by the one received. Which means the both two nodes align the XnAP resource status when the duplicated X2 setup request happen. While for NR, due to introducing network sharing scenario, multiple Xn setup requests are allowed for a Xn-C interface. It is because different Xn setup request message is triggered to setup different Xn-C interface instance. While the current specification does not cover the case for duplicated Xn setup request for the same instance. Based on implementation, the receiving node may response Xn setup failure for the request. While in current specification, receiving Xn setup failure means the Xn interface is not available, then the sending node shall regard the already established Xn interface is not operational and therefore all established Xn-c interfaces in different instance pair shall be removed. The issue described above should be corrected . In order to solve the issue, when receiving node received a duplicated Xn setup request for a Xn-C interface instance, the receiving node will erase any existing application level configuration data and replaces it by the one received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To update NOTE 1 of general part of the procedure text to solve the issue. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the abnormal cases of Xn setup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to unexpected consequences when the receiving node makes the interface not operational. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: Update based on discussion of RAN3#126. |  |  |  |  |  |  |  |  |

Change Begins

8.4.1	Xn Setup

8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE 1:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received per Xn-C interface instance.

Change Ends