3GPP TSG-RAN WG3 Meeting #127	R3-250734

Athens, Greece, 17 – 21 February, 2025

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

MRO for LTM In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI. For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM. For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell. Whether network based solution is needed? What’s the UE context to be stored in RAN side? UHI and ping-pong: Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)? CU sends UHI to DUs?

At the RAN3#126 meeting, we have been made above agreements and introduced stage-2 definition of LTM detection mechanism.

For ping-pong issue in intra-NR LTM, since CU determines the LTM configuration and sends to target candidate gNB-DU(s), CU should be responsible for avoiding LTM cell switch ping-pong with all the UHI information when choosing the candidate LTM cells. When inappropriate cell switch triggering made by DU, DU should be responsible for the issue. CU only sends the UHI to DUs when inappropriate cell switch triggering made by DU.

Proposal 1: CU should be responsible for avoiding LTM cell switch ping-pong.

Proposal 2: CU only sends the UHI to DUs when inappropriate cell switch triggering made by DU.

Due to time constraints, we didn't discuss RACH-less and RACH-based access related topic at the last meeting, following the agreement from RAN3#124 meeting stated below.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

Regarding enhancements related to the information of RACH-less access and RACH-based access for LTM, it will be helpful for network to distinguish whether UE perform RACH-less LTM or RACH-based LTM for further optimization which can be included in the RLF report and SHR report.

Proposal 3: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

For legacy mobility, timer-based triggering conditions T304, T310 and T312 are introduced, which can be further reused in the LTM procedure in a straightforward way.

Proposal 4: The T304, T310 and T312 related triggering conditions can be reused for LTM SHR.

2.2	CHO with candidate SCGs

In last RAN3#126 meeting, we have the following agreements.

MRO for CHO with Candidate SCG(s):

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

As we sent our agreements and understandings to RAN2 in the last meeting, RAN3 can further discuss the topic after the feedback and confirmation from RAN2.

Regarding which message is used from the last serving MN to the candidate MN(s), as existing massages are designed for other purpose with some unnecessary mandatory IEs, new message seems more straightforward in this case when SCGFailureInformation to be forwarded between the source MN and the target MN.

Proposal 5: New message can be introduced when SCGFailureInformation to be forwarded between the source MN and the target MN.

2.3	Subsequent CPAC

A conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, or a SCG release, based on pre-configured subsequent CPAC configuration of candidate PSCell(s). UE would keep the subsequent CPAC configuration after a PSCell change, and if the execution condition of other one candidate PSCell is satisfied, UE may apply the stored configuration and synchronises to the candidate PSCell.

In that case, subsequent CPAC failure may occur at the first configured SN modification, as well as subsequent PSCell changes scenario. When subsequent CPAC fails, SCG FAILURE INFORMATION REPORT can be further enhanced to record the corresponding configurations for root cause analysis. Meanwhile, RAN3 needs to discuss how to trigger the forwarding of failure report to the configured node with the potential impacts on UE recording the mobility history information.

And regarding the differentiation between initial and following CPC, we don’t see the strong intention to separate the two case.

Proposal 6: There is no need to differentiate between initial and following CPC.

For subsequent CPAC, network cannot log the UHI since no more SN adding procedure during the mobility phase based on the existing SCG UHI mechanism. It is more straightforward for MN to adding all the visited SN while informing the source SN about the outcome of mobility events for SN initiated S-CPAC.

Proposal 7: SCG UHI mechanism can be enhanced for subsequent CPAC.

Proposal 8: MN to record the SCG UHI  for subsequent CPAC and send it back to source SN.

3	Conclusion

Proposal 1: CU should be responsible for avoiding LTM cell switch ping-pong.

Proposal 2: CU only sends the UHI to DUs when inappropriate cell switch triggering made by DU.

Proposal 3: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

Proposal 4: The T304, T310 and T312 related triggering conditions can be reused for LTM SHR.

Proposal 5: New message can be introduced when SCGFailureInformation to be forwarded between the source MN and the target MN.

Proposal 6: There is no need to differentiate between initial and following CPC.

Proposal 7: SCG UHI mechanism can be enhanced for subsequent CPAC.

Proposal 8: MN to record the SCG UHI  for subsequent CPAC and send it back to source SN.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023