3GPP TSG-RAN WG3 Meeting #127	R3-250745

Athens, Greece, 17-21 Feb, 2025

Title:	Discussion on Ambient IoT RAN Architecture

Agenda Item:	16.2

Source:	China Unicom

Document for:	Discussion

- Introduction

During RAN#106 meeting, a WID "Solutions for Ambient IoT (Internet of Things) in NR" [1] was approved. And the task for RAN3 can be found as the following:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we discuss the RAN architecture aspects for A-IoT.

- Discussion

2.1 Direct/indirect path deployment

In SA2 TR [2], the describtion about AIOTF is as the following can be found:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Send requests that trigger BS Reader(s) (via AIoT RAN) to perform AIoT operations, either directly or via an AMF.

Thus, for RAN aspect, both direct and indirect path deployments shoud be supported. For direct path deployment, gNB connects to AIOTF via NG interface directly. For indirect path deployment, gNB connects with AIOTF indirectly via AMF. To support the two above deployments, one option is to design different procedures/messages separately and the other option is to introduce one unified procedure/message between gNB and CN.  Clearly, the latter approach would streamline RAN3 discussions and eliminate redundant efforts and unnecessary complexity.

Proposal 1: Adopt a unified approach (the same procedure and message) between the gNB and CN, regardless of  direct or indirect path deployment.

2.2 Whether “a new protocol layer over NGAP” for AIoT

As discussed during SI, for AIoT “XXAP” transmission there were two options, one is including AIoTF information in NGAP (Opiton 1) and the other is by carrying a new protocol layer over NGAP (Opiton 2). These two options should be discussed and down selected at this meeting.

For both Option 1 and Option 2, unified design can be achieved for direct or indirect path deployment. And for Option 2, there are some disadvantages:

- Increased complexity: developing and integrating new protocol layers adds complexity to the system, requiring more efforts for standardization

- Potential delays: more time required to design and deploy new protocols which could delay the deployment of A-IoT solutions.

Compared two options, there are some disadvantages for Option 2, we prefered Option 1 to support the RAN CN interface.

Proposal 2: Introduce AIOTF related information in NGAP layer.

- Conclusion

In this paper, the following proposals are made:

Proposal 1: Adopt a unified approach (the same procedure and message) between the gNB and CN, regardless of  direct or indirect path deployment.

Proposal 2: Introduce AIOTF related information in NGAP layer.

- Reference

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19) V1.2.1 (2024-11)