3GPP TSG-RAN WG3 Meeting #127	R3-250861

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of full eNB as regnerative payload and store&forward operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung, CMCC, CATT, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, Xiaomi, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the WID, in Rel-19 IoT NTN, the Store and Forward operation and full eNB as regenerative payload shall be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add stage 2 description of the Store and Forward operation, and full eNB as regenerative payload. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 description to meet the requirements in the WID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 4.12; 19.2.1.0; 19.2.2.7.x; 23.21.5.1; 23.21.x; Annex P |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.410 CR ... TS 36.413 CR 1963 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, e.g. BRID or DAA.

>>Unchanged part skipped<<

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Feeder link: wireless transport link between the NTN Gateway and the NTN payload.

>>Unchanged part skipped<<

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Inter-satellite link: wireless transport link between two regenerative NTN payloads.

>>Unchanged part skipped<<

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. Considering E-UTRAN functions, the NTN payload may be a TNL node (transparent payload), or an eNB (regeneative payload).

Editor’s Note: Pending RAN2 update, if any.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figures 4.12-1 and  4.12-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 4.12.1, the payload is transparent; in Figure 4.12-X, the payload is regenerative and hosts an eNB.

Figure 4.12-1: Overall illustration of an NTN with transparent payload

Figure 4.12-X: Overall illustration of an NTN with regenerative payload hosting an eNB

NOTE:	Figures 4.12-1 and Figure 4.12-X illustrate an NTN; RAN4 aspects are out of scope.

The transparent NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface (via the service link), S1 and X2 intefaces. The following connectivity is supported by the NTN payload:

-	A NTN Gateway may serve multiple transparent or regenerative NTN payloads;

-	A transparent or regenerative NTN payload may be served by multiple eNBs;

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads.

NOTE:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Store and Forward satellite operation is supported as described in TS 23.401 [17].

Editor’s Note: Pending RAN2 update, if any.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

19.2.1.0	General

The S1 interface provides the following functions:

-	E-RAB Service Management function:

-	Setup, Modify, Release.

-	Mobility Functions for UEs in ECM-CONNECTED:

-	Intra-LTE Handover;

-	Inter-3GPP-RAT Handover.

-	S1 Paging function:

-	NAS Signalling Transport function;

-	LPPa Signalling Transport function;

-	S1-interface management functions:

-	Error indication;

-	Reset;

-    Removal.

-	Network Sharing Function;

-	Roaming and Access Restriction Support function;

-	NAS Node Selection Function;

-	Initial Context Setup Function;

-	UE Context Modification Function;

-	UE Context Resume Function;

-	MME Load balancing Function;

-	Location Reporting Function;

-	PWS (which includes ETWS and CMAS) Message Transmission Function;

-	Overload function;

-	RAN Information Management Function;

-	Configuration Transfer Function;

-	S1 CDMA2000 Tunnelling function;

-	Trace function;

-	UE Radio Capability Match;

-	Retrieve UE Information Function;

-	UE Information Transfer Function;

-	Report of Secondary RAT data volumes Function;

-	UE Radio Capability ID Mapping Function.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

19.2.2.7.x	S1 Removal procedure

The purpose of the S1 Removal procedure is used to remove the interface instance between the eNB and the MME in a controlled manner. The S1 Removal procedure is triggered by the eNB.

Figure 19.2.2.7.x-1: S1 Removal procedure

-	The eNB initiates the S1 Removal procedure by sending the S1 REMOVAL REQUEST message to the MME.

-	In the successful case the MME responds with the S1 REMOVAL RESPONSE message. After receiving the NG REMOVAL RESPONSE message, the eNB shall initiate removal of the TNL association towards the MME and may remove all resources associated with that interface instance. The MME may then remove all resources associated with the eNB.

-	If the MME cannot accept to remove the interface instance with eNB, it shall respond with a S1 REMOVAL FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

23.21.5	Switchover

23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

23.21.x	S1 Removal

For the case of regenerative payload, when disconnecting from a MME is required due to e.g. the satellite movement in a LEO constellation, the eNB on board the satellite may request the removal of the S1 interface by initiating the S1 Removal procedure towards the MME.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

Annex P (informative):
Example implementation of Non-Terrestrial Networks

The following figure illustrates an example implementation of a Non-Terrestrial Network for transparent NTN payload:

Figure P-1: NTN based E-UTRAN with transparent NTN payload

The eNB depicted in Figure P-1 may be subdivided into non-NTN infrastructure eNB functions and the NTN Service Link provisioning System. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning System and the NTN Control function. The NTN Service Link provisioning System may consist of one or more NTN payloads and NTN Gateways.

The NTN payload is embarked on a spaceborne (or airborne) vehicle, providing a structure, power, commanding, telemetry, attitude control for the satellite (resp. high altitude platform station) and possibly an appropriate thermal environment and radiation shielding.

The NTN Service Link provisioning System maps the Uu radio protocol over radio resources of the NTN infrastructure (e.g. beams, channels, Tx power).

The NTN control function controls the spaceborne vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure eNB functions of the eNB.

Provision of NTN control data to the eNB is out of 3GPP scope.

NOTE:	The transport of Uu protocol between the NTN Service Link provisioning system and the non-NTN infrastructure eNB functions is out of 3GPP scope.

At least the following NTN related parameters are expected to be provided by O&M to the eNB for its operation.

a) Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) mapped to the beam;

-	The Cell's reference location (e.g. cell's center and range).

b) Quasi Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) and time window mapped to a beam;

-	The Cell's/beam's reference location (e.g. cell's center and range);

-	The time window of the successive switchovers (feeder link, service link);

-	The identifier and time window of all serving NTN payloads and NTN-Gateways.

c) Earth moving beams: for each beam provided by a given NTN-payload:

-	The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam's foot print on Earth;

-	Its elevation with regard to NTN-payload;

-	Schedule of successive serving NTN-Gateways/eNBs;

-	Schedule of successive switchovers (feeder link, service link).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>