3GPP TSG RAN WG3 Meeting #127	R3-250913

Athens, Greece, 17 - 21 February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1341 | rev | 5 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of low-power wake-up signal and receiver for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, NR LPWUS is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce LP-WUS Paging Subgrouping (LP-WUSPS) Assistance Information in XnAP PAGING message. Editor’s note: the inclusion of the LP-WUSPS Assistance Information is to be finally confirmed based on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection). According to S2-2412875, the AMF may configure up to 32 different LP-WUS Subgroup IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of LP-WUS in RAN paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.2.5.2, 9.1.1.7, 9.2.3.x, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR ... TS 38.300 (Draft) TS 38.473 CR 1443 TS 38.470 CR 0157 TS 38.410 CR ... TS 38.420 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmission to RAN3#125bis Rev2: resubmission to RAN3#126 Rev3: resubmission to RAN3#127 Rev4: endorsement as BL CR Rev5: cover page updates |  |  |  |  |  |  |  |  |

START OF CHANGES

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

A2X	Aircraft-to-Everything

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

GUAMI	Globally Unique AMF Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LBT	Listen-Before-Talk

LP-WUSPS	Low Power Wake Up Signal with Paging Subgrouping

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

MT-SDT	Mobile Terminated Small Data Transmission

M-NG-RAN node	Master NG-RAN node

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

RANAC	RAN Area Code

RedCap	Reduced Capability

RSN	Redundancy Sequence Number

RSPP	Ranging/SL Positioning Protocol

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SNPN	Stand-alone Non-Public Network

S-CPAC	Subsequent CPAC

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

SDT	Small Data Transmission

TAC	Tracking Area Code

TAI	Tracking Area Identity

UL	Uplink

UPF	User Plane Function

V2X	Vehicle-to-Everything

NEXT CHANGE

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34], and for eDRX or the UE_ID based subgrouping according to TS 38.304 [33]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2.

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE Specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

When available, the NG-RAN node1 shall include the Hashed UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2. If the Hashed UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33] or TS 36.304 [34].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUSPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

Editor’s note: the inclusion of the LP-WUSPS Assistance Information IE is to be finally confirmed based on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection).

NEXT CHANGE

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE Specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.2.3.144a |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.172 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.2.3.195 |  | YES | ignore |
| LP-WUSPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |

NEXT CHANGE

#### 9.2.3.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..31, …) (FFS) |  |

NEXT CHANGE

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

OMITTED TEXT UNCHANGED

ReportingPeriodicityForDataCollection,

NodeAssociatedInfoResult,

SLPositioning-Ranging-Services-Info,

PDUSessionsListToBeReleased-UPError,

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

LPWUSPSassistanceInformation

OMITTED TEXT UNCHANGED

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-LPWUSPSassistanceInformation

OMITTED TEXT UNCHANGED

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue						CRITICALITY reject	TYPE UEIdentityIndexValue								PRESENCE mandatory}|

{ ID id-UERANPagingIdentity							CRITICALITY ignore	TYPE UERANPagingIdentity									PRESENCE mandatory}|

{ ID id-PagingDRX									CRITICALITY ignore	TYPE PagingDRX												PRESENCE mandatory}|

{ ID id-RANPagingArea								CRITICALITY reject	TYPE RANPagingArea											PRESENCE mandatory}|

{ ID id-PagingPriority								CRITICALITY ignore	TYPE PagingPriority											PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging					CRITICALITY ignore	TYPE AssistanceDataForRANPaging						PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging					CRITICALITY ignore	TYPE UERadioCapabilityForPaging						PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue				CRITICALITY ignore	TYPE ExtendedUEIdentityIndexValue						PRESENCE optional }|

{ ID id-EUTRAPagingeDRXInformation					CRITICALITY ignore	TYPE EUTRAPagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX								CRITICALITY ignore	TYPE UESpecificDRX											PRESENCE optional }|

{ ID id-NRPagingeDRXInformation						CRITICALITY ignore	TYPE NRPagingeDRXInformation							PRESENCE optional }|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE		CRITICALITY ignore	TYPE NRPagingeDRXInformationforRRCINACTIVE			PRESENCE optional }|

{ ID id-PagingCause									CRITICALITY ignore	TYPE PagingCause											PRESENCE optional }|

{ ID id-PEIPSassistanceInformation					CRITICALITY ignore	TYPE PEIPSassistanceInformation						PRESENCE optional }|

{ ID id-HashedUEIdentityIndexValue					CRITICALITY ignore	TYPE HashedUEIdentityIndexValue						PRESENCE optional }|

{ ID id-MT-SDT-Information							CRITICALITY	ignore	TYPE MT-SDT-Information									PRESENCE optional }|

{ ID id-NRPagingLongeDRXInformationforRRCINACTIVE	CRITICALITY ignore	TYPE NRPagingLongeDRXInformationforRRCINACTIVE	PRESENCE optional}|

{ ID id-LPWUSPSassistanceInformation	CRITICALITY ignore	TYPE LPWUSPSassistanceInformation	PRESENCE optional},

...

}

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

OMITTED TEXT UNCHANGED

#### -- L

Local-NG-RAN-Node-Identifier ::= CHOICE {

full-I-RNTI-Profile-List					Full-I-RNTI-Profile-List,

short-I-RNTI-Profile-List					Short-I-RNTI-Profile-List,

choice-extension							ProtocolIE-Single-Container { { Local-NG-RAN-Node-Identifier-ExtIEs} }

}

Local-NG-RAN-Node-Identifier-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-Full-and-Short-I-RNTI-Profile-List		CRITICALITY ignore	TYPE Full-and-Short-I-RNTI-Profile-List	PRESENCE mandatory},

...

}

Full-and-Short-I-RNTI-Profile-List::= SEQUENCE {

full-I-RNTI-Profile-List					Full-I-RNTI-Profile-List,

short-I-RNTI-Profile-List					Short-I-RNTI-Profile-List,

iE-Extensions								ProtocolExtensionContainer { { Full-and-Short-I-RNTI-Profile-List-ExtIEs} }	OPTIONAL,

...

}

Full-and-Short-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Full-I-RNTI-Profile-List ::= CHOICE {

full-I-RNTI-Profile-0	BIT STRING (SIZE (21)),

full-I-RNTI-Profile-1	BIT STRING (SIZE (18)),

full-I-RNTI-Profile-2	BIT STRING (SIZE (15)),

full-I-RNTI-Profile-3	BIT STRING (SIZE (12)),

choice-extension		ProtocolIE-Single-Container { { Full-I-RNTI-Profile-List-ExtIEs} }

}

Full-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Short-I-RNTI-Profile-List ::= CHOICE {

short-I-RNTI-Profile-0	BIT STRING (SIZE (8)),

short-I-RNTI-Profile-1	BIT STRING (SIZE (6)),

choice-extension		ProtocolIE-Single-Container { { Short-I-RNTI-Profile-List-ExtIEs} }

}

Short-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedCell-Item ::= CHOICE {

nG-RAN-Cell						LastVisitedNGRANCellInformation,

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

choice-extension				ProtocolIE-Single-Container { { LastVisitedCell-Item-ExtIEs} }

}

LastVisitedCell-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation	::= OCTET STRING

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LastVisitedPSCellInformation	::= OCTET STRING

LastVisitedPSCellList	::= SEQUENCE (SIZE(1..maxnoofPSCellsPerSN)) OF LastVisitedPSCellList-Item

LastVisitedPSCellList-Item	::= SEQUENCE {

lastVisitedPSCellInformation		LastVisitedPSCellInformation,

iE-Extensions		ProtocolExtensionContainer { { LastVisitedPSCellList-Item-ExtIEs} } OPTIONAL,

...

}

LastVisitedPSCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SCGUEHistoryInformation	::= SEQUENCE {

lastVisitedPSCellList			LastVisitedPSCellList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SCGUEHistoryInformation-ExtIEs} } OPTIONAL,

...

}

SCGUEHistoryInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LCID ::= INTEGER (1..32, ...)

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

ListOfCells ::= SEQUENCE (SIZE(1..maxnoofCellsinAoI)) OF CellsinAoI-Item

CellsinAoI-Item ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

ng-ran-cell-id			NG-RAN-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { {CellsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

CellsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfRANNodesinAoI ::= SEQUENCE (SIZE(1.. maxnoofRANNodesinAoI)) OF GlobalNG-RANNodesinAoI-Item

GlobalNG-RANNodesinAoI-Item ::= SEQUENCE {

global-NG-RAN-Node-ID		GlobalNG-RANNode-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANNodesinAoI-Item-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANNodesinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAIsinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIsinAoI)) OF TAIsinAoI-Item

TAIsinAoI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAIsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

TAIsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LocationInformationSNReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterestInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-AdditionLocationInformation	CRITICALITY ignore	EXTENSION AdditionLocationInformation	PRESENCE optional},

...

}

LoggedEventTriggeredConfig ::= SEQUENCE {

eventTypeTrigger					EventTypeTrigger,

iE-Extensions		ProtocolExtensionContainer { { LoggedEventTriggeredConfig-ExtIEs} } OPTIONAL,

...

}

LoggedEventTriggeredConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggedMDT-NR ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

reportType							ReportType,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration			OPTIONAL,

wLANMeasurementConfiguration			WLANMeasurementConfiguration			OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration				OPTIONAL,

areaScopeOfNeighCellsList			AreaScopeOfNeighCellsList					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {LoggedMDT-NR-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-NR-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

{ID id-earlyMeasurement		CRITICALITY ignore	EXTENSION EarlyMeasurement		PRESENCE optional		},

...

}

LoggingInterval ::= ENUMERATED { ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440, infinity,...}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LPWUSPSassistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions				ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..31, ...)

LTEA2XServicesAuthorized ::= SEQUENCE {

aerialUE				AerialUE													OPTIONAL,

aerialControllerUE		AerialControllerUE											OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {LTEA2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

LTEA2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE											OPTIONAL,

pedestrianUE 		PedestrianUE										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

LTEV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTEUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

LTEUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

OMITTED TEXT UNCHANGED

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-LPWUSPSassistanceInformation																		ProtocolIE-ID ::= xxx

END

-- ASN1STOP

END OF CHANGES