3GPP TSG-RAN WG3 #127		                                                                         R3-250832

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	12.2

Source: 	ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, Lenovo, CATT, Samsung, Huawei, China Telecom

Title: 	                  (TP to 38.455) Support of Location Service Involving WAB-Nodes

Document for:	Discussion

Introduction

This paper provides the TP to BL CR for TS 38.455 to support location service involving WAB-nodes.

- Annex: TP to BL CR for TS 38.455

-------------------------------------------Start of changes-------------------------------------------

  - 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

gNB: as defined in TS 38.300 [3].

NG-RAN node: as defined in TS 38.300 [3].

ng-eNB: as defined in TS 38.300 [3].

Mobile IAB-MT: as defined in TS 38.300 [3].

WAB-MT: as defined in TS 38.300 [3].

-------------------------------------------Next change-------------------------------------------

      - 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the WAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [20] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

-------------------------------------------Next change-------------------------------------------

    - 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp, mobile trp of wab) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.88 |  | YES | ignore |
| >>Common TA Parameters | M |  | 9.2.89 |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |
| WAB-MT UE ID | C-ifMobileTRPofWAB |  | OCTET STRING | The UE ID of the WAB-MT associated with the mobile TRP in WAB-gNB. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifMobileTRP | This IE shall be present if the TRP type IE is set to the value "mobile trp" |
| ifMobileTRPofWAB | This IE shall be present if the TRP type IE is set to the value "mobile trp of wab" |

-------------------------------------------Next change-------------------------------------------

    - 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

id-NR-PCI,

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-WAB-MT-UE-ID,

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

#### -- T

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

TRPInformation ::= SEQUENCE {

tRP-ID							TRP-ID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Mobile-IAB-MT-UE-ID			CRITICALITY reject EXTENSION Mobile-IAB-MT-UE-ID		PRESENCE optional},

--This IE shall be present if the TRP type IE is set to the value "mobile trp"

ID id-WAB-MT-UE-ID			CRITICALITY reject EXTENSION WAB-MT-UE-ID		PRESENCE optional},

--This IE shall be present if the TRP type IE is set to the value "mobile trp of wab"

...

}

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

TRPType ::= ENUMERATED {

prsOnlyTP,

srsOnlyRP,

tp,

rp,

trp,

...,

mobile-trp,

Mobile-trp-of-wab

}

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

#### -- W

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

WAB-MT-UE-ID ::= OCTET STRING

-------------------------------------------Next change-------------------------------------------

    - 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-WAB-MT-UE-ID                                                 ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------