3GPP TSG-RAN WG3 Meeting #127                          R3-250834

Athens, Greece, February 17th – 21st 2025

Title:	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI)

Response to:	R3-250071/S2-2501324

Release:	Rel-19

Work Item:	FS_VMR_Ph2, NR_WAB_5GFemto-Core

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:		Ying Huang

E-mail Address:	huang.ying11@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  TP to 38.413 BL CR (R3-250833)

1. Overall Description:

RAN3 thanks SA2 for the LS reply on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) in S2-2501324.

With respect to Additional ULI, RAN3 has achieved the following agreement:

Include Additional ULI into the User Location Information IE in TS 38.413. Additional ULI contains a CGI and a TAI. In case the additional ULI has changed e.g. due to WAB-node movement, the WAB-gNB derives the new additional ULI and it reports it to the network, if required by the CN via legacy procedures. Add this description into TS38.401 In case of WAB-MT connects via NTN, the Additional ULI is determined based on WAB-node geo-location. The latter applies to intra PLMN and inter PLMN cases. SA2 should be informed of the above agreement.

And RAN3 has agreed the TP to the 38.413 BL CR (R3-250833) to capture the above agreements. RAN3 observes that the above RAN3 agreements has some misalignment with the following highlighted text in the SA2 agreed CR S2-2501322 for case when the WAB-gNB broadcasted PLMN/SNPN is the same as the BH PLMN/SNPN and the co-located WAB-MT accesses via NTN. So RAN3 would like SA2 to take the above RAN3 agreements on additional ULI into account and update the specification if needed.

5.49.4 Support of Additional ULI The TAC and cell ID broadcasted by the MWAB-gNB are configured and reconfigured e.g. upon MWAB mobility as specified in TS 38.401 [42]. The MWAB-gNB provides these in the User Location Information (ULI) to the AMF serving the UE’s accessing MWAB-gNB. The MWAB-gNB shall also provide the Additional User Location Information (Additional ULI) that reflects the location of the MWAB-UE to the AMF when it sends the User Location Information (ULI) over N2 messages. In the Additional ULI, the MWAB-gNB provides the TAI/NR CGI of the MWAB Broadcasted PLMN/SNPN with which the UE is registered as specified in TS 38.401 [42]. NOTE: For the case of MWAB broadcasted PLMN/SNPN being different from the BH PLMN, MWAB-gNB formulates the Additional ULI as specified in TS 38.401 [42] based on MWABs geo-location. The MWAB can determine its geo-location using GNSS or via a MWAB-UE initiated 5GC-MO-LR procedures as specified in TS 23.273 [87]. If the MWAB broadcasted PLMN/SNPN is the same as the BH PLMN, then the TAI/NR CGI of the cell serving the MWAB-UE is included in the Additional ULI as specified in TS 38.401 [42].

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above into account and provide feedback if any.

3. Date of Next RAN3 Meetings:

RAN3#127bis		Apr 7th – 11st 2025		Wuhan, P.R.C.

RAN3#128		May 19th – 23rd 2025		St Julien’s, Malta.