3GPP TSG-RAN WG3 Meeting #127	R3-250828

Athens, Greece, February 17th – 21st 2025

Agenda Item:	12.2

Source:	Ericsson, Huawei

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreements

Introduction

For NG connection management

Capture in TS38.401 that NG connection(s) removal for a WAB-gNB is supported.

FFS whether to Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message.

For WAB authorization

TS 38.401 to capture RAN-related aspects of WAB-node authorization based on TS 23.501.

When the authorization status of a WAB-gNB changes from “authorized” to “not authorized”:

- The WAB-gNB node attempts to hand over and/or releases the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- As agreed in SA2, “backhaul PDU Sessions are available for the MWAB gNB to be able to perform OAM control shutdown, which may include handing ove the UEs it serves”

The above is based on SA2 conclusion to capture handling of the BH PDU sessions of the WAB-MT and the deregistration of WAB-MT.

For User location information

Include Additional ULI into the User Location Information IE in TS 38.413. Additional ULI contains a CGI and a TAI.

In case the additional ULI has changed e.g. due to WAB-node movement, the WAB-gNB derives the new additional ULI and it reports it to the network, if required by the CN via legacy procedures. Add this description into TS38.401

In case of WAB-MT connects via NTN, the Additional ULI is determined based on WAB-node geo-location. The latter applies to intra PLMN and inter PLMN cases.

For PCI collision avoidance

For WAB deployments, the legacy mechanism can be reused for PCI collision avoidance. PCI space can be partitioned by allocating a range of PCIs to WAB cells.

- TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# 3	Definitions and abbreviations

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

OAM	Operation, Administration and Maintenance

PLMN	Public Land Mobile Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

UL	Uplink

ULI	User Location Information

WAB	Wireless Access Backhaul

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.3 	NG connection management

The NG connection(s) of the WAB-gNB can be removed, either due to inter-AMF mobility of a WAB-gNB or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”.

## X.4 	WAB-node authorization

WAB-node authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB.

The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB via backhaul PDU session(s). The WAB-MT is authorized based on one or more S-NSSAIs and DNN dedicated for WAB, as defined in TS 23.501 [3].

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by, e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to hand over, or releases, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- The PDU sessions of the WAB-MT used for backhauling may be released.

The WAB-MT’s PLMN/SNPN is expected to  ensure that the backhaul PDU sessions of the WAB-MT maintained long enough for the WAB-gNB to perform UE handover/release and the removal of NG and Xn connections, as specified in TS 23.501 [3].

## X.5 	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides to the core network the additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s PLMN/SNPN.

If the PLMN/SNPN broadcasted by WAB-gNB is same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI for UEs served by WAB-nodes includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI for UEs served by WAB-nodes, is determined by the WAB-gNB based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the additional ULI for UEs served by WAB-nodes is determined by the WAB-gNB based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is same as or different than the WAB-gNB’s PLMN/SNPN.

In case the additional ULI for UEs served by WAB-nodes changes, e.g., due to WAB-node movement, the WAB-gNB derives the new additional ULI and it reports it via legacy procedures, if required by the core network.

## X.6 	PCI collision avoidance

In WAB-node deployments, the legacy mechanism for PCI collision avoidance can be reused. Alternatively, PCI space can be partitioned by allocating a range of PCIs to WAB cells.

-------------------------------------------End of changes-------------------------------------------