3GPP TSG-RAN3 Meeting #128	R3-253143

Malta, MT, May 19th – 23rd, 2025

Agenda item:	10.3.1

Source:	Qualcomm Incorporated

Title:	SON MDT for NTN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss remaining issues for SON/MDT enhancements for NTN based on agreements and open issues in the previous meetings.

# Discussion

  - Geographical Area Scope in Logged MDT Configuration over NGAP

RAN2 sent a reply LS as follows:

RAN2 would like to thank RAN3 about the progress on SON/MDT for NTN and provide the reply for the following question.

Q: On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

A: RAN2 confirms that the geographical area defined for MBS NTN can be used as a reference for Area Scope of logged MDT over Uu for NTN, including both reference location/radius and polygon-based area indication.

RAN2 also agreed that only geographical area scope is used to indicate applicable logging area to the UE over Uu, i.e., no mapped cell ID(s).

We propose to follow RAN2 reply and agree on a Geographical area scope (both reference location/radius and polygon-based area indication) in Logged MDT Configuration over NGAP as follows:

Extend the Area Scope of MDT in MDT Configuration-NR over NGAP to support a geographical area scope for NTN. This can include either reference location/radius or a polygon-based area.

  - Location-based event trigger for logged MDT

NTN supports location-based measurements in RRC_IDLE and RRC_INACTIVE. Based on this, the UE will measure in RRC_IDLE and RRC_INACTIVE if it has moved a certain distance larger than a threshold and if the radio signal strength is above a certain threshold.

Current event triggers in logged MDT are only for Out-of-coverage or when signal strength is below a threshold. We propose to introduce a location-based event trigger for logged MDT.

Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.

# Conclusion

Proposal 1:	Extend the Area Scope of MDT in MDT Configuration-NR over NGAP to support a geographical area scope for NTN. This can include either reference location/radius or a polygon-based area.

Proposal 2:	Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.