3GPP TSG-RAN WG3 Meeting #128	R3-253185

St Julian’s, Malta, EU, 19-23 May 2025

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 37.340) Reformulation of the forwarding mechanism and MRO Enhancements for optimisation of the S-CPAC selection

Document for:	Discussion

# 1	Introduction

At the RAN3 #123-bis meeting, the discussion on SON for S-CPAC started. At RAN3 #127, a TP correcting the so called forwarding mechanism for the MRO signalling was agreed [1], though it was meant to be still reviewed. At RAN3 #127-bis it was briefly looked at and conclusion was made that:

The description on S-CPAC information forwarding is technically correct. Cleanup of the text is needed.

In this paper, we re-propose to reformulate the forwarding mechanism so that it better matches the existing text (originally proposed in [1]).

We also remind a proposal for enabling better decision of CPAC or S-CPAC configuration, but we discuss also an alternative solution based on the UHI.

# 2	Discussion

## 2.1. The “forwarding mechanism”

As it was concluded at RAN3 #127-bis, the signalling forwarding mechanism agreed in [2] is technically correct. However, the way the new mechanism is described is confusing (and this contributed to the fact that it required extra time for a review) and inconsistent: it uses negative conditions and describes the MN- and SN-initiated S-CPAC in different way, sometimes reusing existing description and sometimes creating a new one. Thus, it shall be reformulated to achieve better clarity.

In [1], it was proposed to align the new conditions with the existing description. Also, the way a failure at the initial and at the subsequent PSCell change shall be consistent: either the SN- and MN-initiated descriptions reuse the paragraph about the initial PSCell change, or both define it with a new set of points.

Proposal 1-1: The newly added description of the forwarding mechanism for S-CPAC is reformulated so that negative conditions are avoided, and the existing text is reused in the same way in case of MN- and SN-initiated S-CPAC.

The new description, in the part related to initial SN change in case of an MN-initiated S-CPAC reuses the existing description of the MN-initiated CPC. However, the description of the initial SN change in case of an SN-initiated S-CPAC is contained in a new bullet. This new bullet is the same as the existing description of the SN-initiated CPC, thus creating duplication.

Proposal 1-2: Considering that description of the initial SN change of SN-initiated S-CPAC is the same as the existing description of the SN-initiated CPC, in order to avoid duplications, it is better to follow the example of the description of the initial SN change of MN-initiated S-CPAC above.

## 2.2. Selection between CPAC and S-CPAC

The consequences of the use of S-CPAC introduced in Rel-18 (NR_Mob_enh2) [3] have been discussed in [4], here we remind them briefly:

Normal Rel-17 CPAC-configured UE has to release the CPAC configurations when completing random access towards the target PSCell. The advantage of S-CPAC, where multiple cells are prepared, is that an immediate UE autonomous execution for fast subsequent cell changes is possible, since conditional configuration for other candidates are not released after PSCell change. In [4], it was shown that the feature is not by default the best option, but that its application is justified in the case of fast subsequent SCG changes when operating FR2 in order to avoid preparation delays and signalling overhead.

However, if such an immediate cell change is not needed, the resource blockage resulting from the prepared and unused candidate cells can be seen as disadvantage, as well as the increased UE memory needed because of the numerous condConfig options for each potential future hop. Obviously, there are disputable pros and cons for the two CPAC methods (classical vs. subsequent) which depend on dedicated cell layout situations. In case of fast subsequent cell changes the “Subsequent CPAC” is the preferred method.

Observation 2-1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

The optimality of S-CPAC usage depends mainly on the time between needed PSCell changes. The shorter time between two consecutive CPCs, the more preferable is S-CPAC. If time is such short that UE fails with RLF before candidate cell preparation and UE configuration is completed, it is obvious that S-CPAC might be beneficial. Long residence time, on the other hand, hints usage of a the classic CPAC, so that resources are not blocked in prepared cells, while there is enough time for proper CPAC preparation.

Observation 2-2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

The key aspect is monitoring the time between PSCell changes: it shall not be too short in case of CPAC, while not too long in case of S-CPAC. This must at least be monitored per cell-pair, since cell change criteria are set per source cell towards each target cell.

The time spent between PSCell changes is key factor, but other information, like location/trajectory may also be taken into account.

Proposal 2-1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

Since - as discussed above - the key criterion for S-CPAC configuration is the UE’s dwell time in the new cell, it seems obvious to monitor that information which is provided on the RAN-side with the SCG UE History Information (UHI). Currently, the SCG history is collected at the MN, but with the support of the SNs: they can either be requested to report each PSCell change using the SN-initiated Modification procedure, or the SCG UHI may be fetched from the last serving SN using the MN-initiated Modification:

- When UE executes the subsequent SN change, the MN informs the last serving SN about it using MN-initiated Modification procedure (steps 25-26 in Figure 10.20-1 in TS 37.340);

- The MN may include the PSCell History Information Retrieve IE in the S-NODE MODIFICATION REQUEST message; in response, it will receive the SCG UE History Information IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

- If the UE performs more PSCell changes within the SN than it can record, the SN may use the S-NODE MODIFICATION REQUIRED message to provide the SCG UE History Information IE to the MN.

Thus, the MN may already now be aware of the inter-PSCell change time. The problem with the UHI is that it does not provide information about the SCG mobility type (i.e. whether the UE may be configured at the same time with CPAC or S-CPAC).

Observation 2-3: The existing mechanism defined for SCG UHI handling enables the MN to be aware of the inter-PSCell changes. However, the detection of a suboptimal configuration works only if MN can rely on that the UE is configured only with CPAC or only with S-CPAC.

Observation 2-4: Using UHI for detecting the suboptimal use of CPAC/S-CPAC, if these two can be used together, would likely require extending the UHI (and possibly reporting from the SN-initiated Modification procedure, if the MN collects SCG UHI) with mobility type information of the executed PSCell change.

Each detected mobility type issue detected by MN would then be needed to be delivered to source SN including the mobility type, target PSCell and the issue type. The delivery of that information to the source SN is missing, if the CPAC or S-CPAC was initiated by an SN. If the MN keeps the DC relation with the source SN (e.g. in case of S-CPAC, where the source stays prepared), the existing MN-initiated Modification can be used. However, for CPAC, where the DC relation is released after the UE moves to other SN, this will not work. Therefore, the ACCESS AND MOBILITY INDICATION message, which is used also for SPR, can be enhanced.

Observation 2-5: If the SCG UHI is to be used to optimise CPAC/S-CPAC usage, the ACCESS AND MOBILITY INDICATION message may be used to deliver the information to the source SN (in case of SN-initiated CPAC/S-CPAC).

The disadvantage of the UHI-based approach is the limited information, since UHI sees only the dwell time but not e.g. the type of the configuration that triggered a PSCell change. A more complete and robust solution covering these aspects, and which enables the optimisation algorithm to take into account also e.g. UE trajectory (based on provided measurements) was proposed in [4]. The UE may be configured to report the suboptimal cases identified by individually configured thresholds depends on the precedingly executed type of PSCell change. In case of CPC configuration, it has to be checked whether the time is shorter than a defined “threshold_min_consecutive_CPC”, and if it is the case, it informs the network about a mobility method issue related to preceding PSCell change, and this information has to be forwarded to the MRO instance responsible for this cell border. The counter there may be incremented, which expresses that the two consecutive CPC-based cell changes are “tight in succession” indicating that S-CPAC might be preferable.

If UE is however already configured with S-CPAC, the opposite is to be monitored by the UE, namely whether the time is longer than a defined “threshold_max_consecutive_S-CPAC”, and this logged information is to be reported to the MRO instance responsible for this cell border, where a counter is incremented, which expresses that the two consecutive S-CPAC based cell changes are “too widely spaced” and would indicate that S-CPAC might waste network resources and normal CPC would be preferable, since time for preparation is not an issue.

Observation 2-6: A UE-based mechanism with configurable time thresholds for getting aware of sub-optimal mobility method usage, dedicated for the S-CPAC and CPAC, is more robust than the SCG UHI-based one. However, it requires involvement of RAN2.

Proposal 2-2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

# 3	Conclusions

In this paper, we start from reviewing the newly added description of the forwarding mechanism for MRO for S-CPAC. We propose that:

Proposal 1-1: The newly added description of the forwarding mechanism for S-CPAC is reformulated so that negative conditions are avoided and the existing text is reused in the same way in case of MN- and SN-initiated S-CPAC.

Proposal 1-2: Considering that description of the initial SN change of SN-initiated S-CPAC is the same as the existing description of the SN-initiated CPC, in order to avoid duplications, it is better to follow the example of the description of the initial SN change of MN-initiated S-CPAC above.

Related corrections are proposed in a TP for stage-2 in the annexes 1 below.

Later, we discuss the problem of selecting the most appropriate mobility scheme for a UE: CPAC or S-CPAC. We make following observations:

Observation 2-1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

Observation 2-2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

Based on those, we propose:

Proposal 2-1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

We discuss the two approaches, based on SCG UHI and on SPR. We observe:

Observation 2-3: The existing mechanism defined for SCG UHI handling enables the MN to be aware of the inter-PSCell changes. However, the detection of a suboptimal configuration works only if MN can rely on that the UE is configured only with CPAC or only with S-CPAC.

Observation 2-4: Using UHI for detecting the suboptimal use of CPAC/S-CPAC, if these two can be used together, would likely require extending the UHI (and possibly reporting from the SN-initiated Modification procedure, if the MN collects SCG UHI) with mobility type information of the executed PSCell change.

Observation 2-5: If the SCG UHI is to be used to optimise CPAC/S-CPAC usage, the ACCESS AND MOBILITY INDICATION message may be used to deliver the information to the source SN (in case of SN-initiated CPAC/S-CPAC).

Observation 2-6: A UE-based mechanism with configurable time thresholds for getting aware of sub-optimal mobility method usage, dedicated for the S-CPAC and CPAC, is more robust than the SCG UHI-based one. However, it requires involvement of RAN2.

And thus we propose:

Proposal 2-2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

Related TP for stage-2 and a draft LS to RAN2 are proposed in the annexes 1 and 2 below.

# References

- R3-251595, RAN3 #127-bis

- R3-250900, RAN3 #127

- RP-223520, RAN #98-e

- R3-241561, RAN3 #123-bis

# Annex 1: Text proposal to TS 37.340 (MRO for S-CPAC)

The below text is proposed to be added to the BL CR to TS 37.340 to correct the information forwarding mechanism:

First change, ommited text not changed

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration/CHO with Candidate SCG(s) configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC/CHO with Candidate SCG(s) or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure:

-	Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or initial S-CPAC initiated by MN or CHO with Candidate SCG(s), if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, the MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated CPC or initial S-CPAC initiated by SN (the source SN is also the initiating SN at S-CPAC preparation), the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For a following S-CPC failure during a SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

Next change, ommited text not changed

Then, the below text is proposed to be appended to the same chapter in the BL CR to TS 37.340 to describe the MRO mechanism helping decide between CPAC and S-CPAC:

Next change, ommited text not changed

Optimisation of S-CPAC

Another function of self-optimisation for S-CPAC is to coordinate the usage of CPAC and S-CPAC. For that purpose, the UE may be configured to provide the needed information:

-	If the UE is configured with the CPC, it may be requested to report when the time between two consecutive CPC executions is lower than 'threshold_min_consecutive_CPC';

-	If the UE is configured with the S-CPAC, it may be requested to report when the time between two consecutive CPC executions is higher than 'threshold_max_consecutive_S-CPAC'.

Remaining text not changed

# Annex 2: Draft LS to RAN2 (MRO for S-CPAC)

3GPP TSG-RAN WG3 #128		R3-25xxxx

St. Julian's, Malta, EU, 19-23 May 2025

Title:	[DRAFT] LS on MRO for S-CPAC

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-25xxxx

1. Overall Description:

RAN3 would like to inform RAN2 about decisions made in the context of MRO for S-CPAC.

- Regarding handling execution failures during S-CPAC operations, RAN3 assumed the existing framework defined for CPAC will be used.

- RAN3 has also decided to enable a mechanism to support selection of the configuration for the UE: CPAC or S-CPAC. For this purpose, RAN3 agreed a TP, as attached to the LS.

2. Actions:

To RAN2

RAN3 would like to ask RAN2 to enable the needed signalling, as described in the attached TP.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129			25-29 August 2025						Bangalore, India

TSG-RAN3 Meeting #129-bis			13-17 October 2025					Prague, Czechia, EU