3GPP TSG-RAN WG3 Meeting #128	R3-253322

Malta, MT, 19th – 23th May, 2025

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Discussion on remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, some remaining issues are discussed based on the latest BL CR.

- Discussion

Security verification

Based on the reply LS [2] from SA3, SA3 had agreed all the verifications item listed below by RAN3, which to be performed by the NR Femto GW or AMF in case the NR Femto directly connected to the AMF.

For the NR Femto Gateway: 1: In case of NGAP INITIAL UE MESSAGE message, whether to verify that reported CAG ID is valid for that NR Femto. 2: In case of NGAP SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid. 3: In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid. For the AMF: 1: In case of an NR Femto directly connected, whether to verify that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell. 2: In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message. 3: In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

Therefore, the NR Femto GW or the AMF needs to support the additional functions related to security verification.

Proposal 1: The NR Femto GW supports the additional functions related to security verification.

-	In case of NGAP INITIAL UE MESSAGE message, verifying, as defined in TS 33.320 [x], that the reported CAG ID is valid for the NR Femto.

-	In case of NG SETUP REQUEST message, verifying, as defined in TS 33.320 [x], that the identity used by the NR Femto is valid.

-	In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, verifying, as defined TS 33.320 [x], that the indicated cell identity is valid.

Proposal 2: The AMF supports the additional functions related to security verification in case of an NR Femto directly connected.

-	verifying as defined in TS 33.320 [x], that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell;

-	verifying as defined in TS 33.320 [x], that the identity used by the NR Femto is valid when receiving the NG SETUP REQUEST message;

-	and verifying, as defined in TS 33.320 [x], that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

Handover

As in LTE HeNB, the source eNB configures the UE with proximity indication control and the UE sends an "entering" proximity indication when it determines it may be near a CSG member cell. Then the source eNB configures the UE with measurement configuration and further SI acquisition and reporting to acquire the relevant system information from the target HeNB.

7) The UE sends a measurement report including (E-)CGI, TAI, CSG ID and "member/non-member" indication. If the target cell is a shared CSG/hybrid cell, the measurement report also includes the subset of the broadcast PLMN identities that pass PLMN ID check and for which the CSG whitelist of the UE includes an entry comprising the cell's CSG ID and the respective PLMN identity.

However, in NR, each NG-RAN node informs the connected neighbour NG-RAN nodes of the list of supported CAG ID(s) per CAG cell in the Xn interface management procedures, e.g., Xn setup procedure. In addition, the cell type (CAG cell or NPN-only cell) can be also obtained implicitly via absent of the PLMN Identities and associated information in the PLMN-IdentityInfoList IE.

Therefore, there is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Terminology

It is FFS how to harmonise terminology for “NR Femto”, “NR Femto Node”. Terminology shall be consistent.

For the terminology, we had agreed in the first meeting that RAN3 to use the term “NR Femto node”. However, during the subsequence normative work, we use “NR Femto” instead. Anyway, the two terminologies have the same meanings for RAN3, and “NR Femto” is preferred to reduce the impacts on the BL CRs.

Proposal 4: To use the term “NR Femto” in R19.

Transport layer address for NG-U

RAN3 #127: The Femto GW hosts the following function: - Selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions. To be continued… RAN3 #127bis: If the SMF sends both IP versions in the Transport Layer Address IE, the NR Femto node selects the correct IP version. It is FFS whether the GW can perform IP Version selection in addition to the NR Femto.

For UE served by the NR Femto, the 5GC and NR Femto setup the PDU session resource via the UE-associated NGAP procedure, which is just relayed by the NR Femto GW, then the SMF is aware of the DL TNL address of the NR Femto for the NG-U transport.

In addition, based on the protocol stack for NG-U of NR Femto, the NR Femto GW just plays a role of IP router and not terminate NG-U from the 5GC. And in last meeting, it was agreed that NR Femto can select the correct IP version in case the SMF sends both IP versions. There is no need for the NR Femto GW to perform IP version selection.

Proposal 5: The NR Femto GW doesn’t need to perform IP version selection in addition to the NR Femto.

# Conclusion

This contribution aims to discuss remaining issues for NR Femto. And following observations and proposals are concluded.

Proposal 1: The NR Femto GW supports the additional functions related to security verification.

-	In case of NGAP INITIAL UE MESSAGE message, verifying, as defined in TS 33.320 [x], that the reported CAG ID is valid for the NR Femto.

-	In case of NG SETUP REQUEST message, verifying, as defined in TS 33.320 [x], that the identity used by the NR Femto is valid.

-	In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, verifying, as defined TS 33.320 [x], that the indicated cell identity is valid.

Proposal 2: The AMF supports the additional functions related to security verification in case of an NR Femto directly connected.

-	verifying as defined in TS 33.320 [x], that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell;

-	verifying as defined in TS 33.320 [x], that the identity used by the NR Femto is valid when receiving the NG SETUP REQUEST message;

-	and verifying, as defined in TS 33.320 [x], that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Proposal 4: To use the term “NR Femto” in R19.

Proposal 5: The NR Femto GW doesn’t need to perform IP version selection in addition to the NR Femto.

Based on the proposals, a TP is drafted to capture the aspects related to NR Femto in another paper [3].

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- R3-253021/ S3-251668. Reply LS on security verification related to NR Femtos. SA3.

- R3-253323. (TP to BL CR 38.300) Function split for NR Femto. Lenovo.