3GPP TSG-RAN WG3 #128                                                 R3-253439

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 38.473 and 38.401) MRO for R18 LTM

Document for:	other

Introduction

In last RAN3 meeting, the following agreements and FFS are captured:

LTM:

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supportsnetwork-based solution for the case ofBFR shortly after a successful LTM cell switch.

The target DUidentifies the BFRhappened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

UHI and ping-pong issue

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - Case 1: BFR shortly after successful LTM cell switch to the wrong beam

In last RAN3 meeting, Case 1 is acknowledged and following network solution has been agreed:

RAN3 supports network-based solution for the case of BFR Shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Here we continue discussing the details.

Once BFR is successfully completed, if it is source DU to make optimization, the target DU should inform CU the suitable beam information immediately. Then CU send this information to source DU. Source DU should fetch UE context, for example, LTM configuration as UE context is used to deduce TCI state from the beam information provided by target DU. Then next time source DU should trigger LTM cell switch with the right TCI state.

Therefore, UE ID should also be provided to source DU to fetch UE context

Proposal 1: UE associated message should be sent immediately from the target DU to CU which includes UE ID and the suitable beam information.

DU-CU ACCESS AND MOBILITY INDICATION message is suitable for this case, so, we propose to reuse DU-CU ACCESS AND MOBILITY INDICATION message.

Proposal 2: It is proposed to reuse DU-CU ACCESS AND MOBILITY INDICATION message to send UE ID and beam information from target DU to CU.

To send recovery beam information from CU to source DU, ACCESS AND MOBILITY INDICATION should be reused.

Proposal 3: It is proposed to reuse ACCESS AND MOBILITY INDICATION to send UE ID and beam information from CU to source DU.

  - Case 2: LTM cell switch failure due to wrong beam

In last RAN3 meeting, for case 1, following FFS is captured.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)

In case of LTM cell switch failure (HOF), since UE fail to access target DU, for example, MSG1 is not received in target DU, then target DU could not collect available failure information (target beam) which is different from case 1.So, the procedure in case1 (target DU->CU->source DU)is not feasible for case2.

Proposal 4: The solution1 could not be used for the HOF in case 2.

In case 2, RLF report would be generated in UE. After receives RLF report, CU could forward it to related DU to make analysis and optimization.

If the last serving DU is source DU, CU send RLF report to source DU, source DU should retrieve UE context which include beam information used in cell switch command. If the suitable beam analysed by source DU does not include in the candidate beam list, it will send the RLF report to target DU to optimize candidate beam selection. Otherwise, source DU perform optimization.

If the last serving DU is target DU, CU send RLF report to target DU, target DU should retrieve UE context which include beam information used in cell switch command. If the suitable beam analysed by target DU is included in the candidate beam list, it will send the RLF report to source DU to optimize candidate beam selection. Otherwise, target DU perform optimization. So, this scenario is too complex for RLF report forwarding, and considering time limit, it is proposed to not support case 2.

Proposal 5: It is proposed to not support case2 for complexity.

  - LTM failure due to outdated TA

If RACH-less LTM cell switch fails and the recovery cell ID is same as target cell ID, the reason may be outdated TA issue or wrong beam issue (case2). And wrong beam issue can be detected by source DU according to RLF report, i.e. source DU select suitable beam according to L1 measurement result in RLF report and then decided whether the beams used in previous LTM cell switch is the suitable beam. If target beam is different from suitable beam, the failure is due to wrong beam. Otherwise, the failure is due to outdated TA. Next time source DU should trigger TA acquisition more early.

Proposal 6: In case recovery cell ID is same as target cell ID in RLF report, CU send the RLF report to source DU via ACCESS AND MOBILITY INDICATION message and source DU could detect whether LTM failure due to outdated TA issue or wrong beam.

According to above analysis, CU should forward RLF report to source DU in case of successful LTM recovery to target cell. Since RLF report has been included in ACCESS AND MOBILITY INDICATION message and failure type is not a Mandatory IE, only stage-2 change is needed.

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn.The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering ,”successful LTM recovery to target cell” may forward the RLF report to the gNB-DU responsible for the failure.

Proposal 7: Agree TP 38.401 in Annex to reflect RLF report need to be sent to DU in case of successful LTM recovery to target cell.

  - Network based or UE based solution for TA acquisition type

There is an open as below:

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

CU decides TA acquisition type and sends UE Based TA Measurement Configuration to UE and DU respectively. Source DU should follow the decision of CU. If CU decides network based TA but source DU does not send PDCCH order to UE, it belongs to the wrong DU implementation which should not be considered in SON.

Observation 1: TA acquisition type should always be available in the CU.

If companies really want to avoid above bad implementation, as a supplementary, an indicator could be included in DU-CU CELL SWITCH NOTIFICATION message to indicate the TA acquisition type.

Proposal 8: If companies want to solve bad DU implementation, TA acquisition type could be included in DU-CU CELL SWITCH NOTIFICATION message as a supplementary for compromise.

  - Mobility Information

According to legacy MRO, for the case that RLF occurs shortly after successful handover, source NG-RAN may not save UE context for the following reason:

1. Successful handover is the common case and usually the number of successful handed over UE is large. NG-RAN may not save UE context for each successful handed over UE.

2. Source NG-RAN stores UE context to wait for handover Report message from target NG-RAN. If no handover Report message received, UE context may have to be stored for 48 hours. For the same reason as above bullet, most of the time source NG-RAN cannot receive handover Report message.

Considering the complexity of saving UE context in source NG-RAN, Mobility Information is introduced in XN interface message e.g., indicating UE type.

If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node.

Observation 2: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

For legacy handover, it is CU to perform MRO based on Mobility Information. While for LTM, it is DU to perform MRO. So, Mobility Information should be provided to DU to avoid UE context saved by DU for each successful handover.

For intra-CU inter-DU LTM, source DU could provide Mobility Information to CU via UE CONTEXT MODIFICATION RESPONSE, DU-CU CELL SWITCH NOTIFICATION or UE CONTEXT RELEASE COMPLETE message, etc. Then CU stores Mobility Information and provides Mobility Information back to DU via ACCESS AND MOBILITY INDICATION message.

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early LTM" or a "LTM to Wrong Cell", the gNB-CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message. If the source gNB-DU controlling that source cell provided the Mobility Information, it is also included in the ACCESS AND MOBILITY INDICATION message. If used, the Mobility Information is prepared at the source gNB-DU of a LTM and may refer to or identify any LTM-related data at this source gNB-DU.

Proposal 9: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

  - Ping-pong issue

In last RAN3 meeting, the following agreements were captured.

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

Ping-pong issue is detected based on UHI in CU, then in order for DU to solve LTM ping-pong, UHI should be sent from CU to DU.

Proposal 10: In order for DU to solve LTM ping-pong issue, it is proposed to send UHI from CU to DU.

UHI is updated by CU in case of successful LTM or L3 triggered handover completion.

For example, UHI in following table may be recorded in CU.

| Index | Cell ID | Stay time | Handover type |
| --- | --- | --- | --- |
| 1 | A | a | X |
|  |  |  | LTM |
| 2 | B | b |  |
|  |  |  | L3 handover |
| 3 | A | c |  |
|  |  |  | L3 handover |
| 4 | D | d |  |
|  |  |  | LTM |
| 5 | A | e |  |
|  |  |  | LTM |
| 6 | D | f |  |
|  |  |  | X |

For index 1~3, i.e. CellA ->LTM ->CellB ->L3 handover ->CellA, although the stay time in CellB is short, there is no LTM ping-pong issue because CellB ->L3 handover ->CellA is L3 handover.

For index 4~6, i.e. CellD ->LTM ->CellA ->LTM ->CellD, if the stay time in CellA is short, there is LTM ping-pong issue.

To detect LTM ping-pong issue, besides UHI, handover type should be known by DU.

Proposal 11: It is proposed to include handover type i.e., L3 handover or L1/L2 cell switch in UHI.

To decrease F1 burden, it may be not needed to send UHI from CU to DU each time PCell changes. CU could also detect LTM ping-pong issue and the optimization on ping-pong base on statistic. We prefer sending UHI in case of LTM ping-pong issue detected by CU.

As for the F1 message to send UHI from CU to DU, ACCESS AND MOBILITY INDICATION message could be reused.

Proposal 12: It is proposed to send UHI in ACCESS AND MOBILITY INDICATION message from CU to DU in case of LTM ping-pong issue is detected by CU.

Conclusions

Proposal 1: UE associated message should be sent immediately from the target DU to CU which includes UE ID and the suitable beam information.

Proposal 2: It is proposed to reuse DU-CU ACCESS AND MOBILITY INDICATION message to send UE ID and beam information from target DU to CU.

Proposal 3: It is proposed to reuse ACCESS AND MOBILITY INDICATION to send UE ID and beam information from CU to source DU.

Proposal 4: The solution1 could not be used for the HOF in case 2.

Proposal 5: It is proposed to not support case2 for complexity.

Proposal 6: In case recovery cell ID is same as target cell ID in RLF report, CU send the RLF report to source DU via ACCESS AND MOBILITY INDICATION message and source DU could detect whether LTM failure due to outdated TA issue or wrong beam.

Proposal 7: Agree TP 38.401 in Annex to reflect RLF report need to be sent to DU in case of successful LTM recovery to target cell.

Observation 1: TA acquisition type should always be available in the CU.

Proposal 8: If companies want to solve bad DU implementation, TA acquisition type could be included in DU-CU CELL SWITCH NOTIFICATION message as a supplementary for compromise.

Observation 2: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

Proposal 9: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

Proposal 10: In order for DU to solve LTM ping-pong issue, it is proposed to send UHI from CU to DU.

Proposal 11: It is proposed to include handover type i.e., L3 handover or L1/L2 cell switch in UHI.

Proposal 12: It is proposed to send UHI in ACCESS AND MOBILITY INDICATION message from CU to DU in case of LTM ping-pong issue is detected by CU.

Reference

[1] R3-251514“LS on SON for LTM”RAN WG3 Meeting #127-bis

TP for TS 38.473

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 8.3.3	UE Context Release (gNB-CU initiated)

- 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection, candidate cells in conditional handover, conditional PSCell addition, or conditional PSCell change or LTM or subsequent CPAC. The procedure uses UE-associated signalling.

- 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CG-SDT-CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication or multi-path relay based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC associated to the cells identified by the included NR CGIs and associated to the UE-associated signalingidentified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the LTM Cells To Be Released List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU is cancelling only the LTM cells identified by the included NR CGIs and associated to the UE-associated signalingidentified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Recommended SSBs for Paging List IE is included in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store it and may use it as assistance information for subsequent paging.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU requests the reporting of DL LBT Failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6],  and act as specified in TS 38.401 [4].

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this informationand use it as specified in TS 38.401 [4].

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

- 8.3.4	UE Context Modification (gNB-CU initiated)

- 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

- 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

<<<Unchanged part is omitted>>>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this informationand use it as specified in TS 38.401 [4].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

- 8.3.9	DU-CU Cell Switch Notification

- 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable thegNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

- 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the Mobility Information IE is provided in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, store this informationand use it as specified in TS 38.401 [4].

8.11	Self Optimisation Support procedures

[snip]

- 8.11.1	Access and Mobility Indication

- 8.11.1.1	General

This procedure is initiated by gNB-CU to sendtheAccess and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

- 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITYINDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITYINDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information ListIE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Beam failure information IE the gNB-DU may take it into account for optimisation of LTM beam failure parameters.

If the ACCESS AND MOBILITY INDICATION message contains the UE History Information IE the gNB-DU may take it into account for optimisation of LTM ping-pong issue.

- 8.11.1.3	Abnormal Conditions

Not applicable.

- 8.11.2	DU-CU Access and Mobility Indication

- 8.11.2.1	General

This procedure is initiated by the gNB-DU to sendtheAccess and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

- 8.11.2.2	Successful Operation

# Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITYINDICATION message contains the DL LBT Failure Information List IE,the gNB-CU shall take it into account for optimisation of mobility parameters.

If the DU-CU ACCESS AND MOBILITYINDICATION message contains the Beam failure information IE the gNB-CU shall forward it to related gNB-DU for optimisation of LTM beam failure parameters.

- 8.11.2.3	Abnormal Conditions

Not applicable.

- 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connectionor candidate cells in conditional handover, conditional PSCell addition,conditionalPSCell change or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Recommended SSBs for Paging List | O |  | 9.3.1.297 |  | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

- 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary pathor for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 ..<maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 ..<maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 ..<maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 ..<maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used inthis version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 ..<maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channelswhich are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channelswhich are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG InformationIEindicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 ..<maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 ..<maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | - |  |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| Dedicated SI Delivery Indication | O |  | ENUMERATED(true,...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 ..<maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes theLTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfigIE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or L2 U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

- 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction:gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includesthe TCI-StateIdIE,as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateIdIE,as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 ..<maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 ..<maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 ..<maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (toolate LTM, too early LTM, LTM to wrongcell, successful LTM recovery to target cell,...) |  | YES | ignore |
| >>MobilityInformation | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 ..<maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-ReportIE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCellChange Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-ReportIE as defined in TS 38.331 [8]. | - |  |
| Beam failure information | O |  |  |  | YES | ignore |
| >UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >SSBIndex | O |  | INTEGER(0..63) |  |  |  |
| >CSI-RSIndex | O |  | INTEGER(0..191) |  |  |  |
| UE History Information | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell ChangeReports. Value is 64. |

- 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by thegNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| >DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam failure information | O |  |  |  | YES | ignore |
| >UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >SSBIndex | O |  | INTEGER(0..63) |  |  |  |
| >CSI-RSIndex | O |  | INTEGER(0..191) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

- 9.3.1.X	UE History Information

This IE contains information about cells that a UE has been served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >Last Visited Cell Information | M |  | 9.3.1.Y |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |

- 9.3.1.Y	Last Visited Cell Information

This IE may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN Cell Information | M |  | 9.3.1.Z |  |

- 9.3.1.Z	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell,or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell,or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Handover Type | O |  | ENUMERATED (L3 handover, LTM, …) |  | YES | ignore |

TP for TS 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn.The gNB-CU performs initial analysisand in case of failure due to inappropriate cell switch triggering,“successful LTM recovery to target cell”may forward the RLF report to the gNB-DU responsible for the failure.

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early LTM" or a "LTM to Wrong Cell", the gNB-CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message. If the source gNB-DU controlling that source cell provided the Mobility Information, it is also included in the ACCESS AND MOBILITY INDICATION message. If used, the Mobility Information is prepared at the source gNB-DU of a LTM and may refer to or identify any LTM-related data at this source gNB-DU.