3GPP TSG-RAN WG3 Meeting #128	R3-253462

Malta, MT, 19 – 23 May 2025

Agenda item:	20.2

Source:	Ericsson

Title:	(TP for TS 38.455) Support of data collection procedure (case 3a)

Document for:	other

# Introduction

## Data Collection

In previous meeting the following agreement on positioning data collection was taken:

- An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned.

And related signalling flow [1] endorsed:

1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. 4. The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

While RAN3 is waiting for SA2 feedback on the NGAP based solution, it is proposed to discuss this meeting how to capture the stage 3 impacts for the NRPPa-based solution in the BL CR. Currently there is an impact to enhance the MEASUREMETN RESPONSE with a new Positioning Data Collection Needed IE.

In case of periodic measurement reporting, it should be possible for the gNB to indicate its request for ground truth also during a MEASUREMENT REPORT message. Hence the MEASUREMENT REPORT message should also be impacted.

Observation 1: the gNB should be able to request for ground truth at any time during a positioning procedure, during on demand or periodic measurements reporting.

Proposal 1: Both the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages should include the new Positioning Data Collection Needed IE.

Furthermore, after step 3, if the LMF cannot provide the requested ground truth to gNB, it should reply with a NRPPa based data collection failure message. Hence it is proposed that a new NRPPa POSITIONING DATA COLLECTION FAILURE message is also defined,

Observation 2: The LMF should be able to fail a request for data collection by sending a NRPPa based data collection failure message.

Proposal 2: Define a new NRPPa POSITIONING DATA COLLECTION FAILURE message

We note the following RAN1 agreement:

Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

It can be understood that the LoS/NLoS information defined in NRPPa should also be included as part of the assistance data for data collection with the ground truth.

## Model Inference

In the context of Rel-19 AI/ML positioning, the LMF should understand if the inferred UL-RTOA measurement was generated using AI/ML methods. One reason is that the related attributes (e.g., quality, LoS-NLoS) may differ if the measurement was not inferred (e.g., the measurement may be in NLoS in case of non-inferred). Another reason is that the LMF may adopt different behaviours if the measurements provided by the RAN are AI/ML inferred. As an example, the LMF may decide to request measurements from more RAN nodes if the measurement received from the RAN are AI/ML inferred. This is to ensure a higher amount of measurements and to be able to filter out measurements with high deviation, while still being able to position the UE.

Observation 3: AI/ML positioning measurements are different from legacy positioning measurements; the attributes may also differ. The LMF may also adopt different behaviours if the measurements provided by the RAN are AI/ML inferred.

It has been proposed several times by many companies that the easiest way to accomplish this is to add a new flag for the inferred AI-ML measurements in the TRP measurement result IE, which would help distinguish the AI/ML positioning measurements from the legacy measurements in the TRP Measurement report.

Proposal 2: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

# 2	Conclusions

Observation 1: the gNB should be able to request for ground truth at any time during a positioning procedure, during on demand or periodic measurements reporting.

Proposal 1: Both the MEASUREMENT RESPONSE and MEASUREMENT REPORT messages should include the new Positioning Data Collection Needed IE.

Observation 2: The LMF should be able to fail a request for data collection by sending a NRPPa based data collection failure message.

Proposal 2: Define a new NRPPa POSITIONING DATA COLLECTION FAILURE message

Observation 3: AI/ML positioning measurements are different from legacy positioning measurements; the attributes may also differ. The LMF may also adopt different behaviours if the measurements provided by the RAN are AI/ML inferred.

Proposal 2: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

# 3	TP to NRPPa BL CR

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Positioning Data Collection Transfer: This function allows the LMF to provide positioning information to the NG-RAN node for the purpose of Data Collection.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Positioning Data Collection Transfer | a) Positioning Data Collection Report c) Positioning Data Collection Failure |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Report | POSITIONING DATA COLLECTION REPORT |
| Positioning Data Collection Failure | POSITIONING DATA COLLECTION FAILURE |

Next change

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

If the AI/ML measurement IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF shall, if supported, consider that the associated positioning information has been derived using AI/ML inference.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT RESPONSE	 message, the LMF shall, if supported, take it into account for providing the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

Next change

### 8.5.2	Measurement Report

#### 8.5.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT REPORT message, the LMF shall, if supported, take it into account for providing the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

#### 8.5.2.3	Abnormal Conditions

Void.

Next change

## 8.X	Positioning Data Collection Transfer

#### 8.X.1	Positioning Data Collection Report

#### 8.X.1.1	General

The Positioning Data Collection Report procedure allows the LMF to provide positioning information to the NG-RAN for data collection.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2.1: Positioning Data Collection Report procedure. Successful operation.

The LMF initiates the procedure by sending a POSITIONING DATA COLLECTION REPORT message to the NG-RAN. The POSITIONING DATA COLLECTION REPORT message contains the requested positioning information for data collection at NG-RAN.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Positioning Data Collection Failure

#### 8.X.2.1	General

The Positioning Data Collection Failure procedure allows the LMF to indicate that it cannot provide any data collection information to the NG-RAN node.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Positioning Data Collection Failure procedure. Successful operation.

The LMF initiates the procedure by sending a POSITIONING DATA COLLECTION FAILURE message to the NG-RAN node.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

Next change

#### 9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Positioning Data Collection Needed | O |  | 9.2.Z |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

Next change

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Positioning Data Collection Needed | O |  | 9.2.Z |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

Next change

### 9.1.X	Messages for Positioning Data Collection Transfer Procedures

#### 9.1.X.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Data Collection Information Type | M |  |  |  | - |  |
| >UE location | M |  |  |  |  |  |
| >>Location Information | M |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14]. | - |  |
| >>Time Stamp | O |  | 9.2.42 | Location time stamp. | - |  |
| >>LoS/NLoS Information | O |  | 9.2.77 |  | - |  |
| >Measurement | M |  |  |  |  |  |
| >>TRP Measurement List |  | 0..1 |  | Indicates the area of interest. | YES | reject |
| >>>TRP Measurement Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>>>TRP ID | M |  | 9.2.24 |  | - |  |
| >>>>UL RTOA | O |  | 9.2.39 |  | - |  |
| >>>>Time Stamp | O |  | 9.2.42 |  | - |  |
| >>>>Measurement Quality | O |  | 9.2.43 |  | - |  |
| >>>>LoS/NLoS Information | O |  | 9.2.77 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.2	POSITIONING DATA COLLECTION FAILURE

This message is sent to indicate that the positioning information cannot be reported.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA, gNB Rx-Tx Time Difference and LoS/NLoS Information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

Next change

### 9.2.Z	Positioning Data Collection Needed

This information element contains the requested positioning information for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Positioning data Information Type | M |  |  |  |
| >UE location | M |  | ENUMERATED(true,…) |  |
| >Measurement type | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a requested measurement type: first bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement type’, Value ‘0’ indicates ‘not requested’. |

Next change

ASN.1 to be added later

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>