3GPP TSG-RAN WG3 Meeting #128	R3-253468

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for general issues in Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

- Discussion

2.1 Signal UE association

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

To guarantee signal UE association, the source and target node shall using the same ID pair for the same UE in the request message for different candidate cells, which means both the source XNAP ID and the target XNAP ID shall be same. To maintain the single association, this principle shall be present in stage3. An example for stage3 detail shown as below

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. For LTM, parallel requests shall use the same pair of UE AP ID. The procedure uses UE-associated signalling.

Proposal 1: Considering the above wording to limited single UE association for multiple request to a same gNB.

2.2 CSI-RS resource

2.2.1 CSI-RS for RSRP

RAN3 had the following agreement in RAN3 #127bis meeting:

Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

Regarding the granularity, RAN2

Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

It seems in RAN2 has concluded to adopt the granularity as resource set. However, according to RAN1 LS [1] regarding L1 parameters, the SP CSI RS resource is provided under LTM-CSI-ResourceConfig-r18 via LTM-NZP-CSI-RS-ResourceSet-r19, which specifics one NZP-CSI-RS resource set from one or more candidate cells. This contrasts with the legacy SP CSI-RS resource set which is for only one cell.

Hence, RAN2 may further discuss the granularity of the MAC CE, e.g. per LTM-NZP-CSI-RS-ResourceSet,

(i.e. for one LTM-NZP-CSI-RS-ResourceSet-r19, all or none of the RS be activated), per candidate cell(allow to activate part candidate cell’s RS in the LTM-NZP-CSI-RS-ResourceSet-r19), per RS (allow to activate part of the RS in the LTM-NZP-CSI-RS-ResourceSet-r19).

Proposal 2: For the granularity of the CSI-RS resource/resource set, postpone the discussion in RAN3 and wait RAN2 further conclusion.

2.2.2 CSI-RS for CSI acquisition

Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

In RAN1 new coming LS [2], RAN1 has informed RAN3 that they support using SP CSI-RS for candidate cell CSI acquisition, and have requested RAN3 to further enhance this function to fully support the inter-DU/inter-CU case..

This issue is similar to the one we previously discussed regarding the support of SP CSI-RS resources for candidate cell L1-RSRP measurement. However, there are some unclearly poins that need to be clarified, as listed below

- Regard of CSI-RS configuration request

In the case of CSI-RS resources for candidate cell L1 RSRP measurement, the source node determines whether the candidate gNB needs to provide CSI-RS resources. A similar logic can be applied for CSI-RS resources used for candidate cell CSI acquisition.

Considering the CSI acquisition can be based on periodic CSI-RS resource or semi-persistent CSI-RS resource, the candidate gNB can decided which kind of CSI-RS resource will be used.

Proposal 3: Introduce a new indicator in HANDOVER REUQETS message to request candidate cell providing CSI-RS resource for CSI acquisition. The decision on whether to provide periodic or semi-persistent CSI-RS resources will be up to the candidate gNB.

- Regard of SP CSI-RS acquisition:

Once the SP CSI-RS resource for CSI acquisition is configured, the activation and deactivation procedure should be considered. Similar to the case of SP CSI-RS for RSRP, there are also two possible Alts for SP CSI-RS activated/ deactivated.

Alt 1: The source node indicates to the candidate node which SP CSI-RS resources should be activated or deactivated, and the candidate gNB provides a response message back to the source node.

Alt 2: The source node triggers activate/deactivate SP CSI-RS for CSI acquisition. The candidate gNB selects which SP CSI-RS resources to be activate/deactivate. Finally, the candidate gNB provides the CSI-RS information back to the source node.

In our view, Alternative 2 is more appropriate for CSI acquisition. This is because CSI acquisition is primarily used for scheduling at the target node. Given that the target node has a more comprehensive understanding of the network conditions and the specific requirements of the incoming UE, it is better positioned to determine which RS are most suitable for the UE.

Proposal 4: It is proposal source node to trigger SP CSI-RS active/ deactive for CSI acquisition, and candidate node decide the specific CSI-RS and send it back to source node.

2.3 Subsequent LTM request.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNOu=iiiiB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

To enable UE context identification between two candidate gNBs, the initial gNB should transmit the target UE’s XnAP ID to the other candidate cells. One feasible approach is to extend the Configuration ID Mapping List IE in the following manner.

.2.1.xx5	LTM Configuration ID Mapping List

This IE indicates the list of LTM candidate cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >LTM Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [10]. |
| Assigned target UE XnAP ID | O | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |

And once the candidate cell decides to establish UE association with another candidate cell, it should include the received Assigned target UE XnAP ID in the LTM configuration update message.

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node ‘UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Assigned target UE XnAP ID | O | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES |  |  |

Proposal 5: The initial source gNB should send the target cell and the assigned target UE XnAP ID mapping list to all candidate cells via the LTM CONFIGURATION UPDATE message.

Proposal 6: When establishing a new UE association between two candidate gNBs, the request node should include the old target UE XnAP ID in the LTM CONFIGURATION UPDATE message.

2.4 REL-19 ID

RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update.

In the new LS[3], RAN2 assumes that the network can configure different R19 set ID for the cells belong to the same CU. This assumption, however, is not fully consistent with REL-18..

To meet the flexible requirement, each candidate gNB should decide whether the key need to update when intra-node handover occurs. One possible approach is the candidate gNB provide assistance information to the source gNB, and enable the source gNB to make the final decision to ensure the global uniqueness of the Rel-19 Set ID.

For example, in the HANDOVER REQUEST ACK message, the candidate gNB shall provide the Sub-Set ID for the requested cell, the usage of Sub-Set ID is similar with Rel-19 Set ID but is specifically used within the same gNB (intra-gNB). After source gNB received the Sub-Set ID of all candidate cells, it shall generate Rel-19 Set ID for each candidate cell. And send the Set ID information to each candidate gNBs for subsequent LTM.

Proposal 7: introduce Sub-Set ID in the HANDOVER REQUEST ACK message, and source gNB shall assign the Rel-19 Set ID based on the received Sub-Set ID.

NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Moreover, RAN2 has also determined that when the Rel-19 set ID is different between the source and target, the NCC shall be include in the cell switch command MAC CE. This required the DU to be aware of the Rel-19 set ID configured by the CU. The CU can include the Rel-19 set ID list for e all candidate cells in UE context modification request message and contain this information in configuration ID mapping list IE.

Proposal 8: Source gNB shall send the R19 set IDs information to each candidate gNB for subsequent LTM.

Proposal 9: CU need to send the R19 set IDs information to serving DUs for execute cell switch, configuration ID mapping list IE can be enhanced.

2.5 TA relative

2.5.1 TA information transfer

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- FFS on the XnAP IDs.

For TA information transfer, RAN3 has identified several IEs should be transferred between candidate gNB and source gNB. One undecided IE is the XnAP ID. However, under the current mechanism, the RACH resource provided by candidate gNB is allocated on a per-source DU basis, rather than on a per-UE basis. As a result, the candidate gNB cannot distinguish between different UEs when it receives an early synchronization request..

Proposal 10: As candidate gNB can not identify the UE ID, the XnAP ID should not contain in TA information transfer message.

MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

In additional, RAN2 has concluded to support MIMO 2TA in inter-CU LTM. In the TA information transfer message, the Tag ID Pointer IE should also be contained.

Proposal 11: Introduce Tag ID Pointer IE in TA information transfer message for indicating the TAG corresponding to the TA value

2.5.2 TA validity

TA validity is discussed in Rel-18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

In Rel-18, though not explicitly specified, the fundamental approach to maintaining TA validity is to initiate a timer at the source side. The range of this timer is typically on the same order of magnitude as TAT (Timing Advance Timer) because they serve the same purpose. As defined in TS 38.331, TAT is specified as ENUMERATED, with the minimum value being 500ms. Generally, the latency between Xn interfaces is considered to be around 10ms. It is evident that the latency of the Xn interface is significantly less than the TAT, i.e., the TA validity time. Therefore, it is still feasible to determine the validity of TA at the source DU and leave it to implementation as per Rel-18.

Proposal 12: Source DU/gNB is in charge of checking the TA validity by implementation.

To avoid unnecessary early synchronization, in Rel-18, we enable the source to transfer the obtained TA (Timing Advance) values in the Cell Switch Notification message. In the intra-DU scenario, the movement range of the UE is limited, so the TA remains valid in the target cell with a high probability. However, extending this to the inter-CU case, where the UE's movement range is larger, it becomes difficult to assert that the TA obtained at the source is still valid at the target side, regardless of whether it is at the source or target. Using an unsuitable TA may lead to handover failure, and the drawbacks of transferring TA among different CUs outweigh the potential advantages.

Proposal 13: Given that TA is relative to UE location and mobility, transferring TA values among different gNB is not support in inter-CU LTM.

2.5.3 UE based TA

From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

In Rel-18, the CU determines whether UE-based TA measurement is supported between two candidate cells based on the requirements defined by RAN4. If there are no new requirements from RAN4, in the inter-CU scenario, the source CU can still generate the UE-Based TA Measurement Configuration for each candidate cell and transfer this configuration to each candidate CU via the XnAP interface

Proposal 14: Source CU generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB via LTM configuration update message.

2.6 Reference configuration

Reference configuration is int roduced in order to reduce the RRC overload. For this purpose, the reference configuration shall contain common configuration as much as possible. As mentioned by many companies, in the inter-CU case, the configuration of different CU sometimes varies greatly, so it may be hard to provide common configuration by the source gNB. As source gNB is aware of distribution of candidate cells, a feasible way is let source decides which node is suitable to provide the reference configuration.

Proposal 15: Enable the source gNB to generate reference configuration or request other candidate gNB to provide the reference configuration.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Considering the above wording to limited single UE association for multiple request to a same gNB.

Proposal 2: For the granularity of the CSI-RS resource/resource set, postpone the discussion in RAN3 and wait RAN2 further conclusion.

Proposal 3: Introduce a new indicator in HANDOVER REUQETS message to request candidate cell providing CSI-RS resource for CSI acquisition. The decision on whether to provide periodic or semi-persistent CSI-RS resources will be up to the candidate gNB.

Proposal 4: It is proposal source node to trigger SP CSI-RS active/ deactive for CSI acquisition, and candidate node decide the specific CSI-RS and send it back to source node.

Proposal 5: The initial source gNB should send the target cell and the assigned target UE XnAP ID mapping list to all candidate cells via the LTM CONFIGURATION UPDATE message.

Proposal 6: When establishing a new UE association between two candidate gNBs, the request node should include the old target UE XnAP ID in the LTM CONFIGURATION UPDATE message.

Proposal 7: introduce Sub-Set ID in the HANDOVER REQUEST ACK message, and source gNB shall assign the Rel-19 Set ID based on the received Sub-Set ID.

Proposal 8: Source gNB shall send the R19 set IDs information to each candidate gNB for subsequent LTM.

Proposal 9: CU need to send the R19 set IDs information to serving DUs for execute cell switch, configuration ID mapping list IE can be enhanced.

Proposal 10: As candidate gNB can not identify the UE ID, the XnAP ID should not contain in TA information transfer message.

Proposal 11: Introduce Tag ID Pointer IE in TA information transfer message for indicating the TAG corresponding to the TA value

Proposal 12: Source DU/gNB is in charge of checking the TA validity by implementation.

Proposal 13: Given that TA is relative to UE location and mobility, transferring TA values among different gNB is not support in inter-CU LTM.

Proposal 14: Source CU generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB via LTM configuration update message,

Proposal 15: Enable the source gNB to generate reference configuration or request other candidate gNB to provide the reference configuration.

4. Reference

[1]	R3-253008 LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells

[2]	R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis

[3]	R3-253009 LS on RAN2 agreements for security key handling in inter-CU LTM