3GPP TSG-RAN WG3 Meeting #128	R3-253485

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	(TP for AI/ML BLCR to TS 38.300) Capturing agreements in Stage 2 RAN specifications

Source: 	Huawei, Orange, InterDigital, CATT, FiberCop, Deutsche Telekom, Jio 	Platforms

Agenda item:	11.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 is discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

This contribution provides a TP for the AI/ML BLCR to TS 38.300 capturing RAN3 agreements up to RAN3#127bis (-April 2025) which are relevant for Stage 2 description of Rel-19 AI/ML for NG-RAN.

# 2. Discussion

## 2.1 RAN3 agreements relevant for the AI/ML BLCR to TS 38.300

***** RAN3#125bis ***** [4] AI/ML-based Network Slicing Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface. AI/ML-based Coverage and Capacity Optimization Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. Introducing separate time information for future coverage state and predicted CCO issue. ***** RAN3#126 ***** [5] AI/ML-based Network Slicing First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. AI/ML-based Coverage and Capacity Optimization Introducing new IE for Future Coverage Modification List over Xn. ***** RAN3#127bis ***** [6] AI/ML-based Network Slicing Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.300 in Annex 1.

# 3. Conclusion

We make the following proposal:

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.300 in Annex 1.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.300

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

St. Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for the AI/ML BLCR to TS 38.300) Support of enhancements on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange, InterDigital, CATT, FiberCop, Deutsche Telekom, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Stage 2 specification is enhanced to cover RAN3 agreements relevant for the Stage 2 description of enhancements on AI/ML for NG-RAN related to AI/ML-based Network Slicing and AI/ML-based CCO. An editorial correction is also considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Stage 2 description of AI/ML for NG-RAN is updated to reflect the following RAN3 agreements: ***** RAN3#125bis ***** AI/ML-based Network Slicing Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface. AI/ML-based Coverage and Capacity Optimization Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. Introducing separate time information for future coverage state and predicted CCO issue. ***** RAN3#126 ***** AI/ML-based Network Slicing First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. AI/ML-based Coverage and Capacity Optimization Introducing new IE for Future Coverage Modification List over Xn. ***** RAN3#127bis ***** AI/ML-based Network Slicing Add Slice UE performance IE in the DATA COLLECTION UPDATE message. Editorial correction in clause 16.20.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements on AI/ML for NG-RAN related to AI/ML-based Network Slicing and AI/ML-based CCO are not captured at Stage 2 level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.20.2, 16.20.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.20	Support of AI/ML for NG-RAN

*** skip unmodified text ***

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope. Model-specific performance information, e.g. model performance indicators specified in clause 6 of TS 28.105 [64], is not exchanged over NG-RAN interfaces in TS 38.401 [4].

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64]. An AI/ML Model needs to be trained, validated and tested before deployment for AI/ML Model Inference.

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including predicted radio resource status per slice;

-	UE performance feedback, including per slice UE performance;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity.

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Predicted coverage modification cause, future coverage modification, and the time for future coverage state are transferred to the neighbour NG-RAN node via the NG-RAN Configuration Update procedure to provide information for, e.g., proactive resolution of CCO issues.

<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>