3GPP TSG-RAN WG3 Meeting #128	R3-253494

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on UE performance feedback for AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC.

This contribution analyzes the technical enablers for specifying support over the Xn interface for mobility optimization in NR-DC, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

The outcome of the last RAN3 meeting concerning the mobility optimization in NR-DC is reported below [4]:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked. The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released. UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture? Whether to introduce assistance information (predicted PSCell ID) from MN to SN? Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID? Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In the next sections we discussed the open issues listed above related to UE performance.

## 2.1 Scenarios for UE performance collection and reporting in NR-DC

In RAN3#125bis meeting the following open issue was captured as an aspect that needs further discussion [5]:

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

Even if it is not explicitly mentioned, the above open issue is related to a scenario being different from the ones already agreed in RAN3#125bis to be addressed (i.e., SN Addition and MN-initiated SN Change). This additional scenario has been described in the contribution [6] submitted in RAN3#125bis meeting, where a UE initially single-connected to a source gNB (say gNB1) is handed over to a target gNB (say gNB2) which, in turn, decides to set-up a NR-DC with a third gNB (say gNB3). In such scenario, the target gNB (gNB2) becomes the target MN while the third gNB (gNB3) becomes the target SN, and UE performance collection is configured from the source gNB (gNB1) to be reported by the target MN (gNB2) by means of the Data Collection Reporting Initiation procedure existing between gNB1 and gNB2. There is however the need to discuss which is the impact of the target SN (gNB3) on the UE performance reported from the target MN to the source gNB, as well as how to configure the collection of UE performance at the target SN side and reporting of such UE performance back to the target MN. The target MN should report to the source gNB – via the Data Collection Reporting procedure – either the aggregated UE performance collected at both target MN and target SN or UE performance measured at MN and SN separately. Moreover, it is our understanding that UE performance are typically collected immediately after the handover (source gNB gNB1 to a target gNB gNB2 in the example above) and for a short time window (configured by the source gNB), so it is unclear whether there will be enough time for the target gNB to establish NR-DC with the third gNB (gNB3). Therefore, considering that Rel-19 is the first release to discuss AI/ML-based mobility optimization in NR-DC, new scenarios other than the “basic” SN Addition and MN-initiated SN Change should be deprioritized and their actual feasibility can be discussed in a future release.

Proposal 1: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in a future 3GPP release.

## 2.2 On UE performance metrics in NR-DC

The UE performance metrics to be reported as part of the feedback information after an AI/ML-based action as well as the UE performance collection configuration have been specified in Rel-18 for XnAP [7] clause 9.2.3.179 and clause 9.2.3.186, respectively. Still in Rel-18, UE performance is only for the basic single connectivity case, where the collection and reporting is done at the target node side towards the source node on a per-UE granularity. For the case of NR-DC, RAN3 needs to investigate which UE performance metrics are triggered to be collected by the SN after successful SN Addition / Change (MN-initiated) for the concerned UE as a function of the bearer type. Even though most of the metrics in TS 28.558 [8] are specified to be measured at RLC (i.e., in the gNB-DU), in the last meeting and as part of the discussion on split architecture support for Rel-18 use cases RAN3 eventually considered the “gNB-CU-UP-centric” approach, based on which UE performance metrics are measured (and aggregated, for the case of packet delay) at PDCP level (and hence in the gNB-CU-UP) and reported to the gNB-CU-CP over the E1 interface. It is however not clear yet whether PDCP-level measurements for Average UE Throughput UL/DL and Average Packet Loss DL/UL can be made available in TS 28.558, therefore RAN3 agreed to send a LS to SA5 in [9] to check whether it is feasible to define PDCP-level measurements for throughput and packet loss.

In addition to the need to define a measurement framework which is valid for all UE performance metrics, it was highlighted in contribution [10] that, in case of NR-DC with split bearers, measurements performed in the gNB-DU (at RLC level) are per leg and hence they cannot be aggregated to derive per-DRB measurements (even though it is possible to collect the individual MCG and SCG leg level measurements), due to the fact that there is no common RLC SDU point for MCG and SCG legs and neither MN nor SN knows the transmission/buffer status of data in the other leg. The issue may be solved by relying on PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN), which allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split). Therefore, and as a general principle, the MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured at SN Addition or MN-initiated SN Change.

Observation 1: PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN) allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split).

Proposal 2: RAN3 to agree the general principle that MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured for the UE at SN Addition or MN-initiated SN Change.

Based on the above, we suggest waiting for the outcome of the discussion on UE performance measurements in split architecture – with related SA5 feedback on feasibility of measurements at PDCP, as requested in [9] – and strive to find commonalities with the corresponding discussions in case of AI/ML-based mobility optimization in NR-DC. Such consideration is in line with the FFS from the last meeting [4]:

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Proposal 3: RAN3 to wait for the outcome of the discussion on UE performance measurements in split architecture and strive to find commonalities with the corresponding discussions in case of NR-DC.

An important issue to discuss is the granularity of the UE performance metrics that the SN needs to report. Our understanding is that UE performance metrics in TS 28.558 [8] are measured on a per-DRB per-UE granularity, other than the Average UE Throughput DL/UL measurements which are already defined with a per-UE granularity. However, the UE performance metrics in clause 9.2.3.179 of TS 38.423 [7] are intended to be reported on a per-UE granularity.

As a starting point, we can consider UE performance to be reported on a per-UE granularity as done in Rel-18. This because the intended use of such UE performance should be to evaluate whether the AI/ML model decision to setup NR-DC with a certain SN for the UE was beneficial with respect to keeping the concerned UE in single connectivity. And single connectivity is addressed in Rel-18 where only per-UE UE performance are reported. To derive per-UE UE performance metrics (Average Packet Delay DL/UL and Average Packet Loss DL) starting from the corresponding per-DRB per-UE measurements the RAN3 common understanding from RAN3#124 meeting can be considered, that is, per-UE UE performance is derived as the average value of per-DRB measurements of the UE performance metrics [11]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

Proposal 4: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity.

## 2.3 Conditions governing collection of UE performance at SN side

Conditions to start the UE performance collection in NR-DC

The following FFS from the last meeting remains to be discussed [4]:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In RAN3#127, contribution [12] proposed to capture an additional condition, i.e., SCG Activation, to be evaluated by the SN – along with the condition related to the successful S-NG-RAN Node Addition, as per the latest version of the XnAP BLCR – in order to understand when to start the collection of UE performance to be then reported to the MN. More in detail, the current version of the XnAP BLCR captures the following text [13]:

### 8.3.1 S-NG-RAN node Addition Preparation #### 8.3.1.1 General The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same. *** skip unmodified text *** Interaction with the Activity Notification procedure Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level. Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures: If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

As per TS 37.340 [14] clause 7.13, an activation/deactivation mechanism of SCG is supported to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. Moreover, the MN can configure the SCG as (de-)activated upon PSCell addition (among other procedures such as, PSCell change, RRC Resume or handover), and it is also allowed that the SN independently (de-)activates the SCG. By taking Proposal 5 into account (for which UE performance metrics should be requested to be reported by the SN only if there is at least a SN-terminated bearer configured for the UE at SN Addition or MN-initiated SN Change), and by also assuming that it is feasible to measure UE performance metrics at PDCP level, we think that for the case of split bearer or SCG bearer the SCG should be activated so that there will be data transmission via SCG RLC bearers, meaning that the improvement in UE performance enabled by data transmission over the SCG can be observed. And the activation of the SCG may be requested by the MN when sending the XnAP S-NODE ADDITION REQUEST message to the SN (via the SCG Activation Request IE included therein). As a consequence, RAN3 should discuss whether UE performance at the SN can be collected only after SCG resources are activated for the case of SN-terminated split bearer or SCG bearer.

Proposal 5: RAN3 to agree whether UE performance at the SN side can be collected after successful S-NG-RAN node addition and after SCG activation in case of SN-terminated split bearer or SCG bearer.

Conditions to stop the UE performance collection in NR-DC

Current TS 38.423 clause 8.4.13.2 captures, for the case of UE in single connectivity, the exit conditions to be considered by the target node in order to evaluate when to stop the collection of UE performance [7]:

### 8.4.13 Data Collection Reporting Initiation ### **** Skip unmodified text **** #### 8.4.13.2 Successful Operation **** Skip unmodified text **** If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. **** Skip unmodified text ****

As already specified in Rel-18 for the AI/ML mobility optimization, in the last meeting RAN3 also discussed about the exit conditions to be evaluated by the SN in order to stop the collection of UE performance at the SN side, and the following was agreed [4]:

The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released.

In light of Proposal 8, we think that an additional exit condition should be introduced to also consider the SCG activation in case SN-terminated split bearers and/or SCG bearers are configured. To be specific:

- for SN-terminated MCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- for SN-terminated split bearers and/or SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE.

In Rel-18 and for the single connectivity case, UE performance can be collected and reported periodically during the collection time duration (the latter identified via the Collection Time Duration for UE Performance IE in XnAP) which start at successful handover. Based on this, for the AI/ML-based mobility in NR-DC, it should be discussed what is the SN behaviour in case SN-terminated split bearers and/or SCG bearers are configured but the SCG is deactivated during the periodic collection and reporting of UE performance within the configured collection time duration. For the UE performance collection, since Rel-18, there is the common understanding in RAN3 that UE performance metrics are collected and reported within a time interval which is typically very short, therefore, in case the SCG is deactivated during this short collection time duration, we believe that the SN should simply stop the collection of UE performance and report the collected measurements to the MN at the first available DATA COLLECTION UPDATE message, without the need to resume the collection process in case the SCG is re-activated during the residual collection time duration which is supposed to be extremely short and close to its expiration, hence without adding further complexity to the UE performance collection and reporting process.

Proposal 6: RAN3 to agree the following additional exit conditions to stop the collection of UE performance at the SN side:

- for SN-terminated split bearers and/or SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

- for SN-terminated split bearers and/or SCG bearers, the SCG is deactivated during the collection time duration.

The TP for the XnAP BLCR for AI/ML for NG-RAN in [15] reflects the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

Scenarios for UE performance collection and reporting in NR-DC

Proposal 1: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in a future 3GPP release.

On UE performance metrics in NR-DC

Observation 1: PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN) allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split).

Proposal 2: RAN3 to agree the general principle that MN requests the SN to report UE performance only if there is at least a SN-terminated bearer configured for the UE at SN Addition or MN-initiated SN Change.

Proposal 3: RAN3 to wait for the outcome of the discussion on UE performance measurements in split architecture and strive to find commonalities with the corresponding discussions in case of NR-DC.

Proposal 4: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity.

Conditions governing collection of UE performance at SN side

Proposal 5: RAN3 to agree whether UE performance at the SN side can be collected after successful S-NG-RAN node addition and after SCG activation in case of SN-terminated split bearer or SCG bearer.

Proposal 6: RAN3 to agree the following additional exit conditions to stop the collection of UE performance at the SN side:

- for SN-terminated split bearers and/or SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

- for SN-terminated split bearers and/or SCG bearers, the SCG is deactivated during the collection time duration.

The TP for the XnAP BLCR for AI/ML for NG-RAN in [15] reflects the above proposals.

# 4. Reference

- RP-250326, Revised New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245608, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia, Jio

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.4.0 (2024-12)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, V19.2.0 (2025-01)

- R3-252436, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE)

- R3-250575, Discussion on Rel-18 UE Performance measurements, Nokia

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-250573, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia

- R3-252504, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital

- 3GPP TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage-2, V18.4.0 (2024-12)

- R3-253496, (TP for AI/ML BLCR to TS 38.423) XnAP enhancements for AI/ML-based Mobility Optimization in NR-DC, Huawei